Information about failures

KDI 1903 M-MP Owner Manual (Rev.01)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
| Technical Documentation | ED0053030860 | 05.1 | 02/2016 | 04/2023 | M. SILVESTRONE | C. TAGLIAVINI |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about failures

## Useful information about failures

- This chapter contains information about the problems that may appear during engine operation with its causes and trouble shooting Tab. 7.2 .

- In some cases, you shall turn off the engine immediately to avoid further damage Tab. 7.1 .

Tab 7.1

| THE ENGINE MUST BE IMMEDIATELY TURNED OFF WHEN: |  |
| --- | --- |
| 1 | The engine rpms suddenly increase and decrease |
| 2 | A sudden and/or unusual noise is heard |
| 3 | The colour of the exhaust fumes suddenly darkens |
| 4 | The oil pressure indicator light turns on while running |

Tab 7.2

| TROUBLES | POSSIBLE CAUSE | SOLUTION | PAR. |
| --- | --- | --- | --- |
| The engine does not start | Sulphated battery terminals corroded | Clean the battery terminals | -- |
|  | Battery voltage too low | Recharge the battery or replace it | -- |
|  | Low fuel level | Refuel | 4.4 |
|  | Frozen fuel | Contact Lombardini Marine authorised workshops | -- |
|  | Clogged fuel filter | Replace with a new filter | 6.3 |
|  | Air suction in fuel system | Contact Lombardini Marine authorised workshops | -- |
|  | Clogged air filter | Replace with a new filter | -- |
|  | Clogged pipes | Contact Lombardini Marine authorised workshops | -- |
|  | Open fuse | Replace with a new fuse; if the problem persists, contact Lombardini Marine authorised workshops | -- |
|  | Intake or exhaust system clogged | Contact Lombardini Marine authorised workshops | -- |
| Engine starts but stops | Inefficient electrical connections | Clean the electrical contacts; if the problem persists, contact Lombardini Marine authorised workshops | -- |
|  | Sulphated battery terminals | Clean the battery terminals | -- |
|  | Clogged fuel filter | Replace with a new filter and clean the tank | 6.3 |
|  | Clogged fuel pipes | Contact Lombardini Marine authorised workshops | -- |
| RPM instability at idle speed | Clogged fuel pipes | Contact Lombardini Marine authorised workshops | -- |
| Low idle speed | Clogged fuel pipes | Contact Lombardini Marine authorised workshops | -- |
|  | Poor quality fuel | Clean the tank and refuel with quality fuel | 2.5 |
| Blue smoke | High oil sump level | Replace the engine oil; if the problem persists, contact Lombardini Marine authorised workshops | 6.1 |
|  | Clogged air filter | Replace with a new filter | 6.8 |
| Excessive fuel consumption | Clogged air filter | Replace with a new filter | 6.8 |
|  | High oil sump level | Replace the engine oil; if the problem persists, contact Lombardini Marine authorised workshops | 6.1 |
| Engine lost its initial performance | Clogged air filter | Replace with a new filter | 6.8 |
|  | Clogged fuel pipes | Contact Lombardini Marine authorised workshops | -- |
|  | Cheap fuel | Clean the tank and refuel with quality fuel | -- |
|  | High oil sump level | Replace the engine oil; if the problem persists, contact Lombardini Marine authorised workshops | 6.1 |
| Slow acceleration | Clogged fuel filter | Replace the fuel filter | 6.3 |
| Engine jerking | Clogged fuel pipes | Contact Lombardini Marine authorised workshops |  |
| Engine overheats | Insufficient coolant level | Fill up to the level | 5.6 |
|  | High oil sump level | Replace the engine oil; if the problem persists, contact Lombardini Marine authorised workshops | 6.1 |
|  | Clogged radiator | Clean the radiator; if the problem persists, contact Lombardini Marine authorised workshops | 5.7 |

In the event that the solutions proposed in Tab. 7.2 do not eliminate the trouble, contact a Lombardini Marine authorized workshop.

## Error signals on the control panel

| Tab. 7.3 shows the errors that can be generated on the control panel. Tab. 7.3 POS.; DESCRIPTION. Engine oil pressure low. High coolant temperature. Disabled alternator (with engine rpm < 850 rpm). Maintenance service (*1). Insufficient battery voltage (flashing display with battery < 9 V). Besides visual signals, acoustic errors are also transmitted from the panel. Any errors disappear automatically once the anomaly has been solved. (*1) - A maintenance service does not automatically carry out a reset; it must be reset manually, as follows: - Press the E push button until the maintenance service page (Service h) is displayed on the screen D . - Simultaneously press push buttons E and F for 10 secs . for screen D to reset the hours until the next maintenance. | Fig 7.1 |
| --- | --- |