Informazioni sulle regolazioni

KDI 2504 M Workshop manual (Rev. 09.6)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
| Technical Documentation | ED005302-961 | 9.6 | 04/2013 | 03/2023 | M. SILVESTRONE | V. MANINI |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information on adjustments

## Air filter check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Hose A must be completely clean and not damaged. - Air filter cartridge B and its housing C must be completely clean and free from impurities. | Fig 12.1 |

## Rubber hose and manifold control

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| The check is carried out by applying slight deflection or bending along the tube/hose and next to the hose clamps. Components must be replaced if they have clear signs of cracks, tears, cuts, leaks, or do not retain a certain degree of elasticity. - Check the condition of all rubber hoses A . - Check whether there are any leakages of air, water, oil or fuel next to their connections. | Fig 12.2 Fig 12.3 |

## Oil leak check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| Check that there are no leakages next to area A . - Start the engine at idle speed or without a load and check whether there are any leakages next to area A. - It is anyhow necessary to also check the seals of all main components and their surface contact, such as: - crankcase and oil seal (side 1 a PTO) - oil sump and exhaust caps - cylinder head and its assembled components - rocker arm cover - Timing system carter and oil seal (side 2 a PTO) - oil dipstick housing or rod support tube. NOTE: Perform the checks described in Points 1 and 2 periodically and during maintenance procedures. It is also necessary to check for leakages on the components that are not listed. If necessary, disassemble the components that have a leakage and investigate the possible cause. The components must be replaced otherwise they do notguarantee their sealing. | Fig 12.4 Fig 12.5 |

## Oil pressure check

| - Insert a thermocouple instead of the oil dipstick A . - Unscrew and remove the oil pressure switch B and screw on a 10 bar pressure gauge in its seat (Fig. 12.8) . - Start the engine at idle speed and without a load, check the oil pressure value according to the oil temperature (Fig. 12.7 ). NOTE : The graph in Fig. 12.7 illustrates the pressure line with speed of 1000 Rpm. - If the pressure values are below the values indicated in Fig. 12.7 , check to identify the cause of the problem. Fig. 12.7 | Fig. 12.6 Fig. 12.8 |
| --- | --- |