Tools information

KDI 3404 TCR Workshop Manual (Rev. 10.4)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Tools information

## Information regarding specific tools

In Tab 13.1 - 13.2 - 13.3  there is a list of all the specific tools that are required and approved to carry out operations of disassembly - assembly - regulations - settings - repairs on engine series KDI

, correctly and safely.

Warning

- KOHLER declines all responsibility for any damage to the engine, persons, or things caused by the use of different types of tools to those indicated in Tab 13.1 - 13.2 - 13.3 , where referred to them in the manual.

| Tab. 13.1 |  |  |  |
| --- | --- | --- | --- |
| SPECIAL TOOLS FOR DISASSEMBLY AND ASSEMBLY |  |  |  |
| "ST" | Picture/Draw | DESCRIPTION | PART NUMBER |
| ST_03 |  | Piston protrusion - electronic injectors cylinder head surface control tool | ED0014602980-S |
| ST_05 |  | Spanner for capscrews Six nicks SN 8 | ED0014603650-S |
| ST_06 |  | Spanner for capscrews Six nicks SN 5 | ED0014603640-S |
| ST_07 |  | Tool for disassembling / reassembling valves | ED0014603720-S |
| ST_08 |  | Tool for gasket valve stem | ED0014603660-S |
| ST_11 |  | Rocker arm cover gasket assembling tool (electronic injector seat) | ED0014603620-S |
| ST_12 |  | Rocker arm cover gasket assembling tool (rocker arm capscrew gudgeon seat - electronic injector fastening capscrew brace seat) | ED0014603630-S |
| ST_13 |  | High-pressure fuel injection pump puller gear | ED0014604050-S |
| ST_17 |  | Rocker arm cover mounting studs | ED0014603730-S |
| ST_18 |  | Intake and oil sump manifold mounting studs | ED0014603740-S |
| ST_34 |  | Crankshaft blocking tool | ED0014604270-S |
| ST_43 |  | Flywheel lifting tool | ED0014604030-S |
| ST_44 |  | Flange bell lifting tool | ED0014604010-S |
| ST_45 |  | Flange bell placing tool | ED0014604020-S |
| ST_46 |  | Flywheel placing tool | ED0014604040-S |
| ST_47 |  | Tool for sealing ring insertion flywheel side and pulley side | ED0014604340-S |
| Tab. 13.2 |  |  |  |
| SPECIFIC EQUIPMENT TO PROTECT COMPONENTS OF THE INJECTION CIRCUIT |  |  |  |
| ST_40 |  | Complete box with caps to close holes and unions for high-pressure injection circuit components. | ED0082051380-S |
| Tab. 13.3 |  |  |  |
| SPECIAL TOOLS TO TEST THE ENGINE ON THE TEST BENCH - DIAGNOSTIC PROCEDURE |  |  |  |
| ST_01 |  | Complete instrument Kit for diagnostics "POLAR XL" | ED0014603690-S |
| ST_49 |  | Complete instrument Kit for diagnostics "DIAGBOX" | ED0014604210-S |
| ST_50 |  | Complete instrument Kit for engines bench test | ED0014604110-S |