Information for disassembly

KDI 1903 M-MP Workshop manual (Rev.02.3)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information for disassembly

## Recommendations for disassembly

Important

- To easily find the specific topics of interest consult the index .

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- The operator should prepare all equipment and tools in order to enable him to carry out the operations correctly and safely.

- Before disassembly, perform the operation described in Chap. 5 .

- Before proceeding with operation, carefully read Chap. 3 .

- In order to operate safely and easily, we recommend positioning the engine on a rotating stand for engine overhauling.

- eal all injection component unions as illustrated in Par. 2.9.7 during assembly.

- Protect all disassembled components and coupling surfaces subject to oxidation with lubricant.

- Where necessary, reference to special tools to use during disassembly operations is indicated (es. ST_05 ), identified in Chap.12 .

## Disassembly of the exhaust line

| - Loosen clamps A and disconnect hoses B . - Disconnect hose C . - Loosen capscrews D and remove the cooled exhaust manifold E . | Fig. 7.1 |
| --- | --- |

## Disassemble the intake line

| - Loosen clamp A and disconnect tube B . - Loosen capscrews C and remove accelerator support D . - Loosen hollow studs E and remove oil dipstick F . - Loosen capscrews G and remove manifold H together with gasket J . | Fig. 7.2 |
| --- | --- |

## Electric components disassembly

| 7.4.1 Electric wiring - Disconnect and remove the engine wiring. NOTE : refer to Par. 2.13.1.3 to disconnect all connectors. |  |
| --- | --- |
| 7.4.2 Starter motor Important - The motor is not repairable. - Perform the operations described in Par 6.1.5 points 1-2. | Fig 7.3 |
| 7.4.3 Belt and alternator - Perform the operations described in Par. 6.4.1 - 6.5.1 . - Undo the screws A and B and remove the alternator C. - Loosen hollow stud D . Important - The belts must always be replaced every time they are disassembled, even if they have not reached the scheduled hours for replacement | Fig 7.4 |
| 7.4.4 Sensors and switches Important - After disassembly, protect the sensors suitably against knocks, dampness and any high temperature sources. - The sensors and switches cannot be repaired, therefore they must be replaced in the event of anomalies. |  |
| 7.4.4.1 Oil pressure switch and sensor ( ) - Loosen capscrew E and remove oil sensor F and oil pressure switch G . | Fig 7.5 |
| 7.4.4.2 Coolant temperature sensor ( ) - Unscrew and remove the sensor H . | Fig 7.6 |
| 7.4.4.3 Coolant temperature switch - Loosen and remove switch J . | Fig. 7.7 |
| 7.4.5 Electric fuel pump ( ) - Loosen capscrew K . - Loosen nuts M and remove pump L . | Fig 7.8 |

## Coolant recirculation components disassembly

| 7.5.1 Coolant recirculation components disassembly Important - The pump B is not reparaible - Perform the operations described in Par. 6.6.1 . |  |
| --- | --- |
| 7.5.2 Thermostatic valve - Undo the screws A and remove the thermostatic valve cover B . - Remove the thermostatic valve C and its gasket D . Important - Always replace the gasket D every time it is disassembled. - Check that the air bleeding hole is not clogged or blocked ( Par. 2.11.3 ). | Fig 7.9 |
| 7.5.3 External water pump - Loosen capscrews E and remove clamps F . - Loosen capscrews G and remove pump H . | Fig 7.10 |

## Crankshaft pulley disassembly

- Perform the operations described in Par. 6.7.1.

## Lubrication circuit disassembly

| 7.7.1 Oil overpressure valve - Perform the operations described in Par. 6.9.1 . |  |
| --- | --- |
| 7.7.2 Timing system semi-crankcase oil filling flange ( ) - Undo the screws A and remove the oil filling flange B ( ST_06 ) . - Remove the gasket C . | Fig 7.11 |
| 7.7.3 Timing system - Perform the operations described in Par. 6.8.3 . |  |
| 7.7.4 Oil pump Important - The oil pump is not repairable. - Perform the operations described in Par. 6.8.4 . |  |
| 7.7.5 Oil filter - Perform the operations described in Par. 6.10.1 . |  |
| 7.7.6. Oil extraction pump - Loosen capscrew D and remove tube E . - Loosen union F . - Loosen capscrew G and remove pump H . | Fig 7.12 |

## Fuel system disassembly

| Important • Seal all injection component unions as illustrated in Par. 2.9.7 . • The fuel injection circuit undergoes high pressure, use safety protections as described in Par 3.4.3 . |  |
| --- | --- |
| 7.8.1 Fuel pipes - Undo the screws A . - Undo the screws B,C,D and remove pipes E . | Fig 7.13 |
| 7.8.2 Fuel filter ( ) - Undo the screws F and remove filter G . | Fig 7.14 |
| 7.8.3 Fuel injection pipes - Perform the operations described in Par 6.1.1 . |  |
| 7.8.4 Rocker arm cover - Perform the operations described in Par 6.1.2 point 2. |  |
| 7.8.5 Fuel return pipes - Perform the operations described in Par 6.1.3 . |  |
| 7.8.6 Injectors - Perform the operations described in Par 6.1.4 . |  |
| 7.8.7 Injection pump Important - Before disassembling, carefully read Par. 2.17.1 . - The injectoion pump cannot be repaired. - Perform the operations described in Par 6.1.5 . |  |

## Timing system gear disassembly

| - Unscrew screws C and remove camshaft gear D . - Remove lock ring A and the shoulder ring B . - Remove the intermediate gear L . | Fig 7.15 |
| --- | --- |
| - Undo screws E and remove plate F . - Remove centring ring G . - Remove the retainer ring H and the shoulder ring J . | Fig 7.16 |  |
| - Remove gear K . - Undo screw M and remove gear N - Undo the screws F and remove the intermediate gear support G. | Fig. 7.17 |  |

## Disassembling the reversing gear

| - Loosen capscrews A and remove inverter B . | Fig. 7.18 |
| --- | --- |

## Disassembling the flexible coupling

| Important - Leave the special flywheel locking tool ST_34 assembled. - Loosen capscrews A and remove coupling B . | Fig. 7.19 |
| --- | --- |

## Flange unit disassembly

| 7.12.1 Flywheel Important - Leave the special tool ST_34 for blocking the flywheel. - Only undo the screw C located upwards. - Insert the tool ST_09 in the seat of the screw C tightening it all the way. - Undo the remaining screws D . Danger - The flywheel E is very heavy, pay utmost attention while removing it in order to prevent it from falling, with serious consequences for the operator. - Remove the flywheel E. - Remove the tool ST_09 . - Remove the tool ST_34 shown in Fig. 7.3 . | Fig 7.20 |
| --- | --- |
| 7.12.2 Flange housing - Undo the screws F and remove the engine housing G . Danger - The housing G is very heavy, pay utmost attention while removing in order to prevent it from falling, with serious consequences for the operator. | Fig 7.21 |

## Cylinder head unit disassembly

| 7.13.1 Rocker arm pin - Undo the screws A . - Remove the rocker arm pin unit B . | Fig 7.22 |
| --- | --- |
| 7.13.1.1 Rocker arm ( ) - Remove the retainer snap ring C . - Remove the shoulder rings D . - Remove the rocker arms E and springs. | Fig 7.23 |
| 7.13.2 Rods and u-bolts - Remove the valve control U-bolts H . - Remove the rocker arm control rods L . | Fig 7.24 |
| 7.13.3 Cylinder head Important - Wait for the engine to reach ambient temperature before to remove the head in order to prevent deforming. - The cylinder head fastening bolts M must be replaced every time they are disassembled. - Undo the bolts M . - Remove the cylinder head N . Important • Only use the eyebolts AE installed by Lombardini Marine to move the cylinder head Q. 3. Remove the head gasket P . | 3 Cylinders Fig. 7.25 4 Cylinders Fig 7.26 Fig 7.27 |
| 7.13.3.1 Valves ( ) - Mount the tool ST_07 on the head Q fixing it on one of the holes for fixing the rocker arm cover. NOTE: change the fixing hole according to the position of the valves to be removed. - Position the tool striker ST_07 on the valve concerned as shown in the figure. | Fig 7.28 |
| - Push the lever of the tool ST_07 downwards, in order to lower the valve plates S in the direction of the arrow T , using a magnet, remove the cotters U . NOTE : repeat all the operations for all the valves concerned. | Fig 7.29 |
| Important - Before removing the valves, make some marks in their original position, in order to prevent confusing them when they are re-assembled, if they are not replaced. - Remove the valves V . | Fig 7.30 |
| 7.13.3.2 Injector sleeve ( ) - Unscrew and remove the sleeves Z from the head Q . - Remove the gaskets AA and AB . | Fig 7.31 |
| 7.13.3.3 Valve steam gasket ( ) - Remove the oil seals AC . | Fig 7.32 |
| 7.13.3.4 Lifting eyebolts ( ) - Undo the screws AD and remove the eyebolts AE . - Make the thorough washing the cylinder head Q . | Fig 7.33 |

## Oil sump unit disassembly

| 7.14.1 Oil sump - Undo the screws A . - Remove the oil sump B by inserting a plate in the areas indicated by the arrow AA. | Fig 7.34 |
| --- | --- |
| 7.14.2 Oil intake pipe - Undo the screws C and remove the oil pipe D . | Fig 7.35 |
| 7.14.3 Oil vapour pipes ( ) - Unscrew and removethe pipes E . | Fig 7.36 |

## Engine block disassembly

| 7.15.1 Crankshaft gasket flange - Undo the screws A . - Remove the flange B and the gasket C . | Fig 7.37 |
| --- | --- |
| 7.15.2 Piston unit/connecting rod Important - Mark some numerical references (cylinder n°) on the connecting rods, connecting rod caps F1 , pistons and gudgeon pins to prevent unintentionally confusing the components not replaced during assembly. Failure to do this may result in engine malfunctions. - References on connecting rod M and cap F1 must only be carried out on a side in correspondence with K1 and K2 , as illustrated in Fig. 7.39 . - Screw the bolt AM temporarily. - Unsrew bolts E1 and remove the connecting rod caps F1 . | Fig 7.38 |
| NOTE : coupling cap F1 on the connecting rod can be carried out with centring taper pins (Fig. 7.40) or broken ( Fig. 7.41 - without centring taper pins). Fig. 7.40 Fig. 7.41 | Fig 7.39 |
| - Pull out the connecting rod - piston assembly from position 2 and 3 by manually applying pressure on the connecting rod big end L in the direction of arrow AK . - Couple the connecting rod big end caps L with the relevant piston and connecting rod unit M . - Turn capscrew AM and rotate the crankshaft by 180°. - Repeat points 2 to 5 to disassemble the connecting rod-piston assembly to position 1 and 4 . | Fig 7.42 |
| Warning - The connecting rod half-bearings Z are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.43 |
| 7.15.3 Lower semi-crankcase 3 CYLINDERS - Undo capscrews E and F by following the order indicated in the figure. - Remove the lower semi-crankcase D and store it in a suitable container for washing. | 3 Cylinders Fig 7.44 |
| 4 CYLINDERS - Undo capscrews E and F by following the order indicated in the figure. - Remove the lower semi-crankcase D and store it in a suitable container for washing. | 4 Cylinders Fig 7.45 |
| 7.15.4 Crankshaft Remove: - Crankshaft G . - The four shoulder semi-rings H . | Fig 7.46 |
| 7.15.5 Piston ( ) - Remove the retainer ring N . - Remove the pin P to separate the piston Q from the connecting rod R . Important - If they are not replaced, keep the components together (connecting rod - piston - gudgeon pin) by using references in order to prevent them from getting mixed up during assembly. | Fig 7.47 |
| 7.15.5.1 Rings ( ) - Remove the seal rings S . | Fig 7.48 |
| 7.15.6 Breather room closing cover ( ) - Undo the screws AE and remove the cover AF . | Fig 7.49 |
| 7.15.7 Camshaft - Remove the retainer ring V . - Extract the camshaft W from the upper crankcase AB . | Fig 7.50 |
| 7.15.8 Camshaft tappets - With a magnet, remove the tappets AA from the upper semi-crankcase AB . | Fig 7.51 |
| 7.15.9 Crankshaft bushings - Remove the crankshaft bushings AC from the upper semi-crankcase AB . Important - The crankshaft half-bearings AC are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.52 |
| - Remove the crankshaft bushings AF from the lower crankcase D . | Fig 7.53 |
| 7.15.10 Flange 3 rd PTO ( ) - Undo the screws AG . - Remove the cover AH and the gasket AL . | Fig 7.54 |