KSD 1403

KSD 1403 Workshop Manual Rev. 00

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | KSD1403-WS |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# General information

## Useful information

- This manual contains the instructions needed to carry out  proper use and maintenance of the engine, therefore it must always be available, for future reference when required.

- Information, descriptions and pictures contained in this manual reflect the basic configuration of the engines ( Par. 1.4 and Par. 1.5 ).

- However, the development of engines is continuous. Therefore, the information in this manual is subject to change without notice.

- KOHLER reserves the right to make, at any time, changes on the engines for technical or commercial reasons.

- These changes do not require KOHLER to intervene on the production marketed up to that time and nor to consider this manual as inappropriate.

- The paragraphs, tables and figures are numbered by chapter and followed by the progressive paragraph, table and/or figure number.

Es: Par. 1.3 - chapter 1 paragraph 3 .
Tab. 2.4 - chapter 2 table 4 .
Fig. 4.5 - chapter 4 figure 5 .

NOTE: The paragraphs may contain sub-paragraphs.

- All technical terms, specific components and symbols ( Tab. 15.1 ) that are in the manual are listed and described inside the glossary, which can be consulted in ( Chap. 15 ).

- The references of the objects described in the text and in the figure are indicated by letters and numbers, which are always and only related to the paragraph you are reading unless there are specific references to other figures or paragraphs.

- Reference to values are indicated by letters or numbers.

- Other important references are highlighted in red.

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- Any additional section that KOHLER will deem necessary to supply at a later stage must be kept with the manual and considered as an integral part of it.

- The information contained in this manual is the sole property of KOHLER , therefore no partial or total reproduction or replication is allowed without the express permission of KOHLER .

1.1.1 Useful Information - accident prevention - environmental impact

- Before proceeding repair - handling the motor , read the entire Chap. 3 , which contains important information about the procedures to be followed for safety and environment .

## Manufacturer and engine identification

1.1 Pos.; Descrizione. 1; Production place. 2; Manufacturer identification. 3; Engine model. 4; Engine serial number. 5; Month and year of production. 6; Approval data. 7; Engine version. 8; Data matrix.

## Homologation labels

1.7.1 Label for EPA rules (compilation example) 1.2 POS.; DESCRIPTION. 1; Model year in compliance with the rules. 2; Power category (kW). 3; Engine displacement (L). 4; Particulate emission limit (g/kWh). 5; Engine family ID. 6; Emission Control System = ECS. 7; Fuel with low sulphur content. 8; Injection timing. 9; Electronic injector opening pressure (bar). 10; Production date (example: 2013.JAN). 1.7.2 Label for China Standards (compilation example) 1.3 POS.; DESCRIPTION. 1; Manufacturer. 2; Engine model. 3; Manufactoring date. 4; Certificate N°. 5; Power range (kW). 6; Emission level. 7; Rated power. 8; Aftertreat system. 1.7.3 Label for Korea Standards (compilation example) 1.4 POS.; DESCRIPTION. 1; Tier 4 Final. 2; Engine model. 3; Manufactoring date and manufacturer code. 4; N° Korea emission certificate.

## Identification of the main internal components of the engine

WIEW OF EXHAUST SIDE

WIEW OF FLYWHEEL SIDE

WIEW OF TIMING SYSTEM SIDE

The following chapters contain operating references in order to clearly understand the engine. This paragraph illustrates these references that may be recognised by means of some main internal components.

Should you need to execute complex operations, always consult this paragraph

1.5

| POS. | DESCRIPTION |
| --- | --- |
| 1 | Cylinder/Piston N. 1 |
| 2 | Cylinder/Piston N. 2 |
| 3 | Cylinder/Piston N. 3 |
| 4 | Fuel injection pre-chamber |
| 5 | Crankshaft pulley (2 nd PTO) |
| 6 | Gear timing system |
| 7 | Thermostatic valve |
| 8 | Oil pump |
| 9 | Oil suction hose |
| 10 | Crankshaft |
| 11 | Exhaust manifold |
| 12 | Intake manifold |
| 13 | Engine valves camshaft |
| 14 | 3rd/4th PTO |
| 15 | Flywheel (1 st PTO) |
| 16 | Fuel injection pump camshaft |

## Engine component identification

NOTE: Some components are for illustrative purposes only and may vary or they are not supplied by Kohler.

1.6 POS.; DESCRIPTION. 1; ECU. 2; Oil filler cap. 3; Fuel injection pump. 4; Lub. oil filter. 5; Oil cooler. 6; Fuel filter. 7; Oil sump. 8; Oil drain plug. 9; Oil dipstick. 10; Turbocharger (TC & TCA version only). 11; Alternator. 12; Starter motor. 13; Flywheel. 14; Coolant pump. 15; Thermostatic valve. 16; 3rd PTO.

# Technical information

## General description of the engine

- 4-stroke, in-line cylinders Diesel engine; - Liquid-cooling system; - 2 valves per cylinder; - Indirect injection.

## Engine specifications (base version)

2.1 TECHNICAL DATA; UNIT OF MEASURE. Engine type; KSD 1403 NA; KSD 1403 TC; KSD 1403 TCA. Cylinders; n.; 3. Bore; mm; 81. Stroke; mm; 90. Displacement; cm 3; 1391. MAX INCLINATION DURING OPERATION (even in combined); α; 35° max. α; 45° max 1 min.. OIL CAPACITY (MAX level.) including oil filter; Compact Sump; lt.; 3.79. Deep Sump; 5.37. DRY WEIGHT; kg; 124; 128; 127.

## Engine dimensions (mm)

KSD 1403 NA

KSD 1403 TC

KSD 1403 TCA

NOTE : Dimensions vary according to engine configuration.

## Oil

Important

- The engine may be damaged if operated with improper oil level.

- Do not exceed the MAX level because a sudden increase in engine rpm could be caused by its combustion.

- Use only the approved oil to ensure adequate protection, efficiency and service life of the engine.

- The use of lubricants other than recommended may shorten the engine life.

- Viscosity must be appropriate to the ambient temperature to which the engine is to be exposed.

Danger

- Prolonged skin contact with the exhausted engine oil can cause cancer of the skin.

- If contact with oil cannot be avoided, thoroughly wash your hands with soap and water as soon as possible.

- For the exhausted oil disposal, refer to the Par.  DISPOSAL and SCRAPPING .

2.4.1 SAE oil classification

- In the SAE classification, oils are identified according to viscosity without considering any other qualitative characteristic.

- The code is composed of two numbers, which indicate, and must correspond to, the ambient temperature in which the engine operates, the first number refers to the viscosity when cold, for use during winter (" W "), while the second number is for viscosity at high temperatures.

Use oil viscosity based on the experienced air temperature range during the period between oil changes as indicated

in the table below.

Kohler / Rehlko X-treme 10W-40 formula pro; X-treme 10W-40; X-treme 5W-40 oil are preferred:

Oils meeting the following specifications are also recommended.

2.2

| RECCMMENDED OIL |  |  |
| --- | --- | --- |
| WITH SPECIFICATIONS | API | CI-4 or better |
|  | ACEA | E7 or better |
| VISCOSITY | SAE | 0w-40 (-40°C ÷ +50°C) 10w-40 (-25°C ÷ +50°C) |

## Fuel

| Important - Use of other types of fuel could damage the engine. Do not use dirty diesel fuel or mixtures of diesel fuel and water since this will cause serious engine faults. | Warning - Clean fuel prevents the fuel injectors from clogging. Immediately clean up any spillage during refuelling. - Never store diesel fuel in galvanized containers (i.e. coated with zinc). Diesel fuel and the galvanized coating react chemically to each other, producing flaking that quickly clogs filters or causes fuel pump and/or injector failure. - Any failure resulting from the use of fuel other than Tab. 2.3, 2.4 will not be covered by warranty. |
| --- | --- |
| Cetane number of 40 minimum. Cetane number greater than 47 is preferred, especially for temperatures below –20 °C (–4 °F) or elevations above 1675 m (5500 ft.). 2.3 Certification -->; Stage 5; Stage 3A 4; Tier III IV; no certification; Remarks :. Fuel Type -->; (EN 590 - DIN 51628); (EN 590 - DIN 51628); Do NOT USE vegetable oils as a biofuel not compliant with EN590 prescription.. Grade 1-D S15; For ambient temperatures below 0°C (32°F) with no arctic fuel available (Grade 1-D S15, Grade 2-D S15, ASTM D 975) use the following additives to prevent possible engine damage with low engine load in cold weather: - Power Service Diesel Fuel Supplement+Cetane Boost Different additives are not allowed. The use of approved additives has no impact on the engine maintenance schedule. Do NOT USE vegetable oils as a biofuel not compliant with ASTM D975 Grade1 and Grade2 prescription.. Grade 2-D S15. 2.5.1 Fuel Additives To aid in maintaining performance of the engines injection system, Rehlko has developed a family of fuel additives product for North America market. The Extra Performance Fuel Additive Arctic Formula a fuel conditioner in winter, the Extra Performance Fuel Additive a fuel injector deposit removal and prevention. See your local Rehlko dealer for availability. 2.4 Extra Performance Fuel Additive (North America only). Use Rehlko Injector Cleaner Diesel Fuel Additive before filling the tank once a year or every 500hrs with summer or winter grade fuel. Formulated to clean deposits from injectors and provide lubrication to high pressure pump and injectors to reduce wear, increasing fuel quality. Compatible with all exhaust emission system, including diesel particulate filters (DPFs). Follow all recommended instruction on product label.. Extra Performance Fuel Additive Arctic Formula (North America only). Use Rehlko Extra Performance Fuel Additive Arctic Formula, which contains anti-gel chemistry to treat non-winter grad fuel (No. 2-D in North America) during the cold-weather season. This extends the operability to about 10°C (18°F) below the could point. For operability at even lower temperatures, use winter grade fuel. Treat the fuel when the outside temperature drops below 0°C (32°F). For best results, use untreated fuel and follow all recommended instruction on product label. Cloud point is the temperature at which wax begins to from in the fuel causing filters to plug.. Fuel additives with biocide/algaecide functions are allowed in case of storage of fuel in the tank for long periods (one year or more).; For suggested brands and types contact Rehlko staff. NOTE: These additives must be diluted in the fuel following the product prescribed percentages when filling the tank.. |

## Coolant recommendation

Kohler / Rehlko XtendedTM prediluted coolant 50% ethylene glycol OAT is preferred. Coolant meeting the following specifications are also recommended. A mixture of 50% demineralized water and 50% low silicate ethylene glycol based coolant liquid must be used. Use a Long Life or Extended Life Heavy Duty OAT coolant free of: silicates, phosphates, borates, nitrites and amines. The following ethylene-glycol based engine coolant for all models within KSD engine family may be used: - OAT (Organic Acid Technology) Low Silicate: ASTM D-3306 D-6210 - HOAT (Hybrid Organic Acid Technology) Low Silicate: ASTM D-3306 D-6210 The above coolants in concentrated formulation must be mixed with distilled, deionized, or demineralized water. A pre-mixed formulation (40-60% or 50-50%) can be used directly when available. Important - Do not mix ethylene glycol and propylene glycol based coolants. Do not mix OAT and HOAT based coolant. OAT performance life can be drastically reduced if contaminated with nitrite-containing coolants. - Never use automotive-type coolants. These coolants do not contain the correct additives to protect heavy – duty diesel engines. OAT coolants are maintenance free up to 5 years or 4000hrs of operation , provided that the cooling system is topped up using the same type of coolant. Do not mix different coolant types. Test the coolant condition annually with coolant test strips. HOAT are not all maintenance free and it is recommended to have SCA (Supplemental Coolant Additives) added at the first maintenance interval.

## Battery recommendation

Battery not supplied by Kohler

2.5

| RECOMMENDED BATTERIES |  |
| --- | --- |
| STANDARD OPERATING CONDITIONS AMBIENT TEMPERATURE > -15°C |  |
| BATTERY CAPACITY | BATTERY DISCHARGE CURRENT |
| 70 Ah | SAE 650 EN 715 |

| STANDARD OPERATING CONDITIONS AMBIENT TEMPERATURE < -15°C |  |
| --- | --- |
| BATTERY CAPACITY | BATTERY DISCHARGE CURRENT |
| 100 Ah | SAE 850 EN 935 |

## Fuel system

Important - The high pressure supply injection system is highly susceptible to damage if the fuel is contaminated. - It is crucial that all components of the injection circuit are thoroughly cleaned before the components are removed. - Thoroughly wash and clean the engine before maintenance. - Contamination in the fuel supply injection system may cause a reduction in effectiveness / operation of engine fault indication. - If the engine is cleaned with high pressure washer, then the nozzle must be kept at a minimum distance of 200mm from the surface, and not directed at electrical components and connectors.

| 2.8.1 Supply system NOTE : The representation of fuel tank is purely indicative. Component not necessarily supplied by KOHLER . |  |
| --- | --- |
| Con pompa alimentazione meccanica § 2.6a POS.; DESCRIPTION. 1; Fuel tank. 2; Fuel hose. 3; Fuel feed pump. 4; Fuel filter. |  |
| Con pompa alimentazione elettrica § 2.6b POS.; DESCRIPTION. 1; Fuel tank. 2; Fuel hose. 3; Fuel Pre-filter. 4; Electrical fuel feed pump. 5; Fuel filter. |  |
| 2.8.2 Injection 2.7 POS.; DESCRIPTION. 1; Fuel filter. 2; Fuel hose. 3; Injection pump. 4; High pressure fuel pipe. 5; Common Rail. 6; Electronic injector. 7; Pre-chamber. |  |
| 2.8.3 Fuel return circuit The fuel return circuit is under low pressure. NOTE : The representation of fuel tank is purely indicative. Component not necessarily supplied by KOHLER . 2.8 POS.; DESCRIPTION. 1; Electrical fuel feed pump. 2; Fuel filter. 3; Injection pump. 4; Fuel return line. T; Fuel tank. |  |

| 2.8.4 Injection pump Pressure into the injection pump must be positive in all operating conditions. NOTE: In the event of leakage from the high pressure circuit do not intervene with the engine running, but turn it off and wait 5 - 10 minutes before checking the leak. 2.9 POS.; COMPONENTS DESCRIPTION. 1; Inlet suction fuel. 2; High pressure delivery. 3; Gasket. 4; Tappet. 5; Fuel intake regulating valve connector. |  |
| --- | --- |
| 2.8.5 Common Rail Fuel is injected under pressure into the Common Rail, from the high-pressure fuel injection pump. - The internal volume of the Common Rail is optimised to obtain the best compromise in order to minimise pressure peaks due to the cyclical flow of the injection pump; - Opening the electronic injectors; - The high speed response of the system to the requests of the ECU control unit. The pressure sensor measures the pressure of the fuel in the Common Rail. Pressure inside the Common Rail is regulated by the highpressure fuel injection pump by means of the fuel intake regulation valve. Important - Common Rail is NOT reparaible. - It is NOT possible to perform any maintenance on the fuel pressure sensor, as it is an integral part of the Common Rail unit. - Do NOT remove the pressure sensor from the Common Rail. - If the pressure sensor is not working, replace the entire Common Rail unit. 2.10 POS.; COMPONENTS DESCRIPTION. 1; Common Rail. 2; Fuel pressure sensor. 3; Electronic injector connections. 4; Tube inlet union from fuel injection pump. |  |

| 2.8.6 Electronic injector The electronic injector is equipped with an integral solenoid valve which, when excited electronically, manages a valve controlled from inside the electronic injector to commence fuel injection. The ECU output signal is digital. Important - The electronic injector is NOT repairable. - Fuel containing impurities causes serious damage to the electronic injectors. 2.11 POS.; COMPONENTS DESCRIPTION. 1; Inlet fuel. 2; Gasket. 3; Connector for solenoid control. 4; Gasket. 5; Gasket. |  |
| --- | --- |

| 2.8.7 Fuel filter The fuel filter is situated on the crankcase of the engine or it may be assembled on the frame of the vehicle. 2.12 POS.; COMPONENTS DESCRIPTION. 1; Fuel filter support cartridge. 2; Air bleeding screw. 3; Cartridge. 4; Water draining device. 5; Hole water drainage. |  |
| --- | --- |

| 2.8.8 Guards for fuel injection circuit components High-pressure injection circuit components are particularly sensitive to impurities. To prevent impurities, even microscopic ones, from accessing the fuel input or output unions, you are required to close these accesses by means of specific caps as soon as the various tubes are disassembled and disconnected. Disassembly of any component of the injection circuit must not occur in dusty environments. Cap guards must remain closed in their housing (ST_xx) until the moment they are to be used. Pay special attention when using the caps and avoid any contamination of dust or dirt of any kind. Even after using the caps illustrated in this paragraph, all components of the injection circuit must be placed with care in environments that are free of any type of impurity. Cap guards must be accurately washed after use and placed back in their housing (ST_xx). Important - It is highly recommended to have this page visible during disassembly operations of the components of the fuel injection circuit. | TBD |
| --- | --- |

## Lubrication circuit

| 2.9.1 Lubrication circuit diagram The oil pump is driven by the crankshaft on the timing system side. On the parts of the systems shown in green on In the parts in green, the oil is in intake, in the parts in red, the oil is under pressure and in those in yellow the oil is returning towards the oil sump 2 (not under pressure). 2.13 COLOUR; DESCRIPTION. Oil in intake. Oil under pressure. Oil returning to the oil sump. 2.14 POS.; DESCRIPTION. 1; Oil pump rotors. 2; Oil sump. 3; Crankshaft. 4; Camshaft for engine valves control. 5; Turbocharger. 6; Rocker arm pin. 7; Hydraulic tappets. 8; Rocker arm cover. 9; Cylinder head. 10; Crankcase. 11; Camshaft for injection pump control. 12 (1); Oil filter. 13 (1); Oil Cooler. 14 (1); Idle gear Housing. (1): - STD Exhaust side - Opional Intake side |
| --- |
| 2.9.2 Oil pump The oil pump rotors are trochoidal (with lobes) and are activated from the crankshaft by means of gears. It is imperative to assemble the rotors with reference A visible by the operator. 2.15 POS.; DESCRIPTION. 1; Internal rotor. 2; External rotor. 3; Oil pump crankcase. 4; Oil pump control key. 5; Crankshaft. |  |

2.9.3 Oil filter and Oil Cooler Tab 2.16 POS.; DESCRIPTION. 1; Oil arriving from the pump. 2; Oil cooling. 3; Oil Cooler. 4; Coolant. 5; Oil directly from the cartridge. 6; Oil filtering. 7; Oil filter cartridge. 8; Oil returning into the circuit.

## Cooling circuit

2.10.1 Cooling circuit diagram 2.17 POS.; DESCRIPTION. 1; Coolant pump. 2; Coolant intake. 3; Coolant, cylinder. 4; Coolant, cylinder head. 5; Coolant to radiator. 6; Coolant into radiator. 7; Thermostatic valve. 8; Coolant input into the Oil Cooler. 9; Coolant in the Oil Cooler. 10; Vent line. 11; Coolant output from the Oil Cooler.. 2.10.2 Water pump 2.18 POS.; DESCRIPTION. 1; Coolant pump control pulley. 2; Coolant intake fitting. 3; Coolant return hose from the Oil Cooler.. 2.10.3 Thermostatic valve 2.19 POS.; DESCRIPTION. 1; Cylinder head. 2; Coolant outlet cover. 3; Thermostatic valve. 4; Gaskets. Starting opening temperature of +80° ± 2°C..

## Intake and exhaust circuit

2.11.1 Intake and exhaust circuit diagram NOTE: the turbocharged is installed only for TC and TCA versions. NA versions have the air filter intake line directly installed on the intake manifold. Filtered air is sucked by the turbocharger, which compresses and sends it to the intercooler ( the Intercooler is installed only for TCA versions - as a consequence of compression, the air increases the temperature - the Intercooler cools it - this process enables better performance during combustion inside the cylinders). From the Intercooler, it is sent to the intake manifold and, via ducts in the cylinder head, enters the cylinders. Compressed air inside the cylinders and mixed with the fuel inside the prechamber transforms into Gas after combustion. The gas is expelled from the cylinders and sent to the exhaust manifold. The exhaust manifold sends the Gases to the turbocharger body (the expelled Gases activate the turbine), the Gases then proceed towards the muffler and definitely expelled. 2.20 POS.; DESCRIPTION. 1; Air in intake from air filter. 2; Air in compression. 3; Intake manifold. 4; Air in head intake. 5; Prechamber. 6; Electronic injector. 7; Glow plug. 8; Air to intake manifold hose (only for TC versions). 9; Gas in head outlet. 10; Exhaust manifold. 11; The exhaust gases are activating the turbocharger. 12; Muffler. 2.11.2 Air filter NOTE: Component not necessarily supplied by KOHLER . Important - The air filter is a dry type of filter with a paper filtering element; the filter cartridge 2 is replaceable (refer to Tab. 4.1 for procedure frequency on components). - Filter suction must be positioned in a cool place. 2.21 POS.; DESCRIPTION. 1; Filter support. 2; Air filter cartridge. 3; Filter cover. 4; Dust exhaust valve.

## Electric system

| 2.12.1 ECU input and output signals diagram 2.22 SENSORS/SWITCH (INPUT); DEVICES (OUTPUT). Power relay; ECU; Electronic injector 1. Engine revolutions sensor; Electronic injector 2. Engine phase sensor; Electronic injector 3. Coolant temperature sensor; Fuel flow control. Common Rail pressure sensor; Revolution indicator. Oil pressure switch; Glow plugs Relay. Fuel temperature sensor; Diagnosis indicator lights. T-MAP sensor; Electric fan control (1-2 speeds or variable speed). Main accelerator pedal (double track); CAN 1 (ISO15765 diagnostics). Secondary accelerator pedal (optional); CAN 2 (Vehicle SAE J1939). Hydraulic oil pressure sensor (optional). Fuel level sensor (optional). Air filter clogging sensor (optional). Water in fuel sensor. |
| --- |
| 2.12.2 Control unit (ECU) The ECU is a central processor, which monitors and controls engine operation. The electronic control unit is responsible for engine management. It is fitted on the frame of the vehicle, or in the cab (refer to the technical documentation of the vehicle). Important - The ECU must only be used with the configuration defined by KOHLER , for each individual engine. |  |
| Tab. 2.23 ECU AND ENGINE IDENTIFICATION PLATES. POS.; DESCRIPTION. 1; Engine model. 2; Engine specifications. 3; Engine serial number. 4; Datamatrix. 5; Validation code. 6; ECU identification code. - Do NOT mount or replace the control unit with that of another engine. - Although externally each ECU seems to be identical, internally they are specifically configured only for use on the engine that they are supplied with. - To install a new control unit, is required to recharge on it's the original configuration relating to that specific engine . - The control units are not interchangeable nor modifiable. - Each control unit is accompanied by its adhesive identification plate. |
| 2.12.4 Engine electrical wiring. |

| Tab. 2.24 RIF.; DESCRIZIONE. 1; Vehicle interface connector. 2; ECU Connector E. 3; ECU Connector C. 4; CANR. 5; Ground. 6; D+ Connector Alternator. 7; Starter motor connector. 8; Engine speed sensor connector. 9; T-MAP sensor connector. 10; Electronic Injector connector 1. 11; Common Rail pressure sensor connector. 12; Electronic Injector connector 2. 13; Electronic Injector connector 3. 14; Coolant temperature sensor connector. 15; Fuel flow control. 16; Engine phase sensor connector. 17; Oil pressure switch connector. 18; Diagnosis Connector. 19; Doubble track accelerator connector. |  |
| --- | --- |
| 2.12.4.1 Wiring disconnection All sensor connectors and electronic control devices are sealed. The connectors must be disconnected by means of pressure on tabs A or unblock the retainers B . |  |

## Sensors and switches

| 2.13.1 Revolution sensor on target wheel Speed sensor A is situated on the crankcase. The sensor detects the signal from the target wheel B situated on the flywheel. It sends it to the ECU as an analogical signal. The sensor sends and analogue signal to the ECU. The sensor produces a 5V square wave signal having a Hall effect while the flywheel in rotation detects its position and speed. The data sent by this sensor enables the ECU to pilot fuel anticipation injection for each piston. |  |
| --- | --- |
| 2.13.2 Camshaft sensor Camshaft sensor C is situated on the timing system carter. The purpose of the camshaft sensor C is to identify the position of the Camshaft control gear D with respect to the engine shaft and consequently the position of the pistons with respect to the T.D.C. The sensor produces a 5V square wave signal having a Hall effect while the camshaft in rotation detects the phases of the 4 strokes of the 1st cylinder. As a consequence, ECU by means of internal calculations, also recognises the phases of the other cylinders. The data sent by this sensor enables the ECU to pilot fuel anticipation injection for each piston. |  |
| 2.13.3 T-MAP sensor The T-MAP F sensor is situated on the intake manifold. It detects the input pressure in the intake manifold by means of electrical voltage variation and the air temperature by means of an electrical resistor. The sensor sends signals to the ECU, which determines the values and modifies the injection strokes. Tab. 2.35 reports the electrical resistor values according to the intake air temperature. NOTE : R indicates the pin where it is possible to measure electrical resistance. Tab 2.25 °C (°F); R ( Ω ). -30 (-22); 23475 - 25945. 0 (32); 5370 - 5935. 25 (77); 1900 - 2100. 50 (122); 772 - 854. 100 (212); 177 - 195. 120 (248); 107 - 119. |  |
| 2.13.4 Common Rail pressure sensor Fuel pressure sensor G assembled on the Common Rail, detects the fuel pressure inside it by means of electrical voltage variation. Depending on the signal sent, ECU manages the fuel intake valve on the injection pump and, if necessary, modifies the injection strokes. |  |
| 2.13.5 Oil pressure switch Oil pressure switch N is assembled on the crankcase near to the injection pump. It is a N/C sensor, calibrated at 0.6 bar ± 0.1 bar. With oil low pressure the sensor closes the electrical circuit and the warning lamp in the panel board switches on. |  |
| 2.13.6 Coolant temperature sensor The P coolant temperature sensor of the coolant circuit is applied to the cylinder head on the side of the thermostatic valve. It is used by the ECU in order to obtain information regarding the coolant temperature. 2.26 CHARACTERISTICS. Temperature °C; R min Ω; R max Ω. -40; 38720; 49720. 0; 5669; 6407. +140; 77; 98.8. |  |
| 2.13.7 Air cleaner clogging switch NOTE: Component not necessarily supplied by KOHLER. The switch is assembled on the air cleaner. When the filter is clogged, it sends a signal to the panel. |  |

## Electrical components

| 2.14.1 Alternator Externally controlled by the crankshaft by means of a belt. | 12V-45A |
| --- | --- |
|  | 12V-80A 12V-100A |
| 2.14.2 Starter Motor - Type Bosch 12 V - Potenza 3.2 kW - Anticlockwise rotation (seen from timing system side) |  |
| 2.14.3 Glow plugs Glow plugs A are located on the engine head, at low temperature the glow plugs will activate heating the prechamber B facilitating the fuel combustion sprayed by the injectors C . |  |
| 2.14.4 Electric fuel pump (optional) NOTE: Component not necessarily supplied by KOHLER. The electric pump is located before the fuel filter. 2.27 POS.; Description. B; Electrical connection. C; Pump Prefilter. IN; Ingoing fitting (IN) from tank. OUT; Outgoing fitting (out) to fuel filter. A; Value. Voltage; 12 V. Delivery; 60.56 L/h @ 0.41 bar. |  |

## Timing system and tappets

| The timing system is equipped with hydraulic tappets that automatically recover the operation of the rocker rods assembly. No registration is therefore required. 2.15.1 Components identification |  |
| --- | --- |
| 2.28 POS.; DESCRIPTION. 1; Crankshaft. 2; Camshaft. 3; Camshaft tappets. 4; Rocker arm control rod. 5; Rocker arms. 6; Valves. 7; Injection pump camshaft control gear. 8; Engine valves camshaft control gear. 9; Crankshaft gear. 10; Idle gear. 11; Valve tappet. 12; Hydraulic tappets. |  |
| 2.15.2 Rocker arm pin 2.29 POS.; DESCRIPTION. 1; Rocker arm pin. 2; Rocker arm distancing spring. 3; Rocker arm pin support. 4; Exhaust rocker arm. 5; Intake rocker arm. |  |
| 2.15.3 Rocker arms 2.30 POS.; DESCRIPTION. 1; Rocker arm body. 2; Oil flow line. 3; Hydraulic tappet oil refill line. 4; Valve tappet. 5; Hydraulic tappet. |  |
| 2.15.4 Hydraulic tappets 2.31 POS.; DESCRIPTION. A; Hydraulic tappets. B; Hight pressure chamber. 1; Hydraulic tappets oil refill pipe. 2; Retaining ring. 3; Piston. 4; Unidirectional valve. 5; Tappet body. 6; Spring. 2.15.4.1 Hydraulic tappet operation The operating principle of the hydraulic tappet is based on the incompressibility of the liquids and on controlled leakage. The oil under pressure enters the tappet chamber A , providing a constant supply of oil in the low-pressure chamber. Through the non-return valve, 4 the oil can only access the high-pressure chamber B and exit via the clearance between the piston 3 and the tappet body 5 (controlled leakage). The chamber B is filled when the rocker arm is on the base radius of the cam and the spring 6 keeps the piston 3 against the valve stem, thus eliminating any system play. Thanks to the spring extension, the tappet "extends", creating a small depression in the chamber B , making the non-return valve 4 open, and allowing the oil in the chamber A to pass to chamber B , restoring the proper amount of oil required to eliminate any play in the valves. |  |

2.15.4.2 Difficult operating conditions For proper operation on the hydraulic tappets it is essential that the low pressure chamber of the piston 3 is always full of oil. In some conditions this may not occur (due to the fact that the oil leaks away when the engine is switched off, which can also partially drain the tappets). This situation will be the cause of clearances that will result in a characteristic noise similar toa ticking sound. - When the engine is cold, the tappet filling time could be very long if the oil used is not suitable for the specific environmental conditions ( Tab. 2.2 ) - If the engine is very hot: at idle speed, oil pressure may be low, and small air bubbles could form in the circuit. Because of this, this compressing the tappet slightly and producing valve play which is responsible for the ticking sound. On account of this, the tappet compresses slightly giving rise to a valve clearance, thus generating a slight ticking sound, which however disappears rapidly ( MAX 10 seconds) once normal operating conditions have been restored. Anyway the duration of ticking Anyway the duration of ticking sound must be MAX 30 seconds. If not, the problem is surely due to the poor quality of the oil, wear or impurities that, transported by the oil, can infiltrate between the ball valve and its seat inside the piston, compromising the operation of the tappet itself; In these cases, the only solution is to replace the oil or hydraulic tappets. The prolonged persistence of the ticking sound or abnormal noise must be investigating in order to prevent any malfunctions; if necessary, replace the hydraulic tappets and engine oil.

## Components handling

| 2.16.1 High-pressure fuel injection pump - Only handle by means of the points marked by Y . - It is forbidden to handle using the points marked by N . |  |
| --- | --- |
| 2.16.2 Electronic injector - Only handle by means of the points marked by Y . -It is forbidden to handle using the points marked by N . |  |
| 2.16.3 Common Rail - Only handle by means of the points marked by Y . - It is forbidden to handle using the points marked by N . |  |
| 2.16.4 Turbocharger - Only handle by means of the points marked by Y . - It is forbidden to handle using the points marked by N . Important - Refer to Par. 2.18 . |  |

## Turbocharger

| 2.17.1 What to do and what not to do What to do: - Before assembling the turbocharger, make sure that the protection caps are fitted on all openings of the turbo. - Ensure pre-lubrication of the turbocharger. - Periodically check that the joints are sealed against oil and air. - Use lubricating oil according to the specifications described in Par. 2.4 . - Check the engine oil level. - Before switching it off after it has been used, make the engine run idle, or without a load, for approximately 1 minute. - Ensure that controls and maintenance intervals of the engine are observed as specified in Chapt 4 . - Make sure that the engine and equipment are used correctly so as not to compromise the life of the turbocharger. | What not to do: - Do not store turbocharges in damp, wet places if they are not in their original packaging. - Do not expose the turbocharger to dust and dirt if it is not in its original packaging. - Do not lift of hold the turbocharger from the actuator rod if it is not in its original packaging. - Do not apply additives to the lubricating oil and fuel, unless instructed to do so by Kohler. - Do not increase engine speed, or apply loads, immediately after start-up. - Do not intervene on the actuator settings A . - Do not let the vehicle / engine run at idle speed for more than 20-30 minutes at a time. |
| --- | --- |
| 2.17.2 Practical operating rules Users can help to maximise the duration of their turbocharger by following the rules described below. - Start-up Start the engine at idle speed, or without a load, for approximately one minute. Oil operating pressure is reached within a few seconds and enables the moving parts to warm up and be lubricated. Immediately increasing the engine speed upon start-up means making the turbocharger run at high speed with suboptimal lubrication, which may compromise the life of the turbocharger. - After maintenance or a new installation Proceed with pre-lubrication by filling new oil into the oil supply duct B until filling it completely. Start the engine at idle speed, or without a load, for a few minutes in order to ensure that the oil and bearings system operate satisfactorily. - Low temperature air or engine inactivity If the engine has been inactive for some time, or the air temperature is very low, start the engine at idle speed or without a load for a few minutes. - Engine shutdown Before switching the engine off after intense activity, one must allow the turbocharger to cool down. One must therefore let the engine run at idle speed or without a load for at least 2 minutes, thus allowing the turbocharger to cool. - Engine at idle speed Avoid using the engine at idle speed or without a load for long periods (more than 20-30 minutes). When operating at idle speed or without a load, the turbocharger is at low pressure in the exhaust chamber C and air supply D ; this may cause oil leaks from seals E to the extremity of the shaft. Even if this does not cause damage, it can cause blue smoke from the exhaust when the engine speed and load are increased. |  |
| 2.17.3 Before installing a new turbocharger Important - Do not lift the turbocharger with one hand from the box. - Do not lift turbocharger from Comp hsg side. - Lift the turbocharger with both hands from box. - Make sure to use clean gloves. - Handle the turbocharger as indicated in Par. 2.16. |  |
| - Avoid lifting from the intake side G . - Remove cap guard F and check that there is no excessive shaft axial and radial clearances. |  |
| - Check for any signs of friction of the turbine on the turbocharger body. - Check for any traces of oil leaks on the turbocharger body. - After having check everything, reapply cap F on intake opening H of the turbocharger and do not remove it until assembly has been completed. |  |
| - Check the correct assembly of the capscrews and the presence of paint on them. |  |
| 2.17.4 Installation instructions - Remove the cap guards with care only when assembling. Handle carefully avoiding erratic movements. |  |
| 2.17.5 Replacement instructions Always understand the cause of the breakage of the turbocharger before replacing it. Correct the cause of the breakage before replacing it with a new turbocharger. If in doubt, contact KOHLER service department. Important - Failure to comply with these instructions can cause damage to the turbocharger and void the warranty. - Modifying the calibration of the turbocharger damages the turbocharger/engine. - Always use the correct gaskets, and fit carefully to avoid blocking holes when mounting. - Refer to the manual of the engine / vehicle, for: the correct type and quantity of oil, the correct tightening of components, instructions and installation. - It is forbidden to use liquid gaskets or sealants, particularly for the oil inlet/outlet. - Avoid dirt / debris while installing the turbocharger. - Before mounting the turbocharger, check that the code of the component is correct for the type of engine, as mounting the wrong turbocharger can damage the turbo / engine and void the warranty. |  |

# Safety information

## Before start-up

- Read the manual carefully and carry out the operations described below in compliance with the instructions specified. - Periodic inspection and maintenance operations must be carried out as indicated in this manual and under the user's responsibility. Important - Only use original spare parts and accessories. - The use of non-original parts, as well as voiding the warranty, affects the life and performance of the engine, and may be dangerous. - Non compliance with the operations described in the following pages may result damage to the engine and vehicle on which it is installed, as well as to people and/or property.

## Safety information

- The intended use of the engine is in conformity with the machine on which it is mounted. - Any use of the machine other than that described cannot be considered as complying with its intended purpose as specified by KOHLER . - KOHLER declines all responsibility for any change to the engine not described in this manual made by unauthorized KOHLER personnel. - A proper use of the engine, a strict observance of the rules listed below and the rigorous application of all these precautions will avoid the risk of accidents or injuries. - Those who carry out the use and maintenance on the engine must wear the safety equipment and the accident-prevention guards. - KOHLER declines all direct and indirect liability for failure to comply with the standards of conduct contained in this manual. - KOHLER is not responsible for any unreasonable use or neglect that may cause a potential danger.

## General remarks

3.2.1 Note for OEM

- When installing the KDI engines, always bear in mind that any variation to the functional systems may result in serious failures to the engine.

- Any improvement must be verified at KOHLER testing laboratories before application of the engine.

- In case the approval to a modification is not granted, KOHLER shall not be deemed responsible for any consequential failures or damages to the engine.

- Those who carry out the use and maintenance on the engine must wear the safety equipment and the accident-prevention guards.

- KOHLER declines all direct and indirect liability for failure to comply with the standards of conduct contained in this manual.

- KOHLER cannot consider every reasonably unforeseeable misuse that may cause a potential danger.

3.2.2 Note for end user

- The following indications are dedicated to the user of the machine in order to reduce or eliminate risks concerning engine operation and the relative routine maintenance work.

- The user must read these instructions carefully. Failure to do this could lead to serious danger for his personal safety and health and that of any persons who may be in the vicinity of the machine.

- On starting, make sure that the engine is as horizontal as possible, unless the machine specifications differ.

- Make sure that the machine is stable to prevent the risk of overturning.

- The engine must not operate in places containing inflammable materials, in explosive atmospheres, where there is dust that can easily catch fire unless specific, adequate and clearly indicated precautions have been taken and have been certified for the machine.

- To prevent fire hazards, always keep the machine at least one meter from buildings or from other machinery.

- Children and animals must be kept at a due distance from operating machines in order to prevent hazards deriving from their operation.

- Thoroughly wash and clean all the external parts of the engine before performing any operation, in order to avoid the accidental introduction of impurities/foreign bodies. Use only water and/or appropriate products to clean the engine. If cleaning engine with a pressure washer or steam cleaner, it is important to maintain a minimum distance of at least 200mm between the surface to be washed and the nozzle. Avoid directing the nozzle on electrical components, cable connections and sealed rings (oil seals etc). Thoroughly wash and clean the area surrounding the engine following the instructions provided by machine manufacturer.

- Fuel and oil are inflammable. The tank must only be filled when the engine is off. Before starting, dry any spilt fuel.

- Make sure that no soundproofing panels and the ground or floor on which the machine is standing have not soaked up any fuel.

- Fuel vapour is highly toxic. Only refuel outdoors or in a well ventilated place

- Do not smoke or use open flames when refuelling.

- During operation, the surface of the engine can become dangerously hot. Avoid touching the exhaust system in particular.

- Before proceeding with any operation on the engine, stop it and allow it to cool.

- Always open the radiator plug or expansion chamber with the utmost caution, wearing protective garments and goggles.

- The coolant fluid is under pressure. Never carry out any inspections until the engine has cooled.

- If there is an electric fan, do not approach the engine when it is still hot as the fan could also start operating when the engine is at a standstill.

- The oil must be drained whilst the engine is hot. Particular care is required to prevent burns. Do not allow oil to come into contact with the skin because of the health hazards involved. It is recommended to use an oil intake pump.

- During operations that involve access to moving parts of the engine and/or removal of rotating guards, disconnect and insulate the negative wire (-) of the battery to prevent accidental short-circuits and to stop the starter motor from being energized.

- Check belt tension only when the engine is off.

- Fully tighten the tank cap each time after refuelling. Do not fill the tank right to the top but leave an adequate space for the fuel to expand.

- To start the engine follow the specific instructions provided in the engine and/or machine operating manual. Do not use auxiliary starting devices not originally installed on the machine (e.g. Startpilot).

- Before starting, remove any tools that were used to service the engine and/or machine. Make sure that all guards have been refitted.

- Do not mix fuel with elements such as oil or kerosene. Failure to comply with this prohibition will cause the non-operation of the catalyst and non-observance of the emissions declared by KOHLER .

- Pay attention to the temperature of the oil filter when the filter itself is replaced.

- Only check, top up and change the coolant fluid when the engine is off and reached the ambient temperature. Coolant fluid is polluting, it must therefore be disposed of in the correct way.

- Do not use air and water jets at high pressures on cables, connectors and injectors.

Important

- Only use the eyebolts A installed by KOHLER to move the engine (Fig. 3.1) .

- The angle between each lifting chain and the eyebolts shall not exceed 15° inwards.

- The correct tightening of the lifting screws is 25Nm.

- Do not interpose spacers or washers between the eyebolts and engine head.

- Engines equipped with an ATS device require regeneration to be inhibited if the engine is used in environments at risk of fire (e.g. forest areas, areas with flammable materials, areas with flammable gases or liquids and any type of combustible material - if the function is available).

Fig 3.1

## Safety signal description

- To ensure safe operation please read the following statements and understand their meaning. - Also refer to your equipment manufacturer's manual for other important safety information. - This manual contains safety precautions which are explained below. - Please read them carefully. Adhesive safety plates The following is a list of the adhesive safety plates that may be found on the engine, which indicate potential points of danger to the operator.. Read the Operation and Maintenance handbook before performing any operation on the engine.. Hot Parts. Danger of burns.. Presence of rotating parts. Danger of jamming or cutting.. Presence of explosive fuel. Danger of fire or explosion.. Presence of steam and pressurized coolant. Danger of burns.. Warnings Hereunder is a list of safety warnings that may be found in the manual, which advise you to pay attention when carrying out particular procedures that may be potentially dangerous to the operator or things.. Danger This indicates situations of grave danger which, if ignored, may seriously threaten the health and safety of individuals.. Important This indicates particularly important technical information that should not be ignored.. Warning This indicates that failure to comply with it can cause minor damage or injury.. Safety guards Hereunder is a list of safety guards that must be worn prior to carrying out any type of operation and to avoid potential harm to the operator.. Use suitable protective gloves before carrying out any type of operation.. Use protective goggles before carrying out any type of operation.. Use earmuffs before carrying out any type of operation..

## Information and safety signals

| ACCIDENTAL START. Accidental Starts can cause severe injury or death.. Before working on the engine or equipment, disconnect the battery negative (-) wire.. HOT PARTS. Hot Parts can cause severe burns.. Engine components can get extremely hot from operation. Do not touch engine while operating or just after stopping. Never operate the engine with heat shields or guards removed.. ROTATING PARTS. Rotating Parts can cause severe injury.. Stay away while engine is in operation. Keep hands, feet, hair, and clothing away from all moving parts to prevent injury. Never operate the engine with covers, shrouds, or guards removed.. LETHAL EXHAUST GASES. Carbon Monoxide can cause severe nausea, fainting or death.. Avoid inhaling exhaust fumes and never run the engine in a closed building or confined area. Carbon monoxide is toxic, odorless, colorless, and can cause death if inhaled.. ELECTRICAL SHOCK. Electrical Shock can cause injury.. Do not touch wires while engine is running.. | HIGH PRESSURE FLUID RISK OF PUNCTURE. High Pressure Fluids can puncture skin and cause severe injury or death.. Do not work on fuel system without proper training or safety equipment. Fluid puncture injuries are highly toxic and hazardous. If an injury occurs, seek immediate medical attention.. EXPLOSIVE FUEL. Explosive fuel can cause fires and severe burns.. Fuel is flammable and its vapours can ignite. Store fuel only in approved containers, in well ventilated, unoccupied buildings. Do not fill the fuel tank while the engine is hot or running, since spilled fuel could ignite if it comes in contact with hot parts or sparks from ignition. Do not start the engine near spilled fuel. Never use fuel as a cleaning agent.. EXPLOSIVE GAS. Explosive Gas can cause fires and severe acid burns.. Charge battery only in a well ventilated area. Keep sparks, open flames, and other sources of ignition away from the battery at all times. Batteries produce explosive hydrogen gas while being charged. Keep batteries out of the reach of children. Remove all jewelry when servicing batteries. Before disconnecting the negative (-) ground cable, make sure all switches are OFF. If ON, a spark will occur at the ground cable terminal which could cause an explosion.. CALIFORNIA WARNING - DECLARATION 65. Engine exhaust from this product contains chemicals known to the State of California to cause cancer, birth defects, or other reproductive harm.. |
| --- | --- |

## Safety and environmental impact

Every organisation has a duty to implement procedures to identify, assess and monitor the influence of its own activities (products, services, etc.) on the environment. Procedures for identifying the extent of the impact on the environment must consider the following factors: - Liquid waste. - Waste management. - Soil contamination. - Atmospheric emission. - Use of raw materials and natural resources. - Regulations and directives regarding environmental impact. In order to minimise the impact on the environment, KOHLER now provides a number of indications to be followed by all persons handling the engine, for any reason, during its expected lifetime. - All components and fluids must be disposed of in accordance with the laws of the country in which disposal is taking place. - Keep the fuel and engine control systems and the exhaust pipes in efficient working order to limit environmental and noise pollution. - When discontinuing use of the engine, select all components according to their chemical characteristics and dispose of them separately.

# Maintenance procedures

## Cleaning and checking

| Warning - Inspections must be made when the engine is off and cold. - Place engine on level sur face to ensure accurate measurement of oil level. Danger - For safety precautions see Chap. 3 . The intervals of preventive maintenance in Tab. 4.1, Tab. 4.2, Tab. 4.3 and Tab. 4.4 refer to the engine operating under normal operating conditions with fuel and oil meeting the approved specifications. 4.1 CLEANING AND CHECKING. OPERATION DESCRIPTION; PERIOD (HOURS). 100; 500. Engine oil level (8). Coolant level (8) (9). Water presence in fuel filter. Alternator belt (8). Rubber hoses (intake air / coolant) - Fuel hose (3). Starter Motor (1). Alternator (1). (1) - In case of low use: 12 months. (2) - The period of time that must elapse before checking the filter element depends on the environment in which the engine operates. The air filter must be cleaned and replaced more frequently under very dusty conditions. (3) - In case of low use: 36 months. (4) - In case of low use: 5 years. (6) - Contact authorized KOHLER workshops. (7) - The replacement interval is only an indication, it strongly depends from environmental condition and hose status detected during regular visual inspection. (8) - The first check must be done after 10 hours. (9) - Test the coolant condition annually with coolant test strips. (10) - It is recommended to have SCA (Supplemental Coolant Additives) added at the first maintenance interval. |
| --- |
| Engine oil level - Before checking oil engine needs to be level. - Remove the oil dipstick B and check that the level is up to but does not exceed the MAX . - If level is not at the MAX . level, add additional oil. - Re-tighten the cap A, C . |  |
| Coolant level - Loosen the cap A . - Top liquid up until the pipes inside the radiator are covered by about 5 mm. - For engines equipped with expansion tank, pour in fluid until reaching the max level mark. - Tighten the radiator cap A or the expansion tank ( C ) cap B . |  |
| Water presence in fuel filter Important - Before proceeding with operation, read Par. 3.2.2 . Danger - For safety precautions see Chap. 3. - Gently loosen the water drain plug A without removing it. - Spill out the water if present. - Re-tighten the water drain plug A as soon as the fuel spills. |  |
| Check of the radiator heat - exchanger surface Danger - For safety precautions see Chap. 3 . NOTE: Component not necessarily supplied by KOHLER . Important - Before proceeding with operation, read Par. 3.2.2 . - Wear safety goggles when using compressed air. - The radiator heat-exchange surface must be cleaned on both. - Check the radiator heat-exchange surface A . - Clean the surface with a brush soaked in special detergent if it is clogged. |  |
| Alternator belt Important - Before proceeding with operation, read Par. 3.2.2 . Danger - For safety precautions see Chap. 3 . NOTE : The belt is not adjustable. - Check the belt A condition, if worn out or deteriorated, replace it. NOTE : Make sure that the ribs of the belt A are inserted correctly into the grooves of the pulleys B . |  |
| Rubber hoses (intake air / coolant / fuel) Danger - For safety precautions see Cap. 3. The check is carried out by exerting a slight deflection or bending along the pipe and near the hose clamps. Components must be replaced if they have clear signs of cracks, tears, cuts, leaks and do not retain a certain degree of elasticity. Important - Before proceeding with operation, read Par. 3.2.2 . - If hoses are damaged contact an authorized KOHLER workshop. - For other pipes not illustrated refer to the technical documentation of the vehicle. - Check the integrity of the pipes and hoses A . |  |
| - Externally check the condition and clean motor A using compressed air. |  |
| - Externally check the condition and clean the alternator A using compressed air. |  |

## Replacement

| - Periodic inspection and maintenance operations must be carried out as indicated in this manual and are the responsability of the user. - Failure to comply with these service and maintenance intervals increases the risk of technical damage to the engine. Any non compliance makes the warranty become null and void. - In order to prevent personal and property damage read carefully the instructions listed below before proceeding with any operation of the engine. Warning - Inspections must be made when the engine is off and cold. - Place engine on level sur face to ensure accurate measurement of oil level. - Before starting, to avoid spillages of oil make sure that: - the oil dipstick is inserted correctly; - also check that: oil drain plug and oil filler cap are tightened firmly. Important - Before proceeding with operation, read Par. 3.2.2 . Danger - For safety precautions see Chap. 3 . The intervals of preventive maintenance in Tab. 4.1, Tab. 4.2, Tab. 4.3 and Tab. 4.4 refer to the engine operating under normal operating conditions with fuel and oil meeting the approved specifications. 4.2 REPLACEMENT. OPERATION DESCRIPTION; PERIOD (HOURS). 500; 2000; 4000. Air filter Cartridge (2 ). Alternator belt ( 3 ). Rubber hoses (intake air / coolant) (4 ) (6) (7). Fuel line hose ( 4 ) (6) (7). Coolant; OAT ( 4 ) (6). HOAT ( 4 ) (6) (10). 4.3 ENGINE OIL AND OIL FILTER CARTRIDGE REPLACEMENT. ENGINE VERSION; PERIOD (HOURS). 500; 1000. Compact Sump (1). Deep Sump (1). 4.4 FUEL FILTER AND PREFILTER CARTRIDGE REPLACEMENT. ENGINE VERSION; PERIOD (HOURS). 500; 1000. STD Filtration (1). High Filtration (1). (1) - In case of low use: 12 months. (2) - The period of time that must elapse before checking the filter element depends on the environment in which the engine operates. The air filter must be cleaned and replaced more frequently under very dusty conditions. (3) - In case of low use: 36 months. (4) - In case of low use: 5 years. (6) - Contact authorized KOHLER workshops. (7) - The replacement interval is only an indication, it strongly depends from environmental condition and hose status detected during regular visual inspection. (8) - The first check must be done after 10 hours. (9) - Test the coolant condition annually with coolant test strips. (10) - It is recommended to have SCA (Supplemental Coolant Additives) added at the first maintenance interval. |
| --- |
| Engine oil and oil filter |  |
| NOTE : Perform this operation with warm engine, to get a better fluidity of the oil and get a full discharge of oil and impurities contained in it. - Loosen the oil filler cap A or the oil filler cap C if the cap A is not accessible. |  |
| - Unscrew the oil filter F with appropriate wrench. - Any failure resulting from the use of fuel other than Tab. 2.3, 2.4 will not be covered by warranty. |  |
| - Remove the oil dipstick B . - Remove the oil drain plug D and the gasket E (the oil drain plug is on both sides of the oil sump). - Drain oil in an appropriate container. - Replace gasket E . - + apply Loctite 242 on the thread of plug D . - Tighten the drain oil plug D (tightening torque at 40 Nm ). |  |
| - Add the type of oil approved ( Tab. 2.2 ). - Fit and remove the oil dipstick B to check the level. - Pour in fluid until reaching the MAX level mark. - Upon completion, reinstall the oil dipstick B completely. - Tighten the cap A, C . |  |
| Fuel filter cartridge |  |
| Important - Do not fill the new cartridge B with fuel. - Procure a suitable container to collect the fuel. - Loosen and remove cartridge B . - Lubricate the gasket C . - Tighten the new cartridge B onto support D (tighten manually). |  |
| - Replace the prefilter H if present. - Perform the operations described in Par. 5.4 . |  |
| Air filter cartridge |  |
| Importante - Before proceeding with operation, read Par. 3.2.2 . NOTE : Components not necessarily supplied by KOHLER . - Release the two fastenings F of the cover A . - Remove the cartridge B . - Clean the inside components A and D with a damp cloth. - Reinstall: - the new cartridge B . -the cover A checking the right tightness of fastenings F . |  |
| Alternator belt |  |
| - Remove the belt A using tool ST_57 . - Install the new belt A using tool ST_57 . | REMOVE INTALL |
| Rubber hoses |  |
| - Undo the cap A carefully (circuit under pressure). - Drain all the liquid inside the radiator via shut-off valve D into a suitable container. |  |
| - Loosen the screws E and remove the fuel filter bracket G . - Unscrew cap F , remove gasket H to allow all the system fluid contained inside the pipes in the engine crankcase to drain. - Tighten the plug F (tightening torque at 35 Nm ). - Fasten the fuel filter bracket G with screws E (tightening torque 25 Nm ). |  |
| Replace the hoses B . |  |
| - Loosen the cap A and fill the radiator with coolant composed of: 50% ANTIFREEZE and 50% decalcified water. - Top liquid up until the pipes inside the radiator are covered by about 5 mm. Do not overfill the radiator, but leave room for the coolant to expand. - For engines equipped with expansion tank, pour in fluid until reaching the max level mark. - Re-tighten the cap A . - Keep it running at idle speed until the cooling liquid level goes down and becomes steady (the waiting times varies according to the ambient temperature). - Stop the engine and allow it to cool. - If there is an expansion tank ( C ) top liquid up to the mark MAX . - Without expansion tank top liquid up until the pipes inside the radiator are covered by 5 mm. Do not overfill the radiator, but leave room for the coolant to expand. - Tighten the radiator cap A or the expansion tank cap. |  |
| Fuel line hose |  |
| Replace the hoses B . |  |

# Service procedures

## Preliminary information

Important

- The operator should prepare all equipment and tools in order to enable him to carry out the operations correctly and safely.

- Before proceeding with operation, carefully read Chap. 3 .

- In order to operate safely and easily, we recommend positioning the engine on a rotating stand for engine overhauling.

- Seal all injection component unions as illustrated in Par. 2.8.8  during assembly.

- Protect all disassembled components and coupling surfaces subject to oxidation with lubricant.

- Where necessary, reference to special tools to use during disassembly operations is indicated (es. ST_05 ).

- Replace all gaskets or rubber components at each disassembly.

## Cooling circuit

| DRAINING |  |
| --- | --- |
| Radiator NA - TC model |  |
| - Uncscrew the cap A1 of the radiator A to drain the remaining coolant. |  |
| Radiator TCA model |  |
| - Uncscrew the cap A1 of the radiator A to drain the remaining coolant. |  |
| DISASSEMBLING |  |
| Radiator NA - TC model |  |
| - Loosen the clamps B and remove the hose C . - Unscrew the nuts D and the screw E to remove the rods F1 , F2 . |  |
| - Loosen the clamps B and remove the hose C . - Unscrew the nuts D to remove the radiator A . |  |
| Radiator TCA model |  |
| - Loosen the clamps B and remove the hose C . - Unscrew the nuts D and the screw E to remove the rods F1 , F2 . |  |
| - Loosen the clamps G and remove the hose H . - Loosen the clamps B1 and remove the hose L . |  |
| - Unscrew the nuts D to remove the radiator A . |  |
| Oil Cooler hoses |  |
| - Loosen the clamps B and remove the hoses C . |  |
| NOTE: the oil cooler has various configurations and can be installed also on the intake side. - Unscrew the screws E1 and clamps B1 if present. |  |
| Fan and belt |  |
| - Unscrew the screws G and remove the fan H . |  |
| - Remove the belt L using ST_57 tool. | DISASSEMBLING |
| Coolant pump |  |
| - Unscrew the screw M and remove the pulley N . |  |
| - Loosen the clamps B and remove the hose C . |  |
| - Do not disassemble the flange R . NOTE: the flange R for pulley N can be intalled in various positions based from engine specification, each configuration has a specific belt and pulley. It is not necessary to remove the flange R for pulley N to proceed with following operations below. In case of flange disassembling, take note of its original position to avoid a wrong positioning at the assembling. - Apply Loctite 242 on the thread of screws (tightening torque 10 Nm ). - Unscrew the screws P and remove the coolant pump Q . |  |
| Thermostatic valve |  |
| - Unscrew the screws W1 and remove the cover W2 . |  |
| - Remove the thermostatic valve W3 . |  |
| Thermostatic valve specifications Opening start; Opening stop. 0.1 mm; @ 80° C; 7 mm; @ 95° C. |  |
| Radiator support |  |
| - Unscrew the screws S and remove the radiator support T . |  |
| ASSEMBLING |  |
| Radiator support | Option A (screws M10 - tightening torque at 45 Nm ) Option B (screws M8 - tightening torque at 25 Nm ) |
| - Fix the radiator support T on the crankcase V using the screws S and washers S1 . |  |
| Thermostatic valve |  |
| - Place the thermostatic valve W3 inside the seat of cylinder head W . - Place the gasket W4 inside the seat of cover W2 . - Apply Loctite 270 on the thread of screw W1 . § - Fix the cover W2 with gasket W4 on the cylinder head W using the screws W1 (tightening torque 10 Nm ). NOTE: make sure that the gasket W4 stays in its seat during cover W2 assembly process. |  |
| Coolant pump |  |
| - Fix the coolant pump Q on the crankcase V with the gasket Q1 using the screws P (tightening torque 25 Nm ). |  |
| - Install components N1 , N2 inside pulley N . - Apply Loctite 242 on the thread of screw M . § - Fix the pulley N on flange R using the screw M (tightening torque 25 Nm ). |  |
| Fan and belt | INSTALLING |
| - Install the new belt A on pulleys N , Q2 , Z1 , Z2 using the ST_57 tool. |  |
| - Apply Loctite 242 on the thread of screw G . § - Fix the fan H with the plate H1 and the spacer H2 on pulley N (tightening torque 10 Nm ). NOTE: the spacer H2 can be different based on engine specification. |  |
| Oil Cooler hoses |  |
| - Fix the hoses C using the clamps B . |  |
| NOTE: the oil cooler has various configurations and can be installed also on the intake side. - Fix the clamps B1 using screws E1 if present. |  |
| Radiator NA - TC model |  |
| - Place the radiator A on support T . - Fix the radiator A using nuts D (tightening torque 25 Nm ). - Fix the hoses C using the clamps B . |  |
| - Place the rods F1 , F2 for the radiator fixing A . - Fix the rods F1 , F2 using the nuts D with washers D1 and screws E (tightening torque 10 Nm ). - Fix the hoses C using the clamps B . |  |
| Radiator TCA model |  |
| - Place the radiator A on support T . - Fix the radiator A using nuts D (tightening torque 25 Nm ). - Fix the hoses C using the clamps B . |  |
| - Place the rods F1 , F2 for the radiator fixing A . - Fix the rods F1 , F2 using the nuts D with washers D1 and screws E (tightening torque 10 Nm ). |  |
| - Fix the hoses C using the clamps B . - Fix the hoses H using the clamps G . - Fix the hose L using the clamps B1 . |  |

## Air inlet line

| Model NA |  |
| --- | --- |
| DISASSEMBLY - Loosen the clamp C and remove the hose B . - Unscrew the screws D and remove the air filter A . |  |
| ASSEMBLY - Fix the hose B on the inlet manifold E using the clamp C . - Fix the air filter A using the screws D on the support F ( tightening torque 25 Nm ). |  |
| Model TC - TCA |  |
| DISASSEMBLY - Loosen the clamp C and remove the hose B . - Unscrew the screws D and remove the air filter A |  |
| ASSEMBLY - Fix the hose B on the turbocharger E using the clamp C . - Fix the air filter A using the screws D on the support F (tightening torque 25 Nm ). |  |

## Exhaust line

| NA Model |  |
| --- | --- |
| Muffler option A |  |
| DISASSEMBLING - Unscrew the nuts B . - Unscrew the screws A and remove the muffler C . |  |
| ASSEMBLING - Fix the muffler C on the manifold D using the nuts B (tightening torque 25 Nm ). - Fix the muffler C on the crankcase E using the screws A (tightening torque 25 Nm ). |  |
| Muffler option B |  |
| DISASSEMBLING - Unscrew the nuts A . - Unscrew the screws B and remove the muffler C . |  |
| ASSEMBLING NOTE: replace gaskets G1 , G2 if removed |  |
| - Position the muffler C on the bracket E and the tube C1 on the manifold D . - Fix the muffler C on the bracket E using the screws B (tightening torque 25 Nm ). - Fix the tube C1 on the manifold D using the nuts A (tightening torque 25 Nm ). |  |
| TC - TCA Model |  |
| Muffler option A |  |
| DISASSEMBLING - Unscrew the nuts B . - Unscrew the screws A and remove the muffler C . |  |
| ASSEMBLING - Fix the muffler C on the flange C1 using the nuts B (tightening torque 25 Nm ). - Fix the bracket C2 on the bell F using the screws A and washer A1, A2 (tightening torque 50 Nm ). |  |
| Turbocharger |  |
| DISASSEMBLING - Unscrew the screws D3 and remove the tube E2 . - Unscrew the screws D2 and remove the tube E1 . - Loosen the clamp F1 and disconnect the hose E3 . - Loosen the clamp F3 and disconnect the hose E4 . - Unscrew the nuts D4 and remove the turbocharger D . |  |
| ASSEMBLING - Place the turbocharger D on the interfacing falnge of exhaust manifold H . |  |
| - Fix the turbocharger D on the exhaust manifold H using the nuts D4 (tightening torque 25 Nm ). - Connect the hose E3 with the turbocharger D and fix it using the clamp F1 . - Place the tube E2 on the turbocharger D and the crankcase G with screws D3 and copper washers D5 . - Fix the tube E2 using the screws D3 (tightening torque 1 5 Nm ). - Place the tube E1 on the crankcase G and under the turbocharger D with screws D2 and gskets E5 , E6 . - Tighten the screws D2 (tightening torque 10 Nm ). |  |
| Only for TC model |  |
| DISASSEMBLING - Loosen the clamp F2 and remove the hose E3 . § |  |
| ASSEMBLING - Connect the hose E3 with the inlet manifold L and fix it using the clamp F2 . |  |

## Fuel System

| Fuel feed pump |  |
| --- | --- |
| Electric fuel feed pump § |  |
| DISASSEMBLING - Disconnect the junction A1 and remove the tube E1 with the pump F and the pre-filter G . |  |
| ASSEMBLING - Connect the junction A1 on the filter D . - Check the condition of tubes F and pre-filter G . |  |
| Mechanical fuel feed pump § |  |
| DISASSEMBLING - Unscrew the screw B7 . - Unscrew the nuts B8 and remove the pump P . |  |
| ASSEMBLING - Place the pump P with gasket P1 on the crankcase Q . - Fix the pump P on the crankcase Q using the nuts B8 (tightening torque 25 Nm ). - Fix the tube E3 on the pump P using the screw B7 with copper washers B9 (tightening torque 15 Nm ). |  |
| Fuel filter |  |
| DISASSEMBLING - Disconnect the connector A1 from the injection pump A . - Unscrew the screw B1 . - Unscrew the screws B2 and remove the fuel filter D with the tube C . |  |
| ASSEMBLING - Fix the fuel filter D using the screws B2 (tightening torque 25 Nm ). - Conect the connector A1 on the injection pump A . - Fix the tube C using the screw B1 (tightening torque 25 Nm ). |  |
| Rail § |  |
| DISASSEMBLING - Unscrew the nuts A2 , A3 and remove the tube E2 from the rail H and injection pump A . |  |
| - Remove the lockers H2 . - Unscrew the screws B2 and remove the spacers H1 and the rail H . |  |
| ASSEMBLING - Apply oil EMI-385 on injector gaskets. § - Place the rail H on the injectors L . |  |
| - Install the lockers H2 on the rail H . |  |
| - Apply Loctite 242 on the thread of screws B2 . - Place the spacers H1 under the rail H and fix the rail H using the screws B2 (tightening torque 25 Nm ). |  |
| - Place the tube E2 on the injection pump A and Rail H , screw the nuts A2 , A3 . - Tight the nut A2 (tightening torque 28 Nm ). - Tight the nut A3 (tightening torque 28 Nm ). |  |
| Electronic injectors |  |
| DISASSEMBLING - Install studs B3a on the head N . - Place the the sockets L1 of the tool ST_48 on the injectors L . - Fix the tool ST_48 on studs B3a using nuts B3b . |  |
| - Install the lockers L2 on the sockets L1 to lock the injectors L . |  |
| - Unscrew the nuts B4 to extract the injectors L from the cylinder head N . |  |
| ASSEMBLING - In case of injector reinstallation: before the reinstallation, replace all injector gaskets. § - Apply oil EMI-385 on injector gaskets. § - Place the injectors L inside the cylinder head N . - Place the sockets L1 of the tool ST_48 on the injectors L . - Place the washers B6 under the nuts B3 and fix the nuts B3 on the cylinder head N . |  |
| - Install the lockers L2 on the sockets L1 to lock the injectors L . - Screw the nuts B4 to install the injectors L inside the cylinder head N . |  |
| Injection pump |  |
| DISASSEMBLING - Unscrew the screws B5 and remove the injection pump A . |  |
| ASSEMBLING - Install the roller tappet A4 inside the crankcase M if not allready present. - Place the injection pump A on the crackcase M . - Fix the pump A using the screws B5 respecting the tightening cycle below listed: - cycle 1: 15 Nm - cycle 2: 25 Nm |  |

## Sensors and Switches

| T-MAP Sensor (only for model TC - TCA) |  |
| --- | --- |
| DISASSEMBLY - Unscrew the screws A1 and remove the sensor A . |  |
| ASSEMBLY - Place the sensor A on the intake manifold A3 . - Fix the sendor A using the screws A1 (tightening torque 10 Nm ). |  |
| RPM Sensor |  |
| DISASSEMBLY - Unscrew the screw B1 and remove the sensor B . |  |
| ASSEMBLY - Place the sensor B on the crackcase B3 with the shim B2 . - Fix the sensor A using the screw B 1 (tightening torque 10 Nm ). |  |
| Phase Sensor |  |
| DISASSEMBLY - Unscrew the screw C1 and remove the sensor C . |  |
| ASSEMBLY - Place the sensor C on the carter C4 with the shim C2 . - Apply Loctite 242 on the thread of screw C1 . - Fix the sensor C using the screw C1 (tightening torque 10 Nm ). |  |
| Coolant temperature Sensor |  |
| DISASSEMBLY - Unscrew and remove the sensor D . |  |
| ASSEMBLY - Fix the sensor D with the gasket D1 on the cylinder head D2 (tightening torque 12 Nm ). |  |
| Oil Pressure Switch |  |
| DISASSEMBLY - Unscrew and remove the switch E . |  |
| ASSEMBLY - Fix the switch E on the crankcase E1 (tightening torque 25 Nm ). |  |

## Electrical components

| Starter Motor |  |
| --- | --- |
| DISASSEMBLY - Unscrew the screws A and remove the motor B . |  |
| ASSEMBLY - Fix the motor B to the flanging plate/bell C using the screws A (tightening torque 25 Nm ). |  |
| Voltage alternator |  |
| Alternator 45A |  |
| DISASSEMBLY - Unscrew the screw A . |  |
| - Unscrew the nut B and remove the alternator C . |  |
| ASSEMBLY - Place the alternator C on the crankcase D , install the screw B4 inside the fixing hole and install the spacer B3 . - Screw the nut B with the washer B1 on the screw B4 . |  |
| - Fix the bracket C1 on the cylinder head E using the screw A (tightening torque 25 Nm ). - Fix the alternator C using the nut B and the screw B4 (tightening torque 25 Nm ). |  |
| Alternator 80A - 100A |  |
| DISASSEMBLY - Unscrew the screw A . |  |
| - Unscrew the nut B and remove the alternator C . |  |
| ASSEMBLY - Place the alternator C on the crankcase D , install the screw B4 inside the fixing hole and install the spacer B3 . - Screw the nut B with the washer B1 on the screw B4 . |  |
| - Fix the alternator C using the screw A , the nut A1 with the washer A2 on the bracket C1 (tightening torque 25 Nm ). - Fix the alternator C using the nut B and the screw B4 (tightening torque 25 Nm ). |  |
| Glow plugs |  |
| DISASSEMBLY - Unscrew the nuts A and remove the cables B1 , B2 . |  |
| - Unscrew and remove the glow plugs C . |  |
| ASSEMBLY - Install and fix the glow plugs C on cylinder head D (tightening torque 15 Nm ). |  |
| - Fix the cables B1 , B2 on the glow plugs C using the nuts A (tightening torque 2.5 Nm ). |  |

## Intake manifold

| DISASSEMBLY NOTE: for NA version, disconnect the oil return hose E . - Loose the clamp D and disconnect the hose E from the manifold B . |  |
| --- | --- |
| - Unscrew the screws A and remove the manifold B . |  |
| ASSEMBLY - Place the manifold B on the cylinder head C using the tool ST_18 . - Fix the manifold B and the gasket B1 on the cylinder head C using the screws A (tightening torque 25 Nm ). |  |

## Exhaust manifold

| DISASSEMBLY - Unscrew the nuts A and remove the manifold B . |  |
| --- | --- |
| ASSEMBLY - Place the gaskets B1 and the manifold B on the cylinder head C using the studs B2 . - Fix the manifold B on the cylinder head C using the nuts A (tightening torque 25 Nm ). |  |

## PTOs

| 1st PTO |  |
| --- | --- |
| Flywheel |  |
| DISASSEMBLY - Install the tool ST_16 on pulley C . - Install the component T3 on the crankcase D using the screws T2 entering the component T4 inside the groove of the component T3 . - Fix the screws T5 to lock the pulley C to the tool ST_16 . NOTE: in this way the crankshaft is locked. |  |
| - Unscrew the screw A and remove the flywheel B . |  |
| ASSEMBLY - Place the flywheel B on the crankshaft C using the ST_46 . - Fix the flywheel B on the crankshaft C using the screws A (tightening torque 140 Nm ). |  |
| Flanging plate |  |
| DISASSEMBLY - Unscrew the screws A and remove the plate B . |  |
| ASSEMBLY - Place the plate B on the crankcase C . NOTE: use the proper reference pins. - Fix the plate B on the crankcase C using the screws A (tightening torque 45 Nm ). |  |
| Flanging bell (TBD) |  |
| DISASSEMBLY - Unscrew the screws A and remove the bell B . |  |
| ASSEMBLY - Place the bell B on the crankcase C . NOTE: use the proper reference pins. - Fix the bell B on the crankcase C using the screws A (tightening torque 45 Nm ). |  |
| 2nd PTO |  |
| DISASSEMBLY - Unscrew the screw A and remove the pulley B with the tool ST_16 . - Remove the component T3 from the crankcase D . |  |
| ASSEMBLY - Install the pulley B on the crankshaft C using the screw A . NOTE: do not fix the screw A . - Install the tool ST_16 on pulley B . - Install the component T3 on the crankcase D using the screws T2 entering the component T4 inside the groove of the component T3 . - Fix the screws T5 to lock the pulley C to the tool ST_16 . - Fix the screws A (tightening torque 360 Nm ). |  |
| 3rd PTO |  |
| DISASSEMBLY - Unscrew the nuts A and remove the 3rd PTO B . |  |
| ASSEMBLY - Place the 3rd PTO B on the crankcase C with the gasket B1 . - Fix the 3rd PTO B using the nuts A (tightening torque 25 Nm ). |  |

## Cylinder head

| DISASSEMBLY - Unscrew the screws A1 and remove the cover A . |  |
| --- | --- |
| - Unscrew the screws B1 and remove the rocker arms B . |  |
| - Remove the rods C . |  |
| - Unscrew the screws D1 and remove the cylinder head D . |  |
| CHECKING AND OVERHAUL - The MAX value of allowable irregularity of surface D3 is 0.1 mm . - If the value is not observed, you are required to grind surface D3 . - The MAX removal allowed is 0.2 mm . NOTE: Grinding task must be done with pre-chambers F assembled. |  |
| - Measure indentation X1 of each valve with regard to the cylinder head surface D3 , which is to be a MIN of 0.595 mm and MAX of 0.615 mm . - The X1 MAX indentation allowed on worn components is 0 .815 mm . - If the measured value does not correspond with the values indicated, replace the worn component. NOTE: The seats must be machined after their installation to reach value X1 , go to a rectification workshop for such operations. S1 > Exhaust seat S2 > Intake seat |  |
| - Measure the diameters Y1 , Y2 of the rods and guides valve ( Tab. x.x ). - If the diameters don't correspond to the values indicated, replace the valves or guides. - The MAX increasing of clrearance for Y1 , Y2 worn components is 0.1 mm . - Observe the dimension Y3 from surface D5 when assembling guides D4 ( Tab. x.x ) . NOTE: Carry out the measurements in different points to detect any ovalisation and/or concentrated wear. Tab. x.x details the dimensional values of new components only. The guides must be machined for value Y1 after their installation . Contact a machining workshop for such operations. x.x Ref.; MIN; MAX. Y1; 6.03; 6.042. Y2; 5.978; 5.99. Y3; 12.5; 13. |  |
| ASSEMBLY - Measure the piston protrusion P from head level E2 in 4 diametrically opposed points P1 . - Repeat the operation for all pistons P and take note of the highest average value, determining value P1 ( Tab. x.x ). X.X P1 (mm); THICKNESS REFERENCE (N). 0.121 - 0.230. 0.231 - 0.330. 0.331 - 0.436. - Based on the value detected, select the proper gasket D2 as shown in the Tab. x.x . |  |
| - Check the tappets E1 are correctly installed inside the crackcase E . - Place the gasket D2 on the crankcase E using the reference pins E2 . - Place the head D on the gasket D2 using the reference pins E2 . NOTE: Gasket D2 must be replaced at each time the head D is assembled. |  |
| - Apply MOLY GREASE on the screw D1 (20 mm from the head bottom ) - Tighten the screws D1 respecting the tightening order as per illustration indication at each cycle below listed: - cycle 1: 10 Nm - cycle 2: 35 Nm - cycle 3: 90° - cycle 4: 90° - cycle 5: 90° - cycle 6: 90° |  |
| - Install the rods C inside the proper niches inside the cylinder head D . |  |
| - Place the spacers B2 correctly on the valves B3 . - Place the rocker arms B on the head D using the proper reference pins. - Check all spacers B2 are correctly installed on the valves B3 . - Apply Loctite 242 on the thread of screw B1 . § - Fix the rocker arms B using the screws B1 (tightening torque 25 Nm ). |  |
| Check the gasket A2 is correctly installed insde its seat inside the cover A . Place the cover A on the head D using the tool ST_17 . + apply Loctite 242 on the thread of screws A1 . Place the screws A1 inside the fixing holes. |  |
| Tighten the screws A 1 respecting the tightening order as per illustration indication ( tightening torque 10 Nm ). |  |

## Oil circuit

| Oil Cooler |  |
| --- | --- |
| DISASSEMBLY - Unscrew and remove the filter A . |  |
| - Unscrew the joint C and remove the oil cooler D . |  |
| - Unscrew th screw E and remove the support F . |  |
| ASSEMBLY - Install the gasket E1 in the proper seat on the screw E . - Install the gasket F1 on the support F . - Fix the support F on the crankcase G using the screw E with the gasket E2 (tightening torque 25 Nm ). |  |
| - Check the oil cooler gasket is correctly installed. - Fix the oil cooler D on the support F using the joint C (tightening torque 30 Nm ). |  |
| - Check the gasket D1 is correctly installed inside the proper seat of the oil filter D . - Tifgten the filter D on the support F (tightening torque 20 Nm ). |  |
| Oil Pressure valve |  |
| DISASSEMBLY - Unscrew the plug A and remove the gasket A1 , the spring A2 , the piston A3 from the carter B . |  |
| CHECKING AND OVERHAUL - Measure the free length X of spring A2 , which must be 47.91 mm . - If the measured value does not match, replace spring A2 . |  |
| ASSEMBLY - Install the piston A3 , the spring A2 inside the carter B . - Tighten the plug A with the cooper washer A1 on the carter B (tightening torque 17.5 Nm ). |  |
| Oil sump |  |
| DISASSEMBLY - Unscrew the screws A1 and remove the oil sump A . NOTE: Use a metal sheet/spatula between the crankcase and oil sump to detach them. |  |
| - Unscrew the screw B1 . - Unscrew the screw B2 and remove the tube B . - Unscrew the screws C1 and remove the plate C . |  |
| ASSEMBLY - Install the gaskets B3 inside the proper seat of the tube B . - Place the tube B installing it inside the oil inlet hole of the crankcase D . - Apply Loctite 242 on the thread of screw B 1, B2 . - Fix the tube B using the screws B2 (tightening torque 25 Nm ). - Fix the tube B using the screws B1 (tightening torque 10 Nm ). |  |
| - Apply Loctite 242 on the thread of screw C1 . - Fix the plate C using the screws C1 (tightening torque 25 Nm ). |  |
| - Apply about 2.5 mm cord of sealant S ( Loctite 5699 ) on the oil sump A as per illustration. |  |
| - Place the oil sump A on the crankcase sulbasamento D using the tool ST_18 . - Apply Loctite 242 on the thread of screw A1 . - Fix the oil sump A on the crankcase D using the screws A1 (tightening torque 25 Nm ). |  |
| Timing gear cover (oil pump) |  |
| DISASSEMBLY - Unscrew the screws A1 and remove the carter A . |  |
| ASSEMBLY - Apply about 2.5 mm cord of sealant S ( Loctite 5699 ) on the carter A as per illustration. |  |
| - Place the gasket A3 in the proper seat of the carter A . - Install the gasket A2 on the carter A using the tool ST_58 . - Place the carter A on the crankcase B using the tool ST_18 . |  |
| - Apply Loctite 242 on the thread of screw A1 . - Fix the carter A on the crankcase B using the screws A1 respecting the fixing order as per illustration indication (tightening torque 25 Nm ). |  |

## Crankcase

| Timing gear (camshafts) |  |
| --- | --- |
| DISASSEMBLY - Remove the tappets D1 from the crankcase D . |  |
| - Unscrews the screws A1 and remove the gear A . |  |
| - Unscrews the screws B1 and remove the shaft B . - Unscrews the screws C1 and remove the shaft C . |  |
| CHECKING AND OVERHAUL - Measure the diameters of housings Z6 , Z7 . - Measure the diameters of pins K6 , K7 . - Calculate the clearance between the housings and pins, which is to observe the values in Tab. 5. 5 . 5. 5 Ref.; Ø MIN; Ø MAX; Ref.; Ø MIN; Ø MAX; CLEARANCE. Z6; 42; 42.025; K6; 41.94; 41.96; 0.04 - 0.085. Z7; 30; 30.021; K7; 29.94; 29.96; 0.04 - 0.081. NOTE: The MAX value of wear allowed for housing Z6 , Z7 is 0.02 mm according to Ø MAX . The MAX value of wear allowed for pins K6 , K7 is 0.005 mm according to Ø MIN . |  |
| - Measure the diameters of housings Z1 , Z2 , Z3 , , Z4 , Z5 . - Measure the diameters of pins K1 , K2 , K3 , K4 , K5 . - Calculate the clearance between the housings and pins, which is to observe the values in Tab. 5.6 . 5. 6 Ref.; Ø MIN; Ø MAX; Ref.; Ø MIN; Ø MAX; CLEARANCE. Z1; 46; 46.025; K1; 45.92; 45.94; 0.06 - 0.105. Z2; 45; 45.025; K2; 44.92; 44.94; 0.06 - 0.105. Z3; 44.5; 44.525; K3; 44.42; 44.44; 0.06 - 0.105. Z4; 44; 44.025; K4; 43.92; 43.94; 0.06 - 0.105. Z5; 38; 38.025; K5; 37.92; 37.94; 0.06 - 0.105. NOTE: The MAX value of wear allowed for housing Z1 , Z2 , Z3 , , Z4 , Z5 is 0.02 mm according to Ø MAX . The MAX value of wear allowed for pins K1 , K2 , K3 , K4 , K5 is 0.005 mm according to Ø MIN . |  |
| - Measure the diameters J2 , J3 of gear A2 and support A . - Calculate the clearance between J2 , J3 which is to observe the values in Tab. X.X . NOTE: The MAX value of wear allowed is 0.xx mm . - Measure the thickness J5 of washer A3 ( MIN X.XX mm ). |  |
| ASSEMBLY - Install the shaft C inside the crankcase D . - Apply Loctite 242 on the thread of screw C1 . - Fix the shaft C using the screws C1 (tightening torque 10 Nm ). |  |
| - Install the shaft B inside the crankcase D . - Apply Loctite 242 on the thread of screw B1 . - Fix the shaft B using the screws B1 (tightening torque 10 Nm ). |  |
| - Install the gear A2 on the support A . - Install the washer A3 on the support A . |  |
| - Place the gear A2 on the crankcase D aligning the references as per illustration. - Apply Loctite 243 on the thread of screw A1 . § - Fix the support A on the crankcase D using the screws A1 (tightening torque see service letter 500001 ). |  |
| Crankshaft seal support - flywheel side |  |
| DISASSEMBLY - Unscrew the screws A1 and remove the flange A . |  |
| ASSEMBLY - Install the gasket A2 inside the flange A using the tool ST_42 . - Place the flange A on the crankcase B . |  |
| - Apply Loctite 242 on the thread of screw A1 . - Fix the flange A using the screws A1 respecting the fixing order as per illustration (tightening torque 10 Nm ). |  |
| Conrod and piston |  |
| DISASSEMBLY - Unscrew the screws B4 and remove the conrod caps B1a , B2a , B3a . |  |
| - Extract the conrod and piston P from the crankcase C . |  |
| - Replace the bearings D on B1a , B2a , B3a, B1b , B2b , B3b . NOTE: the bearing position D on B1a , B2a , B3a, B1b , B2b , B3b must be centered as per illustration. - Install B1a , B2a , B3a su B1b , B2b , B3b using the screws B4 . NOTE: do not tighten the screws B4 . |  |
| - Check the conrod and piston groups P , weight difference must not exceed 10 gr . - Unscrew the screws B4 and remove B1a , B2a , B3a da B1b , B2b , B3b . |  |
| CHECKING AND OVERHAUL - Measure the axial shift of crankshaft B . - Axial shift must be a MIN 0.xx mm and MAX 0.xx mm . - If the values measured do not correspond, replace thrust washers C4 . |  |
| - Measure the dimension H1 , H2 . - Calculate the clearance between H1 , H2 which is to observe the values in Tab. 5.8 . 5. 8 Ref.; Ø MIN; Ø MAX; CLEARANCE. H1; 24.015; 24.025; 0.015 - 0.03. H2; 23.995; 24. NOTE: The MAX clearance increasing allowed is 0.01 mm . - Measure the dimension H3 , H7 . - Calculate the clearance between H3 , H7 which is to observe the values in Tab. 5.9 . 5. 9 Ref.; Ø MIN; Ø MAX; CLEARANCE. H3; 46.021; 46.056; 0.021 - 0.072. H7; 45.984; 46. NOTE: The MAX clearance increasing allowed is 0.01 mm . - Measure the dimension H4 , H5 . 5.10 Ref.; MIN; MAX. H4; 23.077; 23.8. H5; 131.48; 131.52. |  |
| - Insert the gudgeon pin G into bearing G1 . - Check the axis parallelism of the connecting rod big end and small end. - Parallel deviation (value H6 ) measured at the tip of the gudgeonpin, must be a MIN 0,xx and MAX 0,xx mm . - If the parallelism values do not comply with the specified ones, replace the connecting rod. |  |
| ASSEMBLY - Rotate the crankshaft D moving the crankpin D1 at BDC . |  |
| - The combustion chamber H of the piston must be oriented towards the inlet manifold. |  |
| - Lubricate the piston mantle, segments, conrod bearings and cylinder surface C with engine oil 15W-40 . - Install the conrod and piston groups P about 10 mm of the piston skirt. |  |
| - Align the gudgeon pin axle with engine center axle. - Rotate the piston P about 15° anticlockwise. |  |
| - This operation avoid the conrod impact against the oil sprayer E during the conrod and piston installation inside the cylinder C . - Gently push the piston in the cylinder C paying attention to the sprayer E . |  |
| - Lubricate the crankpins with engine oil 15W-40 . - Apply BRB100 MOLY GREASE under the screw head P1c . - Place the conrod P1a on crankpin D1 . - Install the cap P1b using the screws P1c . NOTE: the caps P1b have the mating plan fractured, each cap can be mate with its origin conrod P1a , pay attention to the right mating. - Tighten the screws P1c respecting the tightening cycle below listed: - cycle 1: 5 Nm - cycle 2: 20 Nm - cycle 3: 33° - cycle 4: 33° |  |
| Piston |  |
| DISASSEMBLY - Remove the lock ring R and extract the gudgeon pin G from the piston P . |  |
| - Remove the rings S1 , S2 , S3 from the piston P . |  |
| CHECKING AND OVERHAUL - Measure the piston diameter on point X ( 15 mm from piston P base ) . - If diemeter is lower than 80.922 mm , replace the short block or long block. |  |
| - Check the clearance between piston rings and their proper seats in piston P (dimension Y1, Y2, Y3 ). - If dimensions Y1, Y2, Y3 detected does not match with values in Tab. 5.11 , replace short block o long block. |  |
| - Insert piston rings S1, S2, S3 inside the cylinder C . - Check the rings gap distance (dimension RG ). - If dimension RG detected does not match with values in Tab. 5.12 , replace the piston rings S1, S2, S3 . 5.12 Ref.; MIN; MAX; MAX WEAR. S1; 0.2; 0.35; 0.45. S2; 0.4; 0.6; 0.7. S3; 0.25; 0.45; 0.55. NOTE: Piston rings S1, S2, S3 cannot be replaced separately . |  |
| ASSEMBLY - Install the rings S1 , S2 , S3 on the piston P . NOTE: position the piston rings gap at 120° from each other. |  |
| - Apply MOLYSLIP on gudgeon pin G . - Mate the conrod P1a with the piston P and install the gudgeon pin G . - Install the lock rings R to lock the gudgeon pin G . |  |
| Crankshaft |  |
| DISASSEMBLY - Take note of caps A1 , A2 , A3 marking their position on the crankcase. - Unscrew the screws A and remove the caps A1 , A2 , A3 , A4 . - Remove the crankshaft B . |  |
| - Unscrew the screws C1 and remove the sprayers C2 . |  |
| - Remove the bearings C3 , C4 . |  |
| - Remove the bearings C5 , C4 from the caps A1 , A2 , A3 , A4 . |  |
| CHECKING AND OVERHAUL - Measure the cylinder diameter on point H8 , H9 , H10 . - According to Tab. 5.13 , if the wear is higher than 81.032 mm , replace the short block or long block. NOTE: Cylinder grinding is forbidden. 5.13 Ø MIN; Ø MAX. 81.013; 81.027. |  |
| - Measure the diameters H11 , H12 . - Calculate the clearance between H11 , H12 which is to observe the values in Tab. 5.14 . NOTE: The MAX value of wear allowed is 0.xx mm . |  |
| - The MAX value of allowable irregularity of surface T is 0.2 mm . - If the value is not observed, replace the short block or long block. |  |
| - Check the oil ducts are free of dust and debris. |  |
| ASSEMBLY - Fix the sprayers C2 on the crankcase C using the screws C1 (tightening torque 8 Nm ). |  |
| - Install the bearings C3 on crankcase C seats respecting the notches C3a and make sure the oil delivery holes C3b is matching with the crankcase ones. |  |
| - Install the bearings C4 on the crankcase C seats. |  |
| - Apply GX100 on half bearing surface C3 . - Install the crankshaft B on the bearings C3 . |  |
| - Install the bearings C5 on caps A1 , A2 , A3 seats respecting the notches C5a . - Apply GX100 on half bearing surface C5 |  |
| - Install the caps A1 , A2 , A3 on the crankcase C respecting their original position using the screws A . - Apply MOLYSLIP under the screw head A - Tighten the screws A respecting the tightening cycle below listed: - cycle 1: 15 Nm - cycle 2: 45° - cycle 3: 45° |  |
| - Install the bearing C5 on cap A4 seat respecting the notch C5a . |  |
| - Install the bearings C4 on the cap A4 seat. |  |
| - Install the gaskets A4a inside the proper seat on cap A4 and keep in position using the tool ST_49 . - Apply GX100 on half bearing surface C5 . - Install the cap A4 inside the crankcase C paying attention to not cut the gasket A4a during installation. - Apply MOLYSLIP under the screw head A - Tighten the screws A respecting the tightening cycle below listed: - cycle 1: 15 Nm - cycle 2: 45° - cycle 3: 45° |  |

# Storage information

## Product preservation

Important: - If the engines are not to be used for 6 months, they must be protected by carrying out the operations described in Engine storage (up to 6 months) ( Par. 6.2 ). - If the engine is still not in use after the first 6 months, it is necessary to carry out a further operation to extend the protection period (more than 6 months) ( Par. 6.3 ). - If the engine is not to be used for an extended period, the protective treatment procedure must be repeated within 24 months of the previous one.

## Engine storage (up to 6 months)

Before storing the engine check that:

- The environments are not humid or exposed to bad weather. Cover the engine with a proper protective sheet against dampness and atmospheric contaminants.

- The place is not near electric panel.

- Avoid storing the engine in direct contact with the ground.

## Engine storage over 6 months

Follow the steps described in Par. 6.2 .

- Engine oil replacement ( Par. 4.2 ).

- Refuel with fuel additives for long storage.

- With expansion tank:
make sure that the coolant is up to the maximum level.

- Without expansion tank: Top liquid up until the pipes inside the radiator are covered by about 5 mm.

Do not overfill the radiator, but leave room for the fuel to expand.

- Start the engine and keep it idle at minimum speed for 2 minutes.

- Bring the engine to 3/4 of the maximum speed for 5÷10 minutes.

- Turn off the engine.

- Completely empty the fuel tank.

- Spray SAE 10W-40 on the exhaust and intake manifolds.

- Seal the exhaust and intake ducts to prevent foreign bodies from entering.

- When cleaning the engine, if using a pressure washer or steam cleaning device, avoid directing the nozzle on electrical components, cable connections and sealed rings (oil seals etc).
If cleaning engine with a pressure washer or steam cleaner, it is important to maintain a minimum distance of at least 200mm between the surface to be washed and the nozzle - avoiding absolutely electrical components such as alternators, starter motors and engine control units (ECU).

- Treat non-painted parts with protective products.

If the engine protection is performed according to the suggestions indicated no corrosion damage should occur.

## Engine starting after storage

- Remove the protective sheet.

- Use a cloth soaked in degreasing product to remove the protective treatment from the external parts.

- Inject lubricating oil (no more than 2 cm 3 ) into the intake ducts.

- Refill the tank with fresh fuel.

- Make sure that the oil and the coolant are up to the MAX  level.

- Start the engine and keep it idle at MIN speed for a two about minutes.

- Bring the engine to 75% of MAX rated speed for 5 to 10 minutes.

- Stop the engine while the oil is still hot, discharge the protective oil in a suitable container.

Warning

  - Over time, lubricants and filters lose their properties, so it is important consider whether they need replacing, also based on the criteria described in Par.  4.2 .

- Replace the filters (air, oil, fuel) with original spare parts.

- Pour new oil (Par.  4.2 ) up to the MAX  level .

- Empty the cooling circuit completely and pour in the new coolant up to the MAX  level ( Par.  4.2 ).

## Unused machine

| If the machine is not used for a certain amount of time, follow the operations below: 4.17.1 Operations for the engine |  |
| --- | --- |
| POINT; OPERATION. 1; Unused machine up to 2 months; - The place must be dry and fresh throughout the period in which the machine is not used. - Consult the machine’s manual to disconnect the battery (before disconnecting the battery, wait for minimum 5 mins after turning off the engine). - Make sure the engine is not exposed to direct sunlight. - Make sure the engine is not near any heat sources.. Starting; - Before starting the engine, check Par. 4.1 - 4.2 for maintenance operations. - Consult the machine’s manual to connect the battery and start the engine.. 2; Unused machine from 2 to 9 months; - Perform the operations related to unused machine described in point 1. - Perform the operations described in Par. 4.1 . - Start the engine at least every 4 months as per operations described in point 1: - Avoid sudden accelerations for the first few minutes. - Bring the engine to the working temperature by pressing the accelerator 3/4 from MAX. - Leave the engine running at minimum speed for a few minutes and turning off the engine.. - Starting; - Before starting the engine, check Par. 4.1 - 4.2 for maintenance operations. - Consult the machine’s manual to connect the battery and start the engine. - Avoid sudden accelerations for the first few minutes.. 3; Unused machine over 9 months; - Perform the operations related to unused machine described in point 1 and 2.. Starting; - Before starting the engine, check Par. 4.1 - 4.2 for maintenance operations. - Check the quality of coolant from the relative testing strips. - Consult the machine’s manual to connect the battery and start the engine. - Avoid sudden accelerations for the first few minutes.. |

# Information regarding discharge of liquids

# Information for replacing the functional units

# Information for disassembly

# Information about overhauling

# Assembly information

# Fluids filling information

# Information about optional components

# Information on adjustments

# Tools information

# Information about failures

## Useful information about failures

- This chapter contains information about the problems that may appear during engine operation with its causes and trouble shooting Tab.  7 . 2 .

- In some cases, you shall turn off the engine immediately to avoid further damage Tab.  7 . 1 .

7 . 1

| THE ENGINE MUST BE IMMEDIATELY TURNED OFF WHEN |  |
| --- | --- |
| 1 | Warning RED light turn on |
| 2 | The oil pressure indicator light turns on while running |
| 3 | The engine rpms suddenly increase and decrease |
| 4 | A sudden and/or unusual noise is heard |
| 5 | Colour the exhaust fumes suddenly darkens |

7 . 2

| TROUBLES | POSSIBLE CAUSE | SOLUTION |
| --- | --- | --- |
| Warning YELLOW light turn on | Engine ECU has detect a malfunctions | Contact KOHLER authorised workshops |
| The engine does not start | Sulphated battery terminals corroded | Clean the battery terminals |
|  | Battery voltage too low | Recharge the battery or replace it |
|  | Low fuel level | Refuel |
|  | Frozen fuel | Contact KOHLER authorised workshops |
|  | Clogged fuel filter | Replace with a new filter |
|  | Air suction in fuel system | Contact KOHLER authorised workshops |
|  | Clogged air filter | Replace with a new filter |
|  | Clogged pipes | Contact KOHLER authorised workshops |
|  | Intake or exhaust system clogged | Replace with a new fuse; if the problem persists, Contact KOHLER authorised workshops |
|  | Intake or exhaust system clogged | Contact KOHLER authorised workshops |
| Engine starts but stops | Inefficient electrical connections | Clean the electrical contacts; if the problem persists, Contact KOHLER authorised workshops |
|  | Sulphated battery terminals | Clean the battery terminals |
|  | Clogged fuel filter | Replace with a new filter and clean the tank |
|  | Clogged fuel pipes | Contact KOHLER authorised workshops |
| Engine doesn't rev up | Safety protocol in starting | Wait some seconds |
|  | Throttle to MAX in starting | Release the throttle and wait some seconds |
| RPM instability at idle speed | Clogged fuel pipes | Contact KOHLER authorised workshops |
| Low RPM at idle speed | Clogged fuel pipes | Contact KOHLER authorised workshops |
|  | Poor quality fuel | Clean the tank and refuel with quality fuel |
| Blue smoke | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops |
|  | Clogged air filter | Replace with a new filter |
| Excessive fuel consumption | Clogged air filter | Replace with a new filter |
|  | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops |
| Engine lost its initial performance | Clogged air filter | Replace with a new filter |
|  | Clogged fuel pipes | Contact KOHLER authorised workshops |
|  | Cheap fuel | Clean the tank and refuel with quality fuel |
|  | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops |
| Slow acceleration | Clogged fuel filter | Replace the fuel filter |
| Engine jerking | Clogged fuel pipes | Contact KOHLER authorised workshops |
| Engine overheats | Insufficient coolant level | Fill up to the level |
|  | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops |
|  | Clogged radiator | Clean the radiator; if the problem persists, contact KOHLER authorised workshops |

In the event that the solutions proposed in Tab.  7 . 2  do not eliminate the trouble, contact a KOHLER authorized workshop

# Glossary

## Glossary

A

| Air gap: | Distance to respect between a fixed component and one in movement. |
| --- | --- |
| Alternator: | A component that transforms mechanical energy into AC electrical energy. |
| Authorised service station: | KOHLER authorised workshop. |
| Authorised workshop: | KOHLER authorised service centre. |

B

| Base configuration: | Engine having components represented in Par. 1.3 - 1.4 . . |
| --- | --- |
| BDC: | Bottom Dead Centre; a moment in which the piston is at the start of its stroke. |
| Bore | Internal diameter of the cylinder in combustion engines. |

C

| Cold Start Advance: | The device provides for advance injection modification to enable advance of the engine at low temperatures. |
| --- | --- |
| Combustion: | Chemical reaction of a mixture composed of fuel and fuel (air) inside a combustion chamber. |
| Crankshaft: | A component that transforms straight operation into rotary operation, and vice-versa. |

E

| EC: | "European Community". |
| --- | --- |

F

| Fig.: | Figure. |
| --- | --- |
| Functional units: | Component, or group of main components, able to carry out specific functions on the engine. |

G

| Galvanised: | Material that has undergone surface protection treatment. |
| --- | --- |
| Grinding (valves and seats): | Cleaning operation of the valves and seats carried out with an abrasive paste (refer to an authorised service station for this type of operation). |

H

| Heater: | A device that heats the intake air by means of an electrical resistor. |
| --- | --- |
| Heavy conditions: | Type of extreme condition referred to the work environment in which the engine is used (very dusty - dirty area, or in a contaminated environment due to various types of gas). |

I

| Idle speed operation: | Operation of a running engine with the vehicle stopped and on idle speed. |
| --- | --- |

K

| KDI: | "Kohler Direct Injection" |
| --- | --- |

M

| Maintenance - periodic | A group of maintenance actions that have the sole objective to control and replace elements on their expiry, without modifying or improving the functions carried out by the system, neither increasing the value nor improving performance. |
| --- | --- |
| MAX: | Maximum. |
| Methyl ester: | It is a mixture of products by means of a chemical conversion of oils and animal and/or vegetable fat, which is used to produce Biofuel. |
| Min.: | Minutes. |
| MIN: | Minimum. |
| Model: | Model, engine identification plate, which indicates the engine's model. |

N

| N/C: | Normally Closed, referred to switches (oil-pressure switch). |
| --- | --- |
| N/O: | Normally Opened, referred to switches (Coolant temperature sensor) |

O

| Oil Cooler: | Small radiator used to cool the oil. |
| --- | --- |

P

| Par.: | Paragraph. |
| --- | --- |
| Paraffin.: | Fatty and solid substance that may form inside the diesel. |
| Pipe cleaner: | An instrument having a metal cylindrical body with bristles that jut outwards. It is similar to a brush and is used to clean areas that are not easily accessible manually (e.g. oil ducts inside an engine). |
| Power operation: | Operation of the engine at high speeds. |
| PTO: | Power Take Off - a point provided to take advantage of alternative operation transmission. |

R

| Ref.: | Reference. |
| --- | --- |
| Rpm: | Rounds per minute. |

S

| s/n: | Serial number (engine identification name plate) indicating the engine identification series/chassis number. |
| --- | --- |
| Spec.: | Specification, (engine identification name plate) indicating the engine version. |
| STD: | (Standard), base configuration of a component, or a group of components. |

T

| Tab.: | Table. |
| --- | --- |
| TDC: | Top Dead Centre; a moment in which the piston is at the end of its stroke. |
| Thermostatic valve: | A valve that adjusts the flow of coolant liquid; it is able to operate by means of temperature variation. |
| Torque: | Force applied to an object that rotates on an idler shaft. |
| Trochoid: | Rounded toothed profile (also known as "lobes"). |
| Turbocharger: | Device that compresses air intake by sending it to the intake manifold by means of a turbine. |

U

| Used oil: | Oil altered by operation or time, which is no longer compliant for correct lubrication of the components. |
| --- | --- |

W

| Warning Lamp: | A warning light (usually red) that indicates a serious anomaly during engine operation. |
| --- | --- |

| SYMBOLS AND UNITS OF MEASUREMENT |  |  |  |
| --- | --- | --- | --- |
| SYMBOL | UNIT OF MEASUREMENT | DESCRIPTION | EXAMPLE |
| α | degree | Rotation/inclination angle | 1° |
| cm 2 | square centimetre | Area | 1 cm 2 |
| Ø | millimetre | Circumference | Ø 1 mm |
| Nm | newton-metre | Torque | 1 Nm |
| mm | millimetre | Length | 1 mm |
| µm | 1/1000 of a millimetre (micron) |  | 1 µm |
| H | hour | Time | 1 h |
| g/kW | grammes per kilowatt per hour | Specific consumption | 1 g/kWh |
| kg/h | kilogramme per hour | Max. flow rate | 1 kg/h |
| Lt./min. | litres per minute | Flow rate | 1 Lt./min. |
| Lt./h | litres per hour |  | 1 Lt./h |
| ppm | parts per million | Percentage | 1 ppm |
| N | newton | Force | 1 N |
| A | Ampere | Intensity of electrical current | 1 A |
| gr. | gramme | Weight | 1 gr. |
| kg | kilogramme |  | 1 kg |
| W | Watt | Power | 1 W. |
| kW | kiloWatt |  | 1 kW |
| pa | pascal | Pressure | 1 pa |
| KPa | Kilopascal |  | 1 KPa |
| bar | barometric pressure |  | 1 bar |
| mbar (1/1000 bar) | barometric pressure |  | 1 mbar |
| R | Resistance | Resistance to electrical current (referred to a component) | 1 Ω |
| Ω | ohm | Resistance of electrical current | 1 Ω |
| Rpm | revs per minute | Rotation of an axis | 1 Rpm |
| Ra | average roughness expressed in microns | Roughness | 1 Ra |
| °C | degree centigrade | Temperature | 1°C |
| V | Volt | Electrical voltage | 1 V |
|  | millimetre | Hex-head capscrew | 1 mm |
| cm 3 | cubic centimetre | Volume | 1 cm 3 |
| Lt. | litre |  | 1 Lt. |