Storage information

KDI 1903TCR / KDI 1903TCRE5 Workshop manual (Rev. 17.8)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Storage information

## Product preservation

Important

- If the engines are not to be used for 6 months, they must be protected by carrying out the operations described in Engine storage (up to 6 months) (Par. 4.2) .

- If the engine is still not in use after the first 6 months, it is necessary to carry out a further procedure to extend the protection period (more than 6 months) (Par. 4.3) .

- If the engine is not to be used for an extended period, the protective treatment procedure must be repeated within 24 months of the previous one.

## Engine storage (up to 6 months)

Before storing the engine check that:

- The environments are not humid or exposed to bad weather. Cover the engine with a proper protective sheet against dampness and atmospheric contaminants.

- The place is not near electric panel.

- Avoid storing the engine in direct contact with the ground.

## Engine storage (over 6 months)

Follow the steps described in Par. 4.2.

- Pour protective oil in the carter up to the MAX level.

- Refuel with fuel additives for long storage.

- With expansion tank:
make sure that the coolant is up to the maximum level.

- Without expansion tank: Top liquid up until the pipes inside the radiator are covered by about 5 mm.

Do not overfill the radiator, but leave room for the fuel to expand.

- Start the engine and run it at idle speed for around 2 minutes.

- Bring the engine to 75% of maximum rated speed for 5 to 10 minutes.

- Turn off the engine.

- Empty out completely the fuel tank.

- Spray SAE 10W-40 on the exhaust and intake manifolds.

- Seal the exhaust and intake ducts to prevent foreign bodies from entering.

- When cleaning the engine, if using a pressure washer or steam cleaning device, avoid directing the nozzle on electrical components, cable connections and sealed rings (oil seals etc).
If cleaning engine with a pressure washer or steam cleaner, it is important to maintain a minimum distance of at least 200mm between the surface to be washed and the nozzle - avoiding absolutely electrical components such as alternators, starter motors and engine control units (ECU).

- Treat non-painted parts with protective products.

- Loosen the alternator belt  Par. 7.3 point 1 and 2

NOTA: For Poly-V belt Par. 11.3 from points 1 to 3 .

If the engine protection is performed according to the suggestions indicated no corrosion damage will be found.

## Engine starting after storage

- Remove the protective sheet.

- Use a cloth soaked in degreasing product to remove the protective treatment from the external parts.

- Inject lubricating oil (no more than 2 cm3) into the intake ducts.

- Adjust the alternator belt tension ( Par. 9.15.2 from points 7 to 10 ) - for a Poly-V belt ( Par. 11.3 from point 5 to 8 )or replace if there are signs of deterioration.

- Refill the tank with fresh fuel.

Warning

- Over time, lubricants and filters lose their properties, so itis important to consider whether they need replacing, also based on the criteria described in Tab. 2.9 .

- Make sure that the oil and the coolant are up to the maximum level.

- Start the engine and run it at idle speed for around 2 minutes.

- Bring the engine to 75% of maximum rated speed for 5 to 10 minutes.

- Stop the engine and while the oil still hot, perform the operation in Par. 5.2 .

- Replace the filters (air, oil, fuel) with original spare parts.

- Perform the operations described in Par. 10.1 .

- Perform the operations described in  Par. 5.1 and Par. 10.2 .