Information about optional components

KDI-TCP 3404E5 Workshop Manual (Rev. 01.1)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about optional components

## Heater (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.1.1 Disassembly - Undo the screws A . - Remove the flange C . - Remove the heater E and the relevant gaskets F . | Fig 11.1 |
| 11.1.2 Assembly Important - Always replace gaskets F , with each assembly. - In sequence, fit the manifold G with the gasket F , the new heater E , the second gasket F , the flange C , the washers H , the screws A and the cable B . - Secure the flange C with the screws A (tightening torque at 22 Nm ). | Fig 11.2 |

## Idler gear (for 3rd / 4th PTO)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.2.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Loosen capscrews A and remove plate B1 . - Extract gear C . - Remove gudgeon D together with plate B2 . | Fig 11.3 |
|  | Fig 11.4 |
| 11.2.2 Assembly Important - Make sure that gudgeon D has no impurities inside. - On capscrew A , assemble: - plate B1 - gudgeon D - gear C - plate B2 . | Fig 11.5 |

| - Position gear unit C1 onto crankcase E , complying with reference J1 , J2 with gear F . - Secure unit C1 by means of capscrew A (tightening torque 25 Nm ). NOTE: the reference J1 can have 2 different configurations for the gear C , Fig. 11.6b shows the correct position of the reference J1 for both configurations. | Fig 11.6 Fig 11.6b |
| --- | --- |

## 3rd PTO (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.3.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Extract gear A . | Fig 11.7 |
| 11.3.2 Assembly - Insert gear A into the seat of crankcase B , fitting the shaft of pump C in gear A . | Fig 11.8 |

## 4th PTO (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.4.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Extract gear A . | Fig 11.9 |
| 11.4.2 Assembly - Insert gear A into the seat of crankcase B , fitting the shaft of pump C in gear A . | Fig 11.10 |

## Balancer device (replacement)

| Importante - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.5.1 Disassembly - Perform the operations described in Par. 11.2.1 . - Extract shaft A1 e A2. | Fig 11.11 |
| 11.5.2 Assembly - Lubricate gudgeon C of shaft A1 and A2 with oil. - Insert shaft A1 into seat B1 of the crankcase, complying with reference D of gear E . - Insert shaft A2 into seat B2 of the crankcase. - Perform the operations described in Par. 11.2.2 . NOTE: the shafts A1 and A2 are different and it is important not to invert the assembly position, the reference D is specific for the shaft A1 and is timed with the gear E , the reference J2 is specific for the shaft A2 and is timed with the idler gear F (see the Fig. 11.12b ). Fig 11.12 Fig 11.12b |  |

## ETB (replacement)

| - Unscrew screws A and remove the ETB valve B with its gasket C . Important - Always replace the gasket C at each assembly. | Fig. 11.13 |
| --- | --- |
| - Fix the ETB valve B and its gasket C by means of screws A (tightening torque 10 Nm ) | Fig. 11.14 |

## Air filter (cartridge replacement)

| - Release the two hooks F and remove the cover A from the body D . - Remove the cartridges B and G . Important - Before proceeding with operation, read Par. 3.3.2 . - The safety cartridge G (if present) must always be replaced if it is dirty or damaged. - Insert the new cartridge G inside the new cartridge B and both of them inside the filter body D . - Secure the cover A via the hooks F . | Fig 11.59 |
| --- | --- |

## Cooling circuit (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . - Before carrying out the operation, refer to the machine manual to disconnect the fuel waste line from the fuel radiator C2 . |  |
| --- | --- |
| 11.8.1 Smontaggio vaschetta di espansione * 1 - Release the clamp A1, A2 . 2 - Disconnect hose B1, B2, B3 . 3 - Loosen capscrews C1 . 4 - Loosen capscrews C3 e rimuovere la vaschetta C4 . * |  |
| 11.8.2 Radiator d isassembly 1 - Release the clamp A3, A5 . 2 - Disconnect hose B4 from radiator C . |  |
| 4 - Release the clamp A2, A4, A5 . 5 - Disconnect hose B3, B5 from radiator C . |  |
| 6 - Loosen all capscrews E1, E2, E3. 7 - Release nut F . 8 - Remove floodgates G1 and G2 . 9 - Loosen capscrews K . 10 - Disconnect radiator C from hoses H1 and H2 , being careful not to deform tubes J1 and J2 . |  |
| 11.8.3 Fuel radiator replacement 1 - Loosen the screws E4 , extract the radiator C2 from the conveyor C1 . 2 - Insert the radiator C2 in the conveyor C1 . 3 - Fasten the radiator C2 with the screws E4 and washers E5 (tightening torque of 10 Nm ). |  |
| 11.8.4 Fan disassembly 1 - Undo the screws P and remove the fan R . |  |
| 11.8.5 Fan assembly 1 - Assemble the fan R on the pulley U . 2 - Fasten the fan R by using the screws P (tightening torque at 10 Nm ). |  |

| 11.8.6 Radiator assembly 1 - Insert the vibration damper V2 correctly on bracket S . 2 - Fit radiator C onto hose H1 , H2 being careful not to deform tube J1 , J2 . 3 - Centre radiator C onto vibration-dampening devices V . 4 - Secure radiator C onto vibration-dampening devices V by means of capscrews K (tightening torque at 25 Nm ). 5 - Fix the vibration damper V2 on the bracket S through the nut F (tightening torque of 25 Nm ). 6 - Secure hose H2 by means of clamp A5 . |  |
| --- | --- |
| 7 - Position floodgate G1 onto radiator C . 8 - Secure all capscrews E1 , E3 , E2 . |  |

| 9 - Secure hoses B3, B4, B5 by means of clamps A2 , A3, A4 . |  |
| --- | --- |
| 11.8.7 Montaggio vaschetta di espansione * 1 - Fissare la vaschetta C4 sul radiatore C tramite le viti C3 (coppia di serraggio a 25 Nm ). * 2 - Fissare i manicotti B3, B1, B2 tramite le fascette A1 , A2 . * |  |