Information about optional components

KDI 3404 TM Workshop Manual (Rev. 08.2)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about optional components

## Heater (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.1.1 Disassembly - Undo the screws A . - Remove the flange C . - Remove the heater E and the relevant gaskets F . | Fig 11.1 |
| 11.1.2 Assembly Important - Always replace gaskets F , with each assembly. - In sequence, fit the manifold G with the gasket F , the new heater E , the second gasket F , the flange C , the washers H , the screws A and the cable B . - Secure the flange C with the screws A (tightening torque at 22 Nm ). | Fig 11.2 |

## Air filter (cartridge replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Release the two hooks A and remove the cover B from the body C . - Remove the cartridges D . | Fig 11.3 |
| - Insert the new cartridge D and both of them inside the filter body C . - Secure the cover B via the hooks A . | Fig. 11.4 |

## Cooling circuit (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.3.1 Radiator disassembly - Release the clamp A1, A2 . - Disconnect hose B from radiator C . | Fig. 11.5 |
| - Release the clamp A3, A4 . - Disconnect hose D from radiator C . | Fig. 11.6 |
| - Loosen all capscrews E1 , E2 and E3 . - Release nut F . - Remove floodgates G1 and G2 . - Loosen capscrews K . - Disconnect radiator C from hoses H1 and H2 , being careful not to deform tubes J1 and J2 . Fig. 11.7 | Fig. 11.8 |
|  | Fig. 11.9 |
| 11.3.2 Fan disassembly - Undo the screws P and remove the fan R . | Fig. 11.10 |
| 11.3.3 Fan assembly - Assemble the fan R on the pulley U . - Fasten the fan R by using the screws P (tightening torque at 10 Nm ). | Fig. 11.11 |

| 11.3.4 Radiator assembly - Fit radiator C onto hose H2 , being careful not to deform tube J2 . - Centre radiator C onto vibration-dampening devices V . - Secure radiator C onto vibration-dampening devices V by means of capscrews K (tightening torque at 25 Nm ). | Fig. 11.12 |
| --- | --- |
| - Position floodgate G1 onto radiator C . - Secure all capscrews E1 . - Place floodgate G2 onto radiator C . - Secure all capscrews E3 and E2 . | Fig. 11.13 |

| - Fit hose H1 onto radiator C , being careful not to deform tube J1 . NOTE : Make sure vibration-dampening device V2 is correctly installed in its place on brace S . | Fig. 11.14 |
| --- | --- |
| - Secure vibration-dampening device V2 onto brace S by means of nut F , inserting washer F1 (tightening torque at 25 Nm). - Secure hoses B and D by means of clamps A2 and A3 ( Fig. 11.5 - 11.6 ). - Secure hoses H1 and H2 by means of clamps A1 and A4 ( Fig. 11.5 - 11.6 ). | Fig. 11.15 |