Information for disassembly

KDI 3404 TCR-SCR Workshop Manual (Rev. 10.3)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information for disassembly

## Recommendations for disassembly

Important

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- The operator should prepare all equipment and tools in order to enable him to carry out the operations correctly and safely.

- Before disassembly, perform the operation described in Chap. 5 .

- Before proceeding with operation, carefully read Chap. 3 .

- In order to operate safely and easily, we recommend positioning the engine on a rotating stand for engine overhauling.

- Seal all injection component unions as illustrated in P ar. 2.9.8 during assembly.

- Protect all disassembled components and coupling surfaces subject to oxidation with lubricant.

- Where necessary, reference to special tools to use during disassembly operations is indicated (es. ST_05 ), identified in Tab. 13.1 - 13.2 - 13.3 .

## EGR circuit disassembly

| 7.2.1 EGR cooler unit - Undo the screws A ( ST_05 ) and B and remove the pipe C with the relative gaskets. | Fig 7.1 |
| --- | --- |
| - Undo the screws D, E ( ST_05 ) and remove pipe F and the relevant gaskets. | Fig 7.2 |
| - Release the clamp M and remove hose N1 . - Release the clamp L , and remove hose N2 . - Undo capscrews G ( ST_05 ) and remove EGR Cooler H. | Fig 7.3 |

## Coolant recirculation components disassembly

| 7.3.1 Oil Cooler manifold - Release the clamps A . - Undo the screw B and remove hoses C ( ST_05 ) . | Fig 7.4 |
| --- | --- |
| - Release the clamp D and remove hoses E . | Fig 7.5 |
| 7.3.2 Coolant pump Important - The pump B is not repairable. - Perform the operations described in Par. 6.5.1 . - Undo capscrews F and remove flange G with the relative gasket. | Fig 7.6 |
| 7.3.3 Thermostatic valve - Undo the screws A and remove the thermostatic valve cover B . - Remove the thermostatic valve C and its gasket. Important - Always replace the gasket D every time it is disassembled. - Check that the air bleeding hole is not clogged or blocked ( Par. 2.11.4 ). | Fig 7.7 |

## Electric components disassembly

| 7.4.1 Electric wiring Important - Refer to Par. 2.13 prior to proceeding with disassembly. - Disconnect the connector A . - Undo the screw B . - Release the clamp C . | Fig 7.8 |
| --- | --- |
| - Disconnect the connector D . | Fig 7.9 |
| - Disconnect the connectors E and F . | Fig 7.10 |
| - Disconnect the connector G . | Fig 7.11 |
| - Remove nuts H and disconnect cables L and M . - Release the clamp P . - Undo the screw Q . | Fig 7.12 |
| - Disconnect the connectors R , S , T , U . - Undo the screws V ( ST_06 ) and remove the wiring support Z . | Fig 7.13 |
| 7.4.2 Starter motor Important - The motor is not repairable. - Perform the operations from point 2 to 3 of Par. 6.6.1 . 7.4.3 Alternator - Undo the screws A1 and B1 and remove the alternator C1 . | Fig 7.14 |
| 7.4.4 EGR Valve - Perform the operations of Par. 6.4.1 . - Undo capscrews D1 and remove flange E1 with the relative gasket. NOTE: The EGR valve is not a serviceable item, and if damaged/worn, it should be replaced with a new one. | Fig 7.15 |
| 7.4.5 Sensors and switches Important - After disassembly, protect the sensors suitably against knocks, dampness and any high temperature sources. - The sensors and switches cannot be repaired, therefore they must be replaced in the event of anomalies. 7.4.5.1 Oil pressure switch ( ) - Unscrew and remove the oil pressure switch AD . | Fig 7.16 |
| 7.4.5.2 Coolant temperature sensor ( ) - Unscrew and remove the coolant temperature sensor AE . | Fig 7.17 |
| 7.4.5.3 Speed sensor ( ) - Undo the screw H1 and remove the sensor L1 with the relative spacer ( ST_06 ) . - Undo the screws M1 and remove the sensor N1 . | Fig 7.18 |
| 7.4.5.4 Camshaft phase sensor - Undo the screw P1 and remove the sensor Q1 with the relative spacer. | Fig 7.19 |
| 7.4.5.5 T-MAP Sensor ( ) - Undo the screw R1 and remove the sensor S1 ( ST_06 ). | Fig 7.20 |
| 7.4.5.6 Fuel filter water detection sensor ( ) Warning - The fuel filter is not always mounted on the engine. - When disassembling the sensor U1 , use a suitable container to recover the fuel contained in the cartridge T1 . - Unscrew the sensor U1 from the cartridge T1 . | Fig 7.21 |

## Turbocharger disassembly

| - Unscrew the fittings A and remove the pipe B with the relative gaskets C . | Fig 7.22 |
| --- | --- |
| - Undo the screws D and remove pipe E and the relevant gaskets. | Fig 7.23 |
| - Undo the nuts F and remove the turbocharger G . | Fig 7.24 |

## Exhaust manifold disassembly

| - Remove nuts A , capscrews B and spacers C , manifold D and gaskets E . - Close the openings and manifolds to prevent foreign bodies from entering. | Fig 7.25 |
| --- | --- |

## Crankshaft and target wheel pulley disassembly

| - Perform the operations from point 4 to 5 of Par. 6.6.1 . |  |
| --- | --- |

## Flange unit disassembly

| 7.8.1 Flywheel - Perform the operations of point 2 Par. 6.2.2 . - Loosen, but not remove, nut A . Important - Leave the special tool ST_34 for blocking the flywheel ( Par. 7.7 ). Danger - The flywheel E is very heavy. Pay the utmost attention while removing it in order to prevent it dropping or falling, as this may have serious consequences for the operative. - Undo the screws B and remove the flywheel C by means of tool ST_43 . | Fig 7.26 |
| --- | --- |
| 7.8.2 Flange housing Danger - The flange housing F is very heavy. Pay the utmost attention while removing it in order to prevent it dropping or falling, as this may have serious consequences for the operative - Secure tool ST_41 onto gear D by means of capscrews B . - Undo capscrews E by following the order indicated in the figure. - Remove the engine housing F by means of tool ST_44 . | Fig 7.27 Fig 7.28 |

## Lubrication circuit disassembly

| 7.9.1 Oil pump Important - The oil pump is not repairable. - Undo the screws A and remove the pump unit B . | Fig 7.29 |
| --- | --- |
| 7.9.2 Oil pressure valve ( ) - Remove cotter pin C . - Remove disk D , spring E , piston valve F using a magnet. | Fig 7.30 |
| 7.9.3 Oil Cooler unit and oil filter - Perform the operations of Par. 6.8.1 . NOTE : To replace the oil cartridge, refer to operations of Par. 6.8.2 . |  |
| 7.9.4 Oil vapour separator unit - Perform the operations of Par. 6.7.1 . - Undo the screws G and remove the support H . - Loosen clamp B and remove hose M . - Undo the screw N and remove hoses P . - Remove quick fitting R and remove hose S . | Fig 7.31 |

## Fuel system disassembly

| Important - Seal all injection component unions as illustrated in Par. 2.9.8 . 7.10.1 Fuel return pipes - Release the clip A . - Disconnect unions B from electronic injectors C . Warning - After removing the union, the clip A must automatically return to its initial position; otherwise it must be replaced. - Undo the screw D. - Disconnect the pipe E . - Undo and remove the screw F with the relative gaskets and put the cap on the Common Rail pressure relief valve G . - Remove the fuel return pipes. | Fig 7.32 Fig 7.33 |
| --- | --- |
| 7.10.2 Fuel flow pipes - Remove the pipes H, L. | Fig 7.34 |
| 7.10.3 High pressure fuel pipes Danger - The fuel injection circuit undergoes high pressure, use safety protections as described in Par 3.4.3 . - Ensure that the Common Rail is not under pressure by slowly and carefully unscrewing one of the nuts N . - Undo the screw M . - Loosen the nuts N, P in sequence. - Fully undo the nuts N, P in sequence and remove the high pressure pipes Q, R. | Fig 7.35 |
| 7.10.4 Common Rail - Undo the screws S and remove the Common Rail T . NOTE: Take care to protect te sensor U from knocks, moisture and any high temperature source. The internal parts of the rail cannot be repaired. | Fig 7.36 |
| 7.10.5 Electronic injectors Important - In the event that the electronic injectors are disassembled (not necessarily replaced), mark them with the relevant cylinder number from which they originate so as not to confuse them during re-assembly (Fig. 7.38) . - The electronic injectors cannot be repaired. - If one or more electronic injectors are to be replaced, the new calibration data must be inserted in the ECU via a specific instrument ( ST_01 ). - Be careful not to damage the gaskets V . - Undo capscrews J and remove them together with the relative washers K and then brace W . - Pull out the electronic injector C . NOTE: Should you be unable to remove the electronic injector (acting only on point X ), use an open-ended spanner (Ø 34 mm), by applying small rotations to unblock the component. - Seal all injection component unions as illustrated in Par. 2.9.8 . - Ensure that gasket Y has remained in the correct position (Fig. 7.38) . Otherwise, recover the gasket from inside the electronic injector Z . | Fig 7.437 Fig 7.38 |
| 7.10.6 Fuel filter ( ) - Undo the fuel cartridge A1 from support B1 . - Undo the screws C1 and remove the filter support B1 . | Fig 7.39 |
| 7.10.7 High-pressure fuel injection pump Important - Before disassembling, carefully read Par. 2.17 . - The injection pump is not repairable - Should the fuel feeding pump need to be replaced, after assembly, it is necessary to perform the Pump Learning procedure by means of instrument ST_01 . - Undo the nut D1 . - Screw the tool ST_13 on the thread of the gear E1 . - Redo the capscrew F1 of tool ST_13 to disconnect the pump H1 from gear E1 . - Undo the screws G1 , remove pump H1 and the relative gasket L1 . - Seal all injection component unions as illustrated in Par. 2.9.8 . | Fig 7.40 Fig 7.41 |

## Intake manifold disassembly

| - Undo the screws A and remove the semi-manifold B ( ST_05 ) . - Remove the separation plate C and the gaskets D . | Fig 7.42 |
| --- | --- |
| - Undo the screws E . - Remove the semi-manifold F and the gasket G . | Fig 7.43 |

## Cylinder head unit disassembly

| 7.12.1 Rocker arms cover - Undo the screws A . - Remove the rocker arms cover B . - Remove the gasket C . | Fig 7.44 |
| --- | --- |
| 7.12.2 Rocker arm pin - Undo the screws D . - Remove the rocker arm pin unit E . | Fig 7.45 |
| 7.12.2.1 Rocker arm ( ) - Remove the retainer ring F . - Remove the shoulder rings G . - Remove the rocker arms H . | Fig 7.46 |
| 7.12.3 Valve rods and bridges - Remove the valve control U-bolts M . - Remove the rocker arm control rods N . | Fig 7.47 |
| 7.12.4 Cylinder head Important - The capscrews P must be replaced every time they are disassembled. - Do NOT remove the capscrews completely, first loosen them by turning them a whole cycle following the order shown in the figure. - Loosen fastening screws P , turning them by one turn following the order shown in the figure. - Undo capscrews P by following the order indicated in the figure. Important - To lift cylinder head Q , only use both eyebolts Y provided by KOHLER (refer to Fig. 7.55 ). - When removing the cylinder head Q and subsequent disassembly, control, and assembly operations, it is necessary to protect the contact surface W of cylinder head Q and crankcase J against impacts. - Remove the cylinder head Q . - Remove the head gasket R . | Fig 7.48 Fig 7.49 |
| 7.12.4.1 Valves ( ) - Mount the tool ST_07 on the head Q fixing it on one of the holes for fixing the rocker arm cover. NOTE: Change the fixing hole according to the position of the valves to be removed. - Position the tool ST_07 on the valve as shown in the figure. | Fig 7.50 |
| - Push the lever of the tool ST_07 downwards, in order to lower the valve plates S in the direction of the arrow T , remove cotters U using a magnet. NOTE: Repeat all the operations for all the valves concerned. | Fig 7.51 |
| Important - Before removing the valves, make some marks to record their original position, in order to avoid confusing them when reassembling (if they are not replaced). - Remove the valves V . | Fig 7.52 |
| 7.12.4.2 Electronic injector sleeves ( ) - Unscrew and remove the sleeves Z from the head Q . - Remove the gaskets J, K . | Fig 7.53 |
| 7.12.4.3 Valve stem gasket ( ) - Remove the gaskets W . | Fig 7.54 |
| 7.12.4.4 Lifting eyebolts ( ) - Undo the screws X and remove the eyebolts Y . - Thoroughly wash the cylinder head Q . | Fig 7.55 |

## Timing system gear disassembly

| Important - For the following operation, turn the engine by bringing the cylinder head surface downwards. - Remove the gear A . - Unscrew screw B and remove the gear C . NOTE: Gear A is assembled on the camshaft by press-fit; remove gear A to also remove the camshaft. | Fig 7.56 |
| --- | --- |

## Oil sump unit disassembly

| 7.14.1 Oil sump - Undo the screws A . - Remove the oil sump B by inserting a plate between surface C of crankcase D and oil sump B . - Remove the oil dipstick E . | Fig 7.57 |
| --- | --- |
| 7.14.2 Oil suction pipe - Undo the screws F and remove the oil pipe G . | Fig 7.58 |
| 7.14.3 Oil drain pipe ( ) - Undo the screws H and remove the pipe L . | Fig 7.59 |

## Engine block disassembly

| 7.15.1 Piston unit/connecting rod Important - Mark some numerical references (cylinder n°) on the connecting rods, connecting rod caps N , pistons and gudgeon pins to prevent unintentionally confusing the components not replaced during assembly. Failure to do this may result in engine malfunctions. - References on connecting rod M and cap N must only be carried out on a side in correspondence with K1 and K2 , as illustrated in Fig. 7.61 . - Screw the bolt M temporarily. | Fig 7.60 |
| --- | --- |
| NOTE : coupling cap N on the connecting rod can be carried out with centring taper pins (Fig. 7.62) or broken ( Fig. 7.63 - without centring taper pins). Fig. 7.62 Fig. 7.63 | Fig 7.61 |
| - Pull out the connecting rod - piston assembly from position 2 and 3 by manually applying pressure on the connecting rod big end L in the direction of arrow X . - Couple the connecting rod big end caps N with the relevant piston and connecting rod unit L . - Rotate the crankshaft by 180°. - Repeat points 2 to 5 to disassemble the connecting rod-piston assembly to position 1 and 4 . | Fig 7.64 |
| Warning - The connecting rod half-bearings P are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.65 |
| 7.15.2 Lower semi-crankcase Important - The capscrews Q must be replaced every time they are disassembled. - Do NOT remove the capscrews completely, first loosen them by turning them a whole cycle following the order shown in the figure. - Loosen fastening screws Q , turning them by one turn following the order shown in the figure. - Undo capscrews Q by following the order indicated in the figure. | Fig 7.66 |
| Important - The capscrews R must be replaced every time they are disassembled. - Do NOT remove the capscrews completely, first loosen them by turning them a whole cycle following the order shown in the figure. - Loosen fastening screws R , turning them by one turn following the order shown in the figure. - Undo capscrews R by following the order indicated in the figure. - Remove the lower semi-crankcase D1 and store it in a suitable container for washing. | Fig 7.67 |
| 7.15.3 Crankshaft Remove: - Crankshaft S . - The shoulder semi-rings T . - gasket U from crankshaft S . | Fig 7.68 |
| 7.15.4 Piston ( ) - Remove the retainer ring V . - Remove the pin Z to separate the piston J from the connecting rod L . Important - If they are not replaced, keep the components together (connecting rod - piston - gudgeon pin) by using references in order to prevent them from getting mixed up during assembly. | Fig 7.69 |
| 7.15.4.1 Rings ( ) - Remove the rings K . | Fig 7.70 |
| 7.15.5 Oil spray nozzles ( ) - Undo the screws W and remove the spray nozzles X from the upper semi-crankcase D2 . | Fig 7.71 |
| 7.15.6 Camshaft tappets - With a magnet, remove the tappets Y from the upper semi-crankcase D2 . | Fig 7.72 |
| 7.15.7 Crankshaft bushings - Remove the crankshaft bushings A1 from the upper crankcase D2 . Important - The crankshaft half-bearings A1, B1 are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.73 |
| - Remove the crankshaft bushings B1 from the lower semicrankcase D2 . | Fig 7.74 |