Informacje o remoncie

KDI 2504 M Workshop manual (Rev. 09.6)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
| Technical Documentation | ED005302-961 | 9.6 | 04/2013 | 03/2023 | M. SILVESTRONE | V. MANINI |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about overhauling

## Recommendations for overhauls and tuning

- The information is laid out in sequence, according to operational requirements, and the intervention methods have been selected, tested and approved by the manufacturer's
technicians.

- This chapter describes procedures for checking, overhauling and tuning units and/or individual components.

NOT E : To easily locate specific topics, the reader should refer to the analytical index or chapter index.

- Before any intervention, the operator should lay out all equipment and tools in such a way as to enable him to carry out operations correctly and safely.

- The operator must comply with the specific measures described in order to avoid errors that might cause damage to the engine.

- Before carrying out any operation, clean the units and/or components thoroughly and eliminate any deposits.

- Do not wash the components with steam or hot water. Use suitable products only.

- Do not use flammable products (petrol, diesel, etc.) to degrease or wash components. Use suitable products only.

- Apply a layer of lubricant over all surfaces of all disassembled components to protect them against oxidation.

- Check the integrity and state of wear of all disassembled components in order to ensure good working condition of the engine.

- When indicated, some components are to be replaced in pairs or together with other parts (e.g. crankshaft half-bearings/connecting rod, piston complete with rings and gudgeon pin, etc.).

- When indicated, some grinding operations are to be carried out in series (e.g. grinding of cylinders, crankpins, journals, etc.).

## Crankcase

8.2.1 Oil line check Use a pipe cleaner in access points A, B, C, D, E to clean the oil ducts of crankcase G . Use compressed air to eliminate any residues. Replace and assemble the conical cap in hole B ( B1 if present - tightening torque at 30 Nm ) and caps in holes D , after having performed cleaning operations. Fig 8.1 8.2.2 Cylinder check Position crankcase G onto a workbench. With a dial gauge, measure the diameter in correspondence to points J-M-N (Fig. 8.2) lengthwise and diagonally with regard to axis H of the crankshaft. If ovalisation or wear detected in a single point in J-M-N is greater than +0.05 mm with regard to the value in Tab. 8.1a , you are required to perform grinding operations on all cylinders F. Refer to Tab. 8.1a to establish the clearance value of cylinders subjected to grinding operations. Important - The grinding involved is of +0.20 mm. - Cylinder grinding operations must observe KOHLER SPECIFICATIONS - cod. ED0035612500. - Grinding must be strictly performed on all cylinders F . - Tab. 8.1a details the dimensional values of new components only. - (1) The increase of +0.20mm , may already be present on the engine. - Se la maggiorazione di +0.20 mm è già presente, non è possibile eseguire ulteriori rettifiche. * Tab 8.1a Grinding values PISTON; Ø CILINDER (± 0.007 mm); Ø PISTON (± 0.007 mm); CLEARANCE VALUE (mm). STD; 88.010; 87.950; 0.046 - 0.074. +0.20 (1); 88.210; 88.150. Fig 8.2 N.B.: if removing the Z2 hole closing cap, the new cap must comply with the MAX measurement of 1.5 mm from the surface G1. Fig 8.2a

| 8.2.3 Block Surface Flatness Use a dial gauge to check if the cylinder head surface A1 is level. The MAX value of allowable irregularity of surface A1 is: - 0.10 mm on the entire area; - 0.03 mm on an area of 100x100 mm. Grinding of surface A1 is not permitted | Fig 8.2b |
| --- | --- |
| 8.2.4 4-cylinder camshaft housing check The camshaft housings only contain the timing system side bushing Q . Use an internal dial gauge to measure the diameters of housings X - W - K - Y - Z . With a micrometer, measure the diameters of gudgeon pins X1 - W1 - K1 - Y1 - Z1 (Fig. 8.4) . According to the values measured, calculate the clearance between the housing and gudgeon, which is to observe the values in Tab. 8.2a . The MAX value of wear allowed is 0.120 mm Important - Tab. 8.2a details the dimensional values of new components only. | Tab 8.2a Housing and camshaft gudgeon dimensions. REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). X; 44.000 - 44.025; 0.040 - 0.085. X1; 43.940 - 43.960. W; 43.000 - 43.025; 0.060 - 0.105. W1; 42.920 - 42.940. K; 42.000 - 42.025; 0.060 - 0.105. K1; 41.920 - 41.940. Y; 41.000 - 41.025; 0.060 - 0.105. Y1; 40.920 - 40.940. Z; 36.000 - 36.025; 0.060 - 0.105. Z1; 35.920 - 35.940. |
| Fig 8.3 |  |

| 8.2.5 Camshaft control for 4 cylinder engine With a micrometer, measure the maximum dimensions of intake camshaft R and exhaust camshaft S (Tab. 8.2b) . The MAX value of wear allowed is 0.1 mm . Important - Tab. 8.2b details the dimensional values of new components only. | Tab 8.2b Camshaft dimensions. REF.; DIMENSIONS (mm). R; 32.638 - 32.700. S; 32.998 - 32.060. |
| --- | --- |
| Fig 8.4 |  |

| 8.2.6 3 -cylinder camshaft housing check The camshaft housings only contain the timing system side bushing Q . Use an internal dial gauge to measure the diameters of housings X - W - K - Z . Use an internal dial gauge to measure the diameters of housings X1 - W1 - K1 - Z1 (Fig. 8.5) . According to the values measured, calculate the clearance between the housing and gudgeon, which is to observe the values in Tab. 8.2a . The MAX value of wear allowed is 0.120 mm Important - Tab. 8.3a details the dimensional values of new components only. | Tab 8.3a Housing and camshaft gudgeon dimensions. REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). X; 44.000 - 44.025; 0.040 - 0.085. X1; 43.940 - 43.960. W; 43.000 - 43.025; 0.060 - 0.105. W1; 42.920 - 42.940. K; 42.000 - 42.025; 0.060 - 0.105. K1; 41.920 - 41.940. Z; 36.000 - 36.025; 0.060 - 0.105. Z1; 35.920 - 35.940. |
| --- | --- |
| Fig 8.5 |  |

| 8.2.7 Camshaft control for 3 cylinder engine With a micrometer, measure the maximum dimensions of intake camshaft R and exhaust camshaft S (Tab. 8.3b) . The MAX value of wear allowed is 0.1 mm . Important - Tab. 8.3b details the dimensional values of new components only. | Tab 8.3b Camshaft dimensions REF.; DIMENSIONS (mm). R; 32.834 - 32.896. S; 33.335 - 33.397. |
| --- | --- |
| Fig 8.6 |  |

## Tappets and tappet housings

| 8.3.1 Tappets check Use a surface plate and a dial gauge as shown in Fig. 8.7 . Check the perpendicularity of the plate C , making the tappet D rotate in the direction of the arrow. The MAX value of wear allowed is 0.02 mm . With a gauge, check the length of value A and B (Tab. 8.4) . The MAX value of wear allowed is 0.08 mm . | Fig 8.7 |
| --- | --- |
| 8.3.2 Tappet housing check Use an internal dial gauge to measure the diameter of the tappet housings X . Use value of A detected (Par. 8.3.1) to calculate the clearance value ( Tab. 8.4 ). If the clearance values are not observed, replace the worn component. Important Tab. 8.4 T appets and t appet housing size. REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). A; 11.966 - 11.984; 0.060 - 0.105. X; 12.000 - 12.018. B; 46.5 ± 0.2; ---. | Fig 8.8 |

## Crankshaft

8.4.1 Dimensional check and overhauling

Wash the crankshaft thoroughly using suitable detergent.

Insert the pipe cleaner into all lubrication ducts B and blow compressed air to free them completely from any dirt residues.
Check the state of wear and integrity of journals C and connecting rod D .

Perform the operations described in Par. 9.3.1 , perform the operations described in Par. 9.3.6 - except Points 2, 4, 9 and 10 .
Measure the crank pins A1 with a micrometer, and using a dial gauge measure the internal diameter of the connecting rod half-bearings A2 .
Measure the main journals B1 , with a micrometer, and using a dial gauge measure the internal diameter of the crankshaft half-bearings B2 .
If the values described in Tab. 8.5 do not correspond, proceed with grinding all gudgeon pins A1 and B1 .

Gear A on the crankshaft is timed by a key, assembly of gear A on the shaft occurs after heating at a stabilized temperature of +180° C for 5 mins.

Fig 8.9

| Important - The crankshaft and connecting rod must be replaced every time they are assembled to prevent seizure, as they are made of special lead-free material. - The MAX allowed value of wear for A1 and A2 is 0.120 mm. - The MAX allowed value of wear for B1 and B2 is 0.120 mm. - To grind the crankshaft, a decrease in diameter of the halfbearings and connecting rod is provided for at 0.25 mm and 0.50 mm, to grind gudgeon pins A1 and B1 , measure the values of diameters A2 and B2 by assembling the decreased half-bearings, define the diameter to grind of pins A1 and B1 , observing the clearance indicated in Tab. 8.5. - La Tab. 8.5 riporta i valori dimensionali solo per i componenti nuovi. | Tab 8.5 Connecting rod and journal diameter REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). A; 53.981 - 54.000; 0.035 - 0.085. A1; 54.035 - 54.066. B; 63.981 - 64.000; 0.035 - 0.102. B1; 64.035 - 64.083. |
| --- | --- |
| 8.4.2 Checking the axial clearance of the crankshaft Perform the operations described in Par. 9.3.5 and 9.3.6 . Using a dial gauge, measure the axial shift of crankshaft E . Axial shift must be a MIN of 0.18 mm and MAX 0.38 mm.. If the values measured do not correspond, replace shoulder rings D . | Fig 8.10 |

## Connecting rod - piston assembly

| Important - In case of replacement, the connecting rods and pistons must always be replaced for all cylinders. |  |
| --- | --- |
| 8.5.1 Connecting rod dimensions check Important - Before assembling the connecting rod and pistons ( Par. 9.3.7 e 9.3.8 ), check that the difference in weight between the complete connecting rod and piston units do not exceed 8 gr to prevent weight imbalances during rotation of the crankshaft and consequent damage. - Mark some references on the connecting rods, caps Q , pistons and gudgeon pins to prevent unintentionally confusing the components during assembly. Failure to do this may result in engine malfunctions. - Connecting rod half-bearings S must be there with each assembly. Check that the contact surfaces are perfectly clean and intact. Assemble the connecting rod cap Q to the connecting rod with the half-bearings S and tighten capscrews P (tightening torque at 25 Nm ). With a dial gauge, measure diameters B and D . The MAX allowed value of wear for B and D is 0.06 mm. Tab 8.6 REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). A; 169.980 - 170.020. B; 30.020 - 30.030; 0.025 - 0.030. C; 29.995 - 30.000. D; 54.035 - 54.066. E; 67.700 - 68.000. F; 29.750 - 29.790. Important - Tab. 8.6 details the dimensional values of new components only. - Check that the connecting rod and crankshaft half-bearings are coupled properly. - Refer to the warnings in Par. 8.4.1 for value D decreased. - If the clearance value between B and C is not observed, you are required to replace bearing R (Fig. 8.12) . Measure value A, C, D, E and F and confront them with those described in Tab.8.6 . If the measured values do not follow those described in Tab.8.6 , replace connecting rod T . | Fig 8.11 Fig 8.12 Fig 8.13 |
| 8.5.2 Checking the gudgeon pin-pin axes are parallel Lubricate gudgeon pin A and bearing R (Fig. 8.12) . Insert the gudgeon pin into bearing R . Use a dial gauge to check the axis parallelism of the connecting rod big end and small end. Parallel deviation (value V ) measured at the tip of the gudgeonpin, must be a MIN of 0,015 and MAX of 0,030 mm. If the parallelism values do not comply with the specified ones,replace the connecting rod with a new one. 8.5.3 Piston rings check Insert ring U into the cylinder, measure value H (distance between the points of ring U ). Repeat for all the seal rings. If the measured value H does not correspond to the values indicated in the table (Tab. 8.7) , replace the seal rings U . Important - Seal rings cannot be replaced separately. NOTE: refer to Fig. 8.19 to locate the rings. Tab. 8.7 RINGS; H (mm). U1; 0.100 - 0.300. U2; 0.250 - 0.500. U3; 0.250 - 0.400. | Fig 8.14 Fig 8.15 |
| 8.5.4 Piston dimension check Clean the piston thoroughly. Measure the diameter of the piston at 12 mm (quota L ) from the base of the skirt in correspondence with the graphite lubrication windows M . Refer to Tab. 8.1b to establish the clearance value of the pistons with a decreased diameter. In correspondence with point W , there are: 3 digits for the STD piston; 3 digits followed by R for a piston with an increased diameter of 0.10 mm; +0.5 for a piston with an increased diameter of 0.50 mm; +1 for a piston with an increased diameter of 1.00 mm; If clearance between cylinder and piston is greater than 0,074 mm, the piston and seal rings must be replaced. Important - Tab. 8.1b details the dimensional values of new components only. Tab. 8.1b PISTON; Ø CYLINDERS (± 0.007 mm); Ø PISTON (± 0.007 mm); CLEARANCE VALUE (mm). STD; 88.010; 87.950; 0.046 + 0.074. +0.10; 88.110; 88.050. +0.50; 88.510; 88.450. +1.00; 89.010; 88.950. | Fig 8.16 Fig 8.17 |
| Important - With a feeler gauge, measure the clearance of the seal ring in the respective seat (value L1, L2 e L3 ). - If the clearance does not comply with the values shown in the Tab. 8.8 , replace the seal rings and the piston. Tab 8.8 SEAL RINGS; CLEARANCE VALUE (mm). U1 (L1); 0.110 - 0.150. U2 (L2); 0.070 - 0.115. U3 (L3); 0.030 - 0.065. | Fig 8.18 and 8.19 |

## Cylinder head

| 8.6.1 Flatness check Put the cylinder head on a surface plate and, with a dial gauge, check the flatness of surface C . The MAX value of allowable irregularity of surface C is 0.10mm. If the value is not observed, you are required to grind surface C . The MAX removal allowed is 0.20 mm. Important - Grinding is to be performed with sleeves A of the injectors assembled. | Fig 8.20a - Fig 8.20b |
| --- | --- |
| 8.6.2 Valve seats check Thoroughly clean the valves and their seats with. Measure indentation B of each valve with regard to the cylinder head surface C , which is to be a MIN of 0.60 mm and MAX of 0.85 mm. The B MAX indentation allowed on worn components is 1.10 mm. If the measured value does not correspond with the values indicated, replace the worn component. Important - The seats must be worked after driving to reach value B , go to a rectification workshop for such operations. 8.6.3 Valve springs Using a dynamometer, subject the spring to two different forces (in Tab. 8.9) and check that the length of the spring corresponds to the values indicated in the table. (*1) The code ED0057551850-S is installed from S/N 4418801760 Tab 8.9 WEIGHT (kg); LENGHT (mm). ED0057552810-S; ED0057551850-S (*1). 0; 0; Z; 48.34. 13.5; 20.4; Z1; 30.00. 19.5; 29.8; Z2; 22.00. | Fig 8.21 Fig 8.22 |
| 8.6.4 Valve guides check Measure the diameters D and E of the rods and guides valve (Tab. 8.10) . If the diameters don't correspond to the values indicated, replace the valves or guides. The MAX allowed value of wear for D and E is 0.10 mm. Observe values G from surface F when assembling guides H (Tab. 8.10) . Important - Carry out the measurements in different points to detect any ovalisation and/or concentrated wear. - Tab. 8.10 details the dimensional values of new components only. Tab 8.10 Valve stem - valve guide dimensions REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). D; 5.978 - 5.990; 0.040 - 0.064. E; 6.030 - 6.042. G; 7.000 - 7.020. | Fig 8.23 |
| 8.6.5 Valve guides replacement The intake and exhaust guides are both made out of grey iron with pearlitic phosphoric matrix and they have the same dimensions. The guides are press-fit assembled; assembly is possible by cooling the guides with the aid of liquid nitrogen. Before assembling a new guide, measure value L and M ,calculate the press-fit value, which must observe the values in Tab. 8.11 . Observe values G from surface F when assembling guides H (Tab. 8.10 - Fig. 8.23) . Important - The guides must be worked for value E (Tab. 8.10 - Fig.8.23) after driving. Contact a rectification workshop for such operations. Tab 8.11 valve guides - housing dimensions REF.; DIMENSIONS (mm); PRESS-FIT VALUE (mm). L; 10.000 - 10.015; 0.030 - 0.054. M; 10.045 - 10.054. | Fig 8.24 |
| 8.6.6 Rocker arm check Measure values W1 in correspondence with holes M located on rocker arm gudgeon L (seen from B in Fig. 8.25 ). Measure values W2 (Fig. 8.27). Based on the values measured, calculate the clearance between W1 and W2 , which is to observe the values in Tab. 8.12. Check that all oil pipes N and M are free from impurities or obstructions. Tab 8. 12 REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). W1; 19.985 - 20.005; 0.035 - 0.076. W2; 20.040 - 20.061. Fig. 8.26 | Fig 8.25 Fig 8.27 |

## Oil pump check

| 8.7.1 Dimensional and visual check Perform the operations described in Par.7.8.1 and Par.7.8.4. Measure clearance value B between the rotor teeth, the value of allowable wear is MAX 0.28 mm. Clean all the components thoroughly, check that the work surfaces C of the rotors and pump body are not worn. Important - Should the results from checks carried out not be in accordance with the conditions described, replace the timing system carter together with the oil pump. On assembly, references A must be visible. | Fig 8.28 Fig 8.29 |
| --- | --- |
| 8.7.2 Check rotors clearance Important Replace carter R complete with its oil pump, if there are signs of wear in area P of surface Q (Fig. 8.32 - 8.32a) . Measure values G and H (Fig. 8.30) . Measure values L, M and N (Fig. 8.31) . According to the values measured, calculate the clearance between G and H, L and M and L and N which are to observe the values in Tab. 8.13 . For assembly, carry out the operations described from Par. 9.11.3 to Par. 9.11.4 . | Fig 8.30 |
| Tab 8.13 REF.; DIMENSIONS (mm); CLEARANCE VALUE (mm). G; 82.820 - 82.855; 0.395 - 0.320. H; 82.460 - 82.500. L; 17.500 - 17.525; 0.036 - 0.086. M; 17.464 - 17.439. N. | Fig 8.31 |
| Fig 8.32 - Fig 8.32a |  |
| 8.7.3 Oil pressure valve check Measure the free length F of spring D , which must be 47.91 mm . If the measured value does not correspond to the value indicated, replace spring D . Tab 8.14 POS; DESCRIPTION. B; Oil stopper. C; Gasket. D; Spring. E; Piston. | Fig 8.33 |