??????????? ??????? ?? ?? ??????????? ??????????

KDI 3404 TCR Workshop Manual (Rev. 10.4)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about optional components

## Heater (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.1.1 Disassembly - Undo the screws A . - Remove the flange C . - Remove the heater E and the relevant gaskets F . | Fig 11.1 |
| 11.1.2 Assembly Important - Always replace gaskets F , with each assembly. - In sequence, fit the manifold G with the gasket F , the new heater E , the second gasket F , the flange C , the washers H , the screws A and the cable B . - Secure the flange C with the screws A (tightening torque at 22 Nm ). | Fig 11.2 |

## Idler gear (for 3rd / 4th PTO)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.2.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Loosen capscrews A and remove plate B1 . - Extract gear C . - Remove gudgeon D together with plate B2 . | Fig 11.3 |
|  | Fig 11.4 |
| 11.2.2 Assembly Important - Make sure that gudgeon D has no impurities inside. - On capscrew A , assemble: - plate B1 - gudgeon D - gear C - plate B2 . | Fig 11.5 |

| - Position gear unit C1 onto crankcase E , complying with reference J1 , J2 with gear F . - Secure unit C1 by means of capscrew A (tightening torque 25 Nm ). NOTE: the reference J1 can have 2 different configurations for the gear C , Fig. 11.6b shows the correct position of the reference J1 for both configurations. | Fig 11.6 Fig 11.6b |
| --- | --- |

## 3rd PTO (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.3.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Extract gear A . | Fig 11.7 |
| 11.3.2 Assembly - Insert gear A into the seat of crankcase B , fitting the shaft of pump C in gear A . | Fig 11.8 |

## 4th PTO (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.4.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Extract gear A . | Fig 11.9 |
| 11.4.2 Assembly - Insert gear A into the seat of crankcase B , fitting the shaft of pump C in gear A . | Fig 11.10 |

## Balancer device (replacement)

| Importante - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.5.1 Disassembly - Perform the operations described in Par. 11.2.1 . - Extract shaft A1 e A2. | Fig 11.11 |
| 11.5.2 Assembly - Lubricate gudgeon C of shaft A1 and A2 with oil. - Insert shaft A1 into seat B1 of the crankcase, complying with reference D of gear E . - Insert shaft A2 into seat B2 of the crankcase. - Perform the operations described in Par. 11.2.2 . NOTE: the shafts A1 and A2 are different and it is important not to invert the assembly position, the reference D is specific for the shaft A1 and is timed with the gear E , the reference J2 is specific for the shaft A2 and is timed with the idler gear F (see the Fig. 11.12b ). Fig 11.12 Fig 11.12b |  |

## ETB (replacement)

| - Unscrew screws A and remove the ETB valve B with its gasket C . Important - Always replace the gasket C at each assembly. | Fig. 11.13 |
| --- | --- |
| - Fix the ETB valve B and its gasket C by means of screws A (tightening torque 10 Nm ) | Fig. 11.14 |

## EGR-T (replacement)

| - Unscrew sensor A and its gasket B . Important - Before assembling the new sensor, see Par. 2.17.5 - Always replace the gasket B at each assembly. | Fig. 11.15 |
| --- | --- |
| - Fix sensor A and gasket B on their support C (tightening torque 20 Nm ). | Fig. 11.16 |

## EGTS (Black | Yellow - replacement)

| - Unscrew sensors A . Important - Before assembling the new sensor, see Par. 2.17.6 | Fig. 11. 17 |
| --- | --- |
| - Fix sensors A on the ATS B (tightening torque 30 Nm ). | Fig. 11.18 |

## DPF & DOC filter (replacement)

| Danger - Highly carcinogenic material! - The dust contained inside the DPF is particularly fine and therefore classified as highly dangerous to living beings. - Before proceeding to any operation, wear: dust mask gloves goggles - Do not allow any other operator who is not equipped with the above mentioned protective equipment to approach. |  |  |  |
| --- | --- | --- | --- |
| - Obtain a new replacement KIT for the DPF filter or a replacement KIT with regenerated DPF filter from your spare parts service. Important - Before disassembling/assembling any sensors, see Par. 2.17.5 and 2.17.6 - See service letter 710023. - Always replace the seal P upon every assembly. - During disassembly, do not expose the DPF to the surrounding environment for a long time and store it in a sealed bag as soon as possible. - During disassembly operations, avoid using electric screwdrivers as vibrations could release the dust inside the DPF - Do not try to clean the DPF - Do not blow with compressed air - Do not release the DPF or the dust contained in it in the environment - Dispose of the DPF only in authorised centres - Perform the operations indicated in point 1 of Par. 11.8 . - Open the package of the DPF replacement KIT, taking care not to damage it. - Unscrew screw A and loosen clamps B , then remove the Delta-P sensor D . - Disconnect pipes C from their unions J and remove the Delta-P sensor D . - Unscrew and remove unions J . | Fig. 11.19 |
| - Loosen clamps E and remove manifold N . | Fig. 11.20 |
| - Loosen clamp G and remove the DPF filter H . NOTE: do not remove clamp G . - Inspect the DPF filter to detect any visible signs of oil contamination. - Put the removed DPF filter in the plastic container supplied with the replacement KIT and send it to your spare parts service using the replacement KIT package. NOTE : a residual value will be recognised depending on whether the used DPF is intact and recyclable or damaged and requiring a correct disposal. | Fig. 11.21 |
| - Loosen the clamps Q and S and remove the manifold with DOC M from the flexible hose R . | Fig. 11.22 |
| - Fit the new manifold with the DOC filter M on the support T , inserting the flexible hose R inside the inlet of the manifold M . | Fig. 11.23 |
| Important - Do not apply any tension during the assembly of components. - Before fastening the manifold M , it must be oriented in the same position as the previously installed DOC filter. - Tighten the clamp Q (tightening torque of 10 Nm ). - Tighten the clamp S (tightening torque of 12 Nm ). | Fig. 11.24 |
| - Insert the gasket P on the manifold with DOC filter M . - Insert the new DPF filter H inside clamp G . - Insert the DPF filter H on manifold M until reaching the gasket P . | Fig. 11.25 |
| Important - Before fastening the DPF filter H , it must be oriented in the same position as the previous filter. - Do not apply any tension during the assembly of components. - Fasten the DPF filter H with clamp E (tightening torque 12 Nm ). | Fig. 11.26 |
| - Insert gasket P on the DPF filter H . - Insert manifold N on the DPF filter H until reaching the gasket P . Important - Before fastening manifold N , it must be oriented in the original position. - Fasten manifold N with clamp E (tightening torque 12 Nm ). | Fig. 11.27 |
| - Position support K in contact with support K1 and fasten clamp G (tightening torque 12 Nm ). | Fig. 11.28 |
| - Screw unions J on the DPF filter H and position them about 20° away from the centre line (as in Fig. 11.30 ). | Fig. 11.29 |
| - Fix unions J (tightening torque see service letter 710015 ). | Fig. 11.30 |
| - Fit pipes C on unions J and fasten them with clamps B . - Fix the Delta-P sensor D on its support K by means of screw A (tightening torque 10 Nm ). - Provide the machine owner with the warranty certificate of the new DPF filter KIT installed. - Perform the " DPF replacement " procedure to reset the ASH & SOOT through the relevant KOHLER diagnostic tool interfaced with the ECU. | Fig. 11.31 |