Ar?zalarla ilgili bilgiler

KDI 1903TCR - TCRE5 - TC Owner manual (Rev_20)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1903tcr |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about failures

## Useful information about failures

- This chapter contains information about the problems that may appear during engine operation with its causes and trouble shooting Tab. 7.2 .

- In some cases, you shall turn off the engine immediately to avoid further damage Tab. 7.1 .

Tab 7.1

| THE ENGINE MUST BE IMMEDIATELY TURNED OFF WHEN |  |
| --- | --- |
| 1 | Warning RED light turn on |
| 2 | The oil pressure indicator light turns on while running |
| 3 | The engine rpms suddenly increase and decrease |
| 4 | A sudden and/or unusual noise is heard |
| 5 | Colour the exhaust fumes suddenly darkens |

Tab 7.2

| TROUBLES | POSSIBLE CAUSE | SOLUTION | PAR. |
| --- | --- | --- | --- |
| Warning YELLOW light turn on | Engine ECU has detect a malfunctions | Contact KOHLER authorised workshops | -- |
| The engine does not start | Sulphated battery terminals corroded | Clean the battery terminals | -- |
|  | Battery voltage too low | Recharge the battery or replace it | -- |
|  | Low fuel level | Refuel | 4.4 |
|  | Frozen fuel | Contact KOHLER authorised workshops | -- |
|  | Clogged fuel filter | Replace with a new filter | 6.4 |
|  | Air suction in fuel system | Contact KOHLER authorised workshops | -- |
|  | Clogged air filter | Replace with a new filter | 6.5 |
|  | Clogged pipes | Contact KOHLER authorised workshops | -- |
|  | Intake or exhaust system clogged | Replace with a new fuse; if the problem persists, Contact KOHLER authorised workshops | -- |
|  | Intake or exhaust system clogged | Contact KOHLER authorised workshops | -- |
| Engine starts but stops | Inefficient electrical connections | Clean the electrical contacts; if the problem persists, Contact KOHLER authorised workshops | -- |
|  | Sulphated battery terminals | Clean the battery terminals | -- |
|  | Clogged fuel filter | Replace with a new filter and clean the tank | -- |
|  | Clogged fuel pipes | Contact KOHLER authorised workshops | -- |
| Engine doesn't rev up | Safety protocol in starting | Wait some seconds | -- |
|  | Throttle to MAX in starting | Release the throttle and wait some seconds | -- |
| RPM instability at idle speed | Clogged fuel pipes | Contact KOHLER authorised workshops | -- |
| Low RPM at idle speed | Clogged fuel pipes | Contact KOHLER authorised workshops | -- |
|  | Poor quality fuel | Clean the tank and refuel with quality fuel | 2.5 |
| Blue smoke | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops | -- |
|  | Clogged air filter | Replace with a new filter | 6.5 |
| Excessive fuel consumption | Clogged air filter | Replace with a new filter | 6.5 |
|  | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops | -- |
| Engine lost its initial performance | Clogged air filter | Replace with a new filter | 6.5 |
|  | Clogged fuel pipes | Contact KOHLER authorised workshops | -- |
|  | Cheap fuel | Clean the tank and refuel with quality fuel |  |
|  | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops | -- |
| Slow acceleration | Clogged fuel filter | Replace the fuel filter | 6.4 |
| Engine jerking | Clogged fuel pipes | Contact KOHLER authorised workshops |  |
| Engine overheats | Insufficient coolant level | Fill up to the level | 4.6 |
|  | High oil sump level | Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops | -- |
|  | Clogged radiator | Clean the radiator; if the problem persists, contact KOHLER authorised workshops | -- |

In the event that the solutions proposed in Tab. 7.2 do not eliminate the trouble, contact a KOHLER authorized workshop