?????????? ???? ?????????? ????????

KDI 2504 M Workshop manual (Rev. 09.6)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
| Technical Documentation | ED005302-961 | 9.6 | 04/2013 | 03/2023 | M. SILVESTRONE | V. MANINI |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Fluids filling information

## Engine oil

| Important - Before proceeding with operation, read Par. 3.3.2 . - Do not use the engine with the oil level below the minimum. |  |
| --- | --- |
| - Loosen the oil filler cap A or the oil filler cap C if the cap A is not accessible. - Add the type and amount of oil recommended ( Tab. 2.2 ). | Fig 10.1 |
| - Remove the oil dipstick B and check that the level is up to but does not exceed the MAX . - If the oil level is not at MAX , insert more oil until the MAX level is reached as indicated on the dipstick. - Re-tighten the cap A or C . NOTE: See the Par. 11.1 for the various configurations of the oil dipstick. | Fig 10.2 |

## Coolant

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Screw the cap. G , replacing the copper gasket (Tightening torque of 50 Nm ). | Fig 10.3 |
| - Tighten the cap E , replacing the copper gasket D . | Fig 10.4 |
| - Refill the radiator with coolant (refer to Par. 2.6 for the liquid specifications). - Top liquid up until the pipes inside the radiator are covered by about 5 mm. - For engines equipped with separate expansion tank, pour in fluid until reaching the max level mark. - Loosen the screw F on the head H , release any air and tighten the screw F (Tightening torque of 8 Nm - Fig. 10.7 ). - Start the engine without the radiator cap A or the expansion tank ( C ) cap B . | Fig 10.5 |
| - Keep it running at idle speed until the cooling liquid level goes down and becomes steady (the waiting times varies according to the ambient temperature). - Stop the engine and allow it to cool. - If there is an expansion tank ( C ) top liquid up to the mark MAX . - Without expansion tank top liquid up until the pipes inside the radiator are covered by 5 mm. Do not overfill the radiator, but leave room for the coolant to expand. - Tighten the radiator cap A or the expansion tank ( C ) cap B . Warning - Before starting make sure that the radiator cap and expansion tank cap, if present, are installed correctly to avoid spillage of liquid or vapour at high temperatures. - After a few hours of operation stop the engine and allow it to cool. Check and top up the coolant liquid. | Fig 10.6 |
| Fig 10.7 |  |