Informacje o awariach

KDI-TCP 3404 E5 Owner Manual (Rev 01.5)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3404tcr |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about failures

## Useful information about failures

- This chapter contains information about the problems that may appear during engine operation with its causes and trouble shooting Tab. 7.2 . - In some cases, you shall turn off the engine immediately to avoid further damage Tab. 7.1 . 7.1 THE ENGINE MUST BE IMMEDIATELY TURNED OFF WHEN. 1; Warning RED light turn on. 2; The oil pressure indicator light turns on while running. 3; The engine rpms suddenly increase and decrease. 4; A sudden and/or unusual noise is heard. 5; Colour the exhaust fumes suddenly darkens. 7.2 TROUBLES; POSSIBLE CAUSE; SOLUTION; PAR.. Warning YELLOW light turn on; Engine ECU has detect a malfunctions; Contact KOHLER authorised workshops; --. The engine does not start; Sulphated battery terminals corroded; Clean the battery terminals; --. Battery voltage too low; Recharge the battery or replace it; --. Low fuel level; Refuel; 4.4. Frozen fuel; Contact KOHLER authorised workshops; --. Clogged fuel filter; Replace with a new filter; 6.3. Air suction in fuel system; Contact KOHLER authorised workshops; --. Clogged air filter; Replace with a new filter; 6.4. Clogged pipes; Contact KOHLER authorised workshops; --. Intake or exhaust system clogged; Replace with a new fuse; if the problem persists, Contact KOHLER authorised workshops; --. Intake or exhaust system clogged; Contact KOHLER authorised workshops; --. Engine starts but stops; Inefficient electrical connections; Clean the electrical contacts; if the problem persists, Contact KOHLER authorised workshops; --. Sulphated battery terminals; Clean the battery terminals; --. Clogged fuel filter; Replace with a new filter and clean the tank; --. Clogged fuel pipes; Contact KOHLER authorised workshops; --. Engine doesn't rev up; Safety protocol in starting; Wait some seconds; --. Throttle to MAX in starting; Release the throttle and wait some seconds; --. RPM instability at idle speed; Clogged fuel pipes; Contact KOHLER authorised workshops; --. Numero dei giri al minimo basso; Clogged fuel pipes; Contact KOHLER authorised workshops; --. Poor quality fuel; Clean the tank and refuel with quality fuel; 2.5. Blue smoke; High oil sump level; Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops; --. Clogged air filter; Replace with a new filter; 6.4. Excessive fuel consumption; Clogged air filter; Replace with a new filter; 6.4. High oil sump level; Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops; --. Engine lost its initial performance; Clogged air filter; Replace with a new filter; 6.4. Clogged fuel pipes; Contact KOHLER authorised workshops; --. Cheap fuel; Clean the tank and refuel with quality fuel. High oil sump level; Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops; --. Slow acceleration; Clogged fuel filter; Replace the fuel filter; 6.3. Engine jerking; Clogged fuel pipes; Contact KOHLER authorised workshops; --. Engine overheats; Insufficient coolant level; Fill up to the level; 4.6. High oil sump level; Replace the engine oil; if the problem persists, Contact KOHLER authorised workshops; --. Clogged radiator; Clean the radiator; if the problem persists, contact KOHLER authorised workshops; --. In the event that the solutions proposed in Tab. 7.2 do not eliminate the trouble, contact a KOHLER authorized workshop