Information about failures

Owner Manual KDW 502 ¦ 702 ¦ 1003 ¦ 1404 - K-HEM 1003 (Rev. 03.3)

$IMG$

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | KDW 502-702-1003-1404 - K-HEM 1003 |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about failures

## Useful information about failures

- This chapter contains information about the problems that may appear during engine operation with its causes and trouble shooting  Tab.  5.2 .

- In some cases, you shall turn off the engine immediately to avoid further damage  Tab.  5.1 .

5.1

| THE ENGINE MUST BE IMMEDIATELY TURNED OFF WHEN: |  |
| --- | --- |
| 1 | The engine rpms suddenly increase and decrease |
| 2 | A sudden and/or unusual noise is heard |
| 3 | The colour of the exhaust fumes suddenly darkens |
| 4 | The oil pressure indicator light turns on while running |

5.2

| TROUBLES | POSSIBLE CAUSE | SOLUTION |
| --- | --- | --- |
| The engine does not start | Sulphated battery terminals corroded | Clean the battery terminals |
|  | Battery voltage too low | Recharge the battery or replace it |
|  | Low fuel level | Refuel |
|  | Frozen fuel | Contact REHLKO authorised workshops |
|  | Clogged fuel filter | Replace with a new filter |
|  | Air suction in fuel system | Contact REHLKO authorised workshops |
|  | Clogged air filter | Replace with a new filter |
|  | Clogged pipes | Contact REHLKO authorised workshops |
|  | Open fuse | Replace with a new fuse; if the problem persists, contact REHLKO authorised workshops |
|  | Intake or exhaust system clogged | Contact REHLKO authorised workshops |
| Engine starts but stops | Inefficient electrical connections | Clean the electrical contacts; if the problem persists, contact REHLKO authorised workshops |
|  | Sulphated battery terminals | Clean the battery terminals |
|  | Clogged fuel filter | Replace with a new filter and clean the tank |
|  | Clogged fuel pipes | Contact REHLKO authorised workshops |
| RPM instability at idle speed | Clogged fuel pipes | Contact REHLKO authorised workshops |
| Low idle speed | Clogged fuel pipes | Contact REHLKO authorised workshops |
|  | Poor quality fuel | Clean the tank and refuel with quality fuel |
| Blue smoke | High oil sump level | Replace the engine oil; if the problem persists, contact REHLKO authorised workshops |
|  | Clogged air filter | Replace with a new filter |
| Excessive fuel consumption | Clogged air filter | Replace with a new filter |
|  | High oil sump level | Replace the engine oil; if the problem persists, contact REHLKO authorised workshops |
| Engine lost its initial performance | Clogged air filter | Replace with a new filter |
|  | Clogged fuel pipes | Contact REHLKO authorised workshops |
|  | Cheap fuel | Clean the tank and refuel with quality fuel |
|  | High oil sump level | Replace the engine oil; if the problem persists, contact REHLKO authorised workshops |
| Slow acceleration | Clogged fuel filter | Replace the fuel filter |
| Engine jerking | Clogged fuel pipes | Contact REHLKO authorised workshops |
| Engine overheats | Insufficient coolant level | Fill up to the level |
|  | High oil sump level | Replace the engine oil; if the problem persists, contact REHLKO authorised workshops |
|  | Clogged radiator | Clean the radiator; if the problem persists, contact REHLKO authorised workshops |

In the event that the solutions proposed in  Tab.  5.2  do not eliminate the trouble, contact a  REHLKO   authorized workshop.