Technical information

KDI 3404 TCR-SCR Owner Manual (REV_05)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3404tcr |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Technical information

## General description of the engine

- 4-stroke, in-line cylinders Diesel engine; - Liquid-cooling system; - 4 valves per cylinder with hydraulic tappets; - Turbocharger with Waste-gate valve; - Common rail - Direct injection.

## Engine specifications

2.1 TECHNICAL DATA; UNIT OF MEASURE. Engine type; KDI 3404 TCR SCR. Cylinders; n.; 4. Bore; mm; 96. Stroke; mm; 116. Displacement; cm 3; 3359. MAX INCLINATION DURING OPERATION (even in combined); α; 40° max 30 min.. α; 45° max 1 min.. OIL CAPACITY (MAX level.) including oil filter; standard version; lt.; 15.6. DRY WEIGHT; Kg; 394.

## Engine dimensions (mm - base version)

## Oil

Important

- The engine may be damaged if operated with improper oil level.

- Do not exceed the MAX level because a sudden increase in engine rpm could be caused by its combustion.

- Use only the recommended oil to ensure adequate protection, efficiency and service life of the engine.

- The use of lubricants other than recommended may shorten the engine life.

- Viscosity must be appropriate to the ambient temperature to which the engine is to be exposed.

Danger

- Prolonged skin contact with the exhausted engine oil can cause cancer of the skin.

- If contact with oil cannot be avoided, thoroughly wash your hands with soap and water as soon as possible.

- For the exhausted oil disposal, refer to the Par.  DISPOSAL and SCRAPPING .

2.4.1 SAE oil classification

- In the SAE classification, oils are identified according to viscosity without considering any other qualitative characteristic.

- The code is composed of two numbers, which indicate, and must correspond to, the ambient temperature in which the engine operates, the first number refers to the viscosity when cold, for use during winter (" W "), while the second number is for viscosity at high temperatures.

Use oil viscosity based on the experienced air temperature range during the period between oil changes as indicated

in the table below.

Rehlko X-treme 10W-40 formula pro; X-treme 10W-40; X-treme 5W-40 oil are preferred:

Oils meeting the following specifications are also recommended.

2.2

| RECCOMENDED OIL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | TCR STAGE-V (*1) (*2) | TCR TIER IV FINAL (*1) | TCR/D TIER III o NON CERTIFICATO (*3) |
| WITH SPECIFICATIONS | API | CJ-4 Low S.A.P.S CK-4 Low S.A.P.S | CJ-4 Low S.A.P.S CK-4 Low S.A.P.S | CI-4 Plus CI-4 CH-4 |
|  | ACEA | E6 Low S.A.P.S. | E6 Low S.A.P.S. | E7 E4 |
| VISCOSITY | SAE | 0w-40 (-40°C ÷ +50°C) 5w-40 (-30°C ÷ +50°C) 10w-40 (-25°C ÷ +50°C) | 0w-40 (-40°C ÷ +50°C) 5w-40 (-30°C ÷ +50°C) 10w-40 (-25°C ÷ +50°C) | 0w-40 (-40°C ÷ +50°C) 5w-40 (-30°C ÷ +50°C) 10w-40 (-25°C ÷ +50°C) |

- Low S.A.P.S. technology (oil with low Sulfated Ash, Phosphorus, Sulfur content) keeps catalyst in good working conditions. The presence of sulfated ash, phosphorus and sulfur causes with time the catalyst clogging and its consequent inefficiency.

- For Mid S.A.P.S oil sequence the sulfated ash level is the same as API CJ-4 ≤ 1.0% but as per ACEA standardization those oils are referenced as mid SAPS.

- Filtration of oils is critical to proper operation and lubrication; always change filters regularly as specified in this manual.

(*1) NOTE : Do NOT use fuel with sulphur content above 15ppm.

(*2) - On all engines compliant with Stage-V emission regulation (engines with DPF device), the oil to use must comply with the specification API CJ-4 Low S.A.P.S or ACEA E6 Low S.A.P.S.

(*3) - NOTE : Do NOT use fuel with sulphur content above 500ppm.

(*3) - NOTE : Low S.A.P.S. oils, sulfate ashes <1% may not be used with fuels with a sulfur content >50ppm.

## Fuel

| Important - Use of other types of fuel could damage the engine. Do not use dirty diesel fuel or mixtures of diesel fuel and water since this will cause serious engine faults. | Warning - Clean fuel prevents the fuel injectors from clogging. Immediately clean up any spillage during refuelling. - Never store diesel fuel in galvanized containers (i.e. coated with zinc). Diesel fuel and the galvanized coating react chemically to each other, producing flaking that quickly clogs filters or causes fuel pump and/or injector failure. - Any failure resulting from the use of fuel other than Tab. 2.3 will not be covered by warranty. |
| --- | --- |
| Cetane number of 40 minimum. Cetane number greater than 47 is preferred, especially for temperatures below –20 °C (–4 °F) or elevations above 1675 m (5500 ft.). Highly Regulated Engines (EU, USA, CANADA, JAPAN, CHINA, INDIA) - EU EN 590 - North America ASTM D975 No. 2-D S15; No. 1-D S15 - Japan JIS K 2204 No. 1; No. 2 - India BS-V ULSD; BS-VI ULSD - China GB 19147-2016 ULSD; GB 17691—2018 ULSD - HVO EN 15940 or ASTM D975 - Military NATO fuel F-54 S < 10ppm Less or Not Regulated Engines (Rest of the World and Military applications) - Equivalent to all above specification - Equivalent to all above specification but with Suplhur < 500ppm - Equivalent to all above specification but with Suplhur < 2000ppm - F-34/F-35 (kerosene NATO designation) (1) - F-44 (kerosene, NATO designation) (1) - F-63 (kerosene, NATO designation, equivalent to F-34/F-35 with additives (1) - JP-8 (kerosene, US military designation) (1) - JP-5 (kerosene, US military designation) (1) - Jet A / A1 (kerosene for civil aviation) (1) (1) - Operations with Jet Fuels: The jet fuels can be used but only adopting an additional fuel filter with lubricity doser. Because of lower density and greater leak fuel volume due to lower viscosity, depending on the engine speed and torque, a power loss up to 10% is possible. There are some problematical fuel proprieties amongst the listed jet fuels (viscosity, lubricating capacities and low boiling point). A slight increase in wear in the injection system is to be expected which can lead to a statistically shorter life of these components. Sulphur content must be below 2000ppm. 2.5.1 Fuel Additives To aid in maintaining performance of the engines injection system, Rehlko has developed a family of fuel additives product for North America market. The Extra Performance Fuel Additive Arctic Formula a fuel conditioner in winter, the Extra Performance Fuel Additive a fuel injector deposit removal and prevention. See your local Rehlko dealer for availability. 2.3 Rehlko Extra Performance Fuel Additive (North America only). Use Rehlko Injector Cleaner Diesel Fuel Additive before filling the tank once a year or every 500hrs with summer or winter grade fuel. Formulated to clean deposits from injectors and provide lubrication to high pressure pump and injectors to reduce wear, increasing fuel quality. Compatible with all exhaust emission system, including diesel particulate filters (DPFs). Follow all recommended instruction on product label.. Rehlko Extra Performance Fuel Additive Arctic Formula (North America only). Use Rehlko Extra Performance Fuel Additive Arctic Formula, which contains anti-gel chemistry to treat non-winter grad fuel (No. 2-D in North America) during the cold-weather season. This extends the operability to about 10°C (18°F) below the could point. For operability at even lower temperatures, use winter grade fuel. Treat the fuel when the outside temperature drops below 0°C (32°F). For best results, use untreated fuel and follow all recommended instruction on product label. Cloud point is the temperature at which wax begins to from in the fuel causing filters to plug.. Fuel additives with biocide/algaecide functions are allowed in case of storage of fuel in the tank for long periods (one year or more).; For suggested brands and types contact Rehlko staff. NOTE: These additives must be diluted in the fuel following the product prescribed percentages when filling the tank.. |

## DEF

| - Also known as "AUS 32" in Europe, "DEF" or "Urea Solution" in the USA, it is registered with the “AdBlue ® ” brand at the Verband der Automobilindustrie (VDA), and must comply with the following ISO standards: - ISO 22241-1 Quality requirements - ISO 22241-2 Test Methods - ISO 22241-3 Handling, transportation and Storing - ISO 22241-4 Refilling Interface - The DEF tank must be filled by means of the specific automatic filling nozzle at the authorised distributors, refer to the car manual for refilling operations. - Upon refilling, comply with the MAX level indicated on the tank. - During the refilling operations prevent any impurity from entering the tank. - At tank inlet there is a filter that must be periodically cleaned or replaced (see maintenance and replacement table - for tank supplied by REHLKO only). - DEF quality must comply with the specifications described in Table 2.5. |  |  |
| --- | --- | --- |
| Warning - Do not mix DEF with fuel or other liquids (including water) and do not fill the fuel tank with DEF. - The presence of DEF inside the specific tank is required to start the engine. - Purchase in containers: the container, even if opened, can be stocked with the same conditions of a sealed container. - Do not stock the container with a temperature higher than 35°C as this could alter the DEF. - In case of DEF freezing inside the container (< -11,5 °C ¦ 11,3 °F), DEF can be used when it returns to its liquid state. - Do not expose DEF to direct sunlight. - In case of opening and closing of the original purchase container, DEF must be checked through a spectrometer to check its quality before use. - Do not insert altered DEF in the tank as the engine could not respect the emission parameters, generate DCU errors and as a consequence turn off or fail to start the engine. |  |  |
| 2.5 PARAMETERS; UNIT OF MEASUREMENT; VALUE. Title; Weight %; 31,8 ÷ 33,2. Density at 20°C; kg/m3; 1.087 ÷ 1.093. Refraction index at 20°C; °C; 1,3814 ÷ 1,3843. Alkalinity like NH3; Weight %; < 0,2. Biuret; Weight %; < 0,3. Aldehyde; mg/kg; < 5. Insoluble; mg/kg; < 20. Phosphates like PO4; mg/kg; < 0,5. Calcium; mg/kg; < 0,5. Iron; mg/kg; < 0,5. Copper; mg/kg; < 0,2. Zinc; mg/kg; < 0,2. Chrome; mg/kg; < 0,2. Nickel; mg/kg; < 0,2. Aluminium; mg/kg; < 0,5. Magnesium; mg/kg; < 0,5. Sodium; mg/kg; < 0,5. Potassium; mg/kg; < 0,5. Freezing point; °C; -11. |

## Coolant recommendation

Rehlko XtendedTM prediluted coolant 50% ethylene glycol OAT is preferred. Coolant meeting the following specifications are also recommended. A mixture of 50% demineralized water and 50% low silicate ethylene glycol based coolant liquid must be used. Use a Long Life or Extended Life Heavy Duty OAT coolant free of: silicates, phosphates, borates, nitrites and amines. The following ethylene-glycol based engine coolant for all models within KSD engine family may be used: - OAT (Organic Acid Technology) Low Silicate: ASTM D-3306 D-6210 - HOAT (Hybrid Organic Acid Technology) Low Silicate: ASTM D-3306 D-6210 The above coolants in concentrated formulation must be mixed with distilled, deionized, or demineralized water. A pre-mixed formulation (40-60% or 50-50%) can be used directly when available. Important - Do not mix ethylene glycol and propylene glycol based coolants. Do not mix OAT and HOAT based coolant. OAT performance life can be drastically reduced if contaminated with nitrite-containing coolants. - Never use automotive-type coolants. These coolants do not contain the correct additives to protect heavy – duty diesel engines. OAT coolants are maintenance free up to 5 years or 4000hrs of operation , provided that the cooling system is topped up using the same type of coolant. Do not mix different coolant types. Test the coolant condition annually with coolant test strips. HOAT are not all maintenance free and it is recommended to have SCA (Supplemental Coolant Additives) added at the first maintenance interval.

## Battery recommendation

Battery not supplied by REHLKO 2.6 RECOMMENDED BATTERIES. AMBIENT TEMPERATURE; STARTER MOTOR; OIL; BATTERY TYPE; MACHINE CONDITION. ≥ - 25°C; 3.2kW + HEATER; 10W-40 5W-40; 880CCA SAE (=1000 A (EN)); light hydraulic parasitic loads or mechanical clutch and gearbox. ≥ - 25°C; 4.2kW + HEATER; 1000CCA SAE =(1250 A (EN)); heavy hydraulic parasitic loads.

## Inducement strategy of ATS system

The inducement is the operation aimed at reducing the engine performance due to a malfunction or tampering with the ATS system detected by the DCU. The inducement degree is decided by the ECU according to the error detected by the DCU. The information on the car panel or the activation of the inducement can occur for the following reasons: - Low DEF level - Poor DEF quality - Interruption of DEF supply - EGR valve malfunctioning - Tampering with the monitoring systems of the ATS system. The inducement strategy is applied according to: - detected problem - hours passed.

NOTE: Hours are reset after 40h if the DCU does not detect any fault, otherwise the hours are added to those already counted. For low DEF level, activation depends on the percentage of liquid inside the DEF tank, and the fault hours are not counted.

The strategy for the different faults is listed below (Stage V - EU): The inducement can have the following 2 levels: - Level 1: 25% reduction of the MAX available torque. - Level 2: 50% reduction of the MAX available torque and 40% reduction of MAX rpm available. Before activating the inducement (level 1 or level 2) the ECU activates a warning or a warning light on the car panel (refer to the car documentation for the warning type). Low DEF level - information activation on car panel: <10% of MAX level - Level 1 inducement: <2.5% of MAX level - Level 2 inducement: 0% of MAX level Poor DEF quality - information activation on car panel: upon fault detection - Level 1 inducement: after 10h from fault detection - Level 2 inducement: after 20h from fault detection Interruption of DEF supply - information activation on car panel: upon fault detection - Level 1 inducement: after 10h from fault detection - Level 2 inducement: after 20h from fault detection EGR valve malfunctioning - information activation on car panel: upon fault detection - Level 1 inducement: after 36h from fault detection - Level 2 inducement: after 100h from fault detection Tampering with the monitoring systems of the ATS system - information activation on car panel: upon fault detection - Level 1 inducement: after 36h from fault detection - Level 2 inducement: after 100h from fault detection

The strategy for the different faults is listed below (Tier 4 Final - USA): The inducement can have the following 3 levels: - Level 1: 25% reduction of the MAX available torque. - Level 2: 50% reduction of the MAX available torque and 40% reduction of MAX rpm available. - Level 3: the engine operates at minimum rpm and MAX torque available Before activating the inducement (level 1, level 2 or level 3) the ECU activates a warning or a warning light on the car panel (refer to the car documentation for the warning type). Low DEF level - information activation on car panel: <10% of MAX level - Level 1 inducement: <5% of MAX level - Level 2 inducement: <2.5% of MAX level - Level 3 inducement: <0.5% of MAX level Poor DEF quality - information activation on car panel: upon fault detection - Level 1 inducement: after 1h from fault detection - Level 2 inducement: after 2h from fault detection - Level 3 inducement: after 3h from fault detection Interruption of DEF supply - information activation on car panel: upon fault detection - Level 1 inducement: after 1h from fault detection - Level 2 inducement: after 2h from fault detection - Level 3 inducement: after 3h from fault detection EGR valve malfunctioning - information activation on car panel: upon fault detection - Level 1 inducement: after 1h from fault detection - Level 2 inducement: after 2h from fault detection - Level 3 inducement: after 3h from fault detection Tampering with the monitoring systems of the ATS system - information activation on car panel: upon fault detection - Level 1 inducement: after 1h from fault detection - Level 2 inducement: after 2h from fault detection - Level 3 inducement: after 3h from fault detection