Technical information

KDI 2504 M Workshop manual (Rev. 09.6)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
| Technical Documentation | ED005302-961 | 9.6 | 04/2013 | 03/2023 | M. SILVESTRONE | V. MANINI |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Technical information

## Engine specifications

| MANUFACTURER SPECIFICATIONS AND OPERATION |  |  |  |
| --- | --- | --- | --- |
| GENERAL INFORMATION | UNIT OF MEASURE | KDI 1903 M | KDI 2504 M |
| Operating cycle |  | diesel - 4 stroke |  |
| Cylinders | N° | 3 | 4 |
| Bore x stroke | mm | 88x102 |  |
| Displacement | cm 3 | 1861 | 2482 |
| Compression ratio |  | 18.4:1 |  |
| Intake |  | Atmospheric pressure |  |
| Cooling |  | Liquid |  |
| Crankshaft rotation (view from flywheel side) |  | Counterclockwise |  |
| Combustion sequence |  | 1-3-2 | 1-3-4-2 |
| Timing System |  |  |  |
| Valves per cylinder | N° | 4 |  |
| Timing System |  | Rods and rocker arms - Camshaft in the crankcase |  |
| Tappets |  | Hydraulic |  |
| Injection |  | Direct |  |
| Engine dry weight | Kg | 210 | 244 |
| MAX inclination 30' continuous operation | (min./a) | 25° |  |
| MAX inclination 1' continuous operation | (min./a) | 35° |  |
| Volume of aspirated air (2600 rpm) | Kg/h | 2.2 | 2.9 |
| POWER AND TORQUE |  |  |  |
| GENERAL INFORMATION | UNIT OF MEASURE | KDI 1903 M | KDI 2504 M |
| MAX operating speed | Rpm | 2600 |  |
| MAX operating power (ISO TR 14396 - SAE J1995 - CE 97/68) | kW | 31 | 41 |
| Maximum torque (at 1500 rpm) | Nm | 133 | 170 |
| Admissible axial load on crankshaft | Kg | 300 |  |
| CONSUMPTIONS |  |  |  |
| GENERAL INFORMATION | UNIT OF MEASURE | KDI 1903 M | KDI 2504 M |
| Specific fuel consumption (best point) | g/kWh | 210 |  |
| Oil consumption | %Fuel | < 0.05 |  |
| FUEL SUPPLY SYSTEM |  |  |  |
| GENERAL INFORMATION | UNIT OF MEASURE | KDI 1903 M | KDI 2504 M |
| Type of fuel |  | Diesel UNI-EN590 - ASTM D975 |  |
| High-pressure fuel injection pump |  | STANADYNE - DB |  |
| Fuel supply |  | Low pressure electric pump |  |
| Fuel filter |  |  |  |
| Filtering surface | cm 2 | 2300 |  |
| Degree of filtration | µm | 5 |  |
| Maximum pressure at injection pump inlet | bar | < 0.5 |  |
| LUBRICATION CIRCUIT |  |  |  |
| GENERAL INFORMATION | UNIT OF MEASURE | KDI 1903 M | KDI 2504 M |
| Fuel |  |  |  |
| Lubrication |  | See Par. 2.4 |  |
| Circuit forced |  | Lobe pump |  |
| Oil sump capacity ( MAX ) | Lt. | 8.9 | 11,5 |
| Oil pressure switch |  |  |  |
| Intervention pressure ( MIN ) | bar | 0.8±0.1 |  |
| Oil filter |  |  |  |
| Maximum operating pressure | bar | 7.0 |  |
| Degree of filtration | µm | 17±2 |  |
| Filtering surface | cm 2 | 1744 |  |  |
| COOLING CIRCUIT |  |  |  |
| GENERAL INFORMATION | UNIT OF MEASURE | KDI 1903 M | KDI 2504 M |
| Coolant | % | See Par. 2.6 |  |
| Water pump | Lt./min | 75 |  |
| Thermostatic valve |  |  |  |
| Opening temperature | °C | +79 |  |
| Stroke at 91°C | mm | 7.50 |  |
| Liquid recirculation | Lt./h | 9 |  |
| ELECTRICAL SYSTEM - ELECTRIC FAN |  |  |  |
| GENERAL INFORMATION | UNIT OF MEASURE | KDI 1903 M | KDI 2504 M |
| Circuit rated voltage | V | 12 |  |
| External alternator (rated current) | A | 80 |  |
| Starter motor power | kW | 2 |  |
| System electrical consumption, excluding: heater, electric pump, electric fan, starter motor | W | 24 |  |
| Coolant temperature indicator light |  |  |  |
| Indicator light operating temperature | °C | +100/+110 |  |

## Engine dimensions (mm)

NOTE : Dimensions vary according to engine configuration.

Fig 2.1

## Performance

Fig. 2.2

N = Automotive rating curve MN = Torque curve C = Specific fuel consumption curve NOTE: Refer to KOHLER for power curves, torque curves and specific consumptions at speeds other than those given above.. Key - N (ISO TR 14396 - SAE J1995 - CE 97/68) AUTOMOTIVE RATING CURVE : Intermittent duty at variable speed and load. Engine capacity at intermittent conditions with variable speed and load. - MN: = TORQUE RATING CURVE : Also called twisting moment, it is the push generated by the engine through transmission. The highest engine performance is obtained at the maximum torque. - C = SPECIFIC CONSUMPTION CURVE : Engine fuel consumption in a given time at a certain revolution value. Expressed in g/kW (grams/kilowatt), it expresses fuel yield. * The above curves express indicative values, in that the overall performance depends on the type of application and the ECU control uni. - The ratings reported in the diagram regard the run-in engine, fitted with air and exhaust filters, at the atmospheric pressure of 1 Bar and at a room temperature of +20°C - Maximum rating is guaranteed with a 5% tolerance. Warning - Non approval by KOHLER for any modification releases the company from liability for damage incurred on the engine.

## Oil

Important

- The engine may be damaged if operated with improper oil level.

- Do not exceed the MAX level because a sudden increase in engine rpm could be caused by its combustion.

- Use only the recommended oil to ensure adequate protection, efficiency and service life of the engine.

- The use of lubricants other than recommended may shorten the engine life.

- Viscosity must be appropriate to the ambient temperature to which the engine is to be exposed.

Danger

- Prolonged skin contact with the exhausted engine oil can cause cancer of the skin.

- If contact with oil cannot be avoided, thoroughly wash your hands with soap and water as soon as possible.

- For the exhausted oil disposal, refer to the Par.  DISPOSAL and SCRAPPING .

2.4.1 SAE oil classification

- In the SAE classification, oils are identified according to viscosity without considering any other qualitative characteristic.

- The code is composed of two numbers, which indicate, and must correspond to, the ambient temperature in which the engine operates, the first number refers to the viscosity when cold, for use during winter (" W "), while the second number is for viscosity at high temperatures.

Use oil viscosity based on the experienced air temperature range during the period between oil changes as indicated

in the table below.

Kohler / Rehlko X-treme 10W-40 formula pro; X-treme 10W-40; X-treme 5W-40 oil are preferred:

Oils meeting the following specifications are also recommended.

2.2

| RECCOMENDED OIL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| VISCOSITY | SAE | 10w-30 (-25°C ÷ +40°C) 10w-40 (-25°C ÷ +50°C) 5w-30 (-30°C ÷ +40°C) 0w-40 (-40°C ÷ +50°C) |  |  |  |
| WITH SPECIFICATIONS | API | CI-4 Plus CI-4 CH-4 |  |  |  |
|  | ACEA | E7 E5 |  |  |  |

- Low S.A.P.S. oils, sulfate ashes <1% may not be used with fuels with a sulfur content >50ppm.

- Filtration of oils is critical to proper operation and lubrication; always change filters regularly as specified in this manual.

## Fuel

| Important - Use of other types of fuel could damage the engine. Do not use dirty diesel fuel or mixtures of diesel fuel and water since this will cause serious engine faults. | Warning - Clean fuel prevents the fuel injectors from clogging. Immediately clean up any spillage during refuelling. - Never store diesel fuel in galvanized containers (i.e. coated with zinc). Diesel fuel and the galvanized coating react chemically to each other, producing flaking that quickly clogs filters or causes fuel pump and/or injector failure. - Any failure resulting from the use of fuel other than Tab. 2.3, 2.4 will not be covered by warranty. |
| --- | --- |
| Cetane number of 40 minimum. Cetane number greater than 47 is preferred, especially for temperatures below –20 °C (–4 °F) or elevations above 1675 m (5500 ft.). Highly Regulated Engines (EU, USA, CANADA, JAPAN, CHINA, INDIA) - EU EN 590 - North America ASTM D975 No. 2-D S15; No. 1-D S15 - Japan JIS K 2204 No. 1; No. 2 - India BS-V ULSD; BS-VI ULSD - China GB 19147-2016 ULSD; GB 17691—2018 ULSD - HVO EN 15940 or ASTM D975 - Military NATO fuel F-54 S < 10ppm Less or Not Regulated Engines (Rest of the World and Military applications) - Equivalent to all above specification - Equivalent to all above specification but with Suplhur < 500ppm - Equivalent to all above specification but with Suplhur < 2000ppm - F-34/F-35 (kerosene NATO designation) (1) - F-44 (kerosene, NATO designation) (1) - F-63 (kerosene, NATO designation, equivalent to F-34/F-35 with additives (1) - JP-8 (kerosene, US military designation) (1) - JP-5 (kerosene, US military designation) (1) - Jet A / A1 (kerosene for civil aviation) (1) (1) - Operations with Jet Fuels: The jet fuels can be used but only adopting an additional fuel filter with lubricity doser. Because of lower density and greater leak fuel volume due to lower viscosity, depending on the engine speed and torque, a power loss up to 10% is possible. There are some problematical fuel proprieties amongst the listed jet fuels (viscosity, lubricating capacities and low boiling point). A slight increase in wear in the injection system is to be expected which can lead to a statistically shorter life of these components. Sulphur content must be below 2000ppm. 2.5.1 Fuel Additives To aid in maintaining performance of the engines injection system, Rehlko has developed a family of fuel additives product for North America market. The Extra Performance Fuel Additive Arctic Formula a fuel conditioner in winter, the Extra Performance Fuel Additive a fuel injector deposit removal and prevention. See your local Rehlko dealer for availability. 2.3 Rehlko Extra Performance Fuel Additive (North America only). Use Rehlko Injector Cleaner Diesel Fuel Additive before filling the tank once a year or every 500hrs with summer or winter grade fuel. Formulated to clean deposits from injectors and provide lubrication to high pressure pump and injectors to reduce wear, increasing fuel quality. Compatible with all exhaust emission system, including diesel particulate filters (DPFs). Follow all recommended instruction on product label.. Rehlko Extra Performance Fuel Additive Arctic Formula (North America only). Use Rehlko Extra Performance Fuel Additive Arctic Formula, which contains anti-gel chemistry to treat non-winter grad fuel (No. 2-D in North America) during the cold-weather season. This extends the operability to about 10°C (18°F) below the could point. For operability at even lower temperatures, use winter grade fuel. Treat the fuel when the outside temperature drops below 0°C (32°F). For best results, use untreated fuel and follow all recommended instruction on product label. Cloud point is the temperature at which wax begins to from in the fuel causing filters to plug.. Fuel additives with biocide/algaecide functions are allowed in case of storage of fuel in the tank for long periods (one year or more).; For suggested brands and types contact Rehlko staff. NOTE: These additives must be diluted in the fuel following the product prescribed percentages when filling the tank.. |

## Coolant recommendation

Rehlko XtendedTM prediluted coolant 50% ethylene glycol OAT is preferred. Coolant meeting the following specifications are also recommended. A mixture of 50% demineralized water and 50% low silicate ethylene glycol based coolant liquid must be used. Use a Long Life or Extended Life Heavy Duty OAT coolant free of: silicates, phosphates, borates, nitrites and amines. The following ethylene-glycol based engine coolant for all models within KSD engine family may be used: - OAT (Organic Acid Technology) Low Silicate: ASTM D-3306 D-6210 - HOAT (Hybrid Organic Acid Technology) Low Silicate: ASTM D-3306 D-6210 The above coolants in concentrated formulation must be mixed with distilled, deionized, or demineralized water. A pre-mixed formulation (40-60% or 50-50%) can be used directly when available. Important - Do not mix ethylene glycol and propylene glycol based coolants. Do not mix OAT and HOAT based coolant. OAT performance life can be drastically reduced if contaminated with nitrite-containing coolants. - Never use automotive-type coolants. These coolants do not contain the correct additives to protect heavy – duty diesel engines. OAT coolants are maintenance free up to 5 years or 4000hrs of operation , provided that the cooling system is topped up using the same type of coolant. Do not mix different coolant types. Test the coolant condition annually with coolant test strips. HOAT are not all maintenance free and it is recommended to have SCA (Supplemental Coolant Additives) added at the first maintenance interval.

## Battery recommendations

Battery not supplied by Kohler

Tab. 2.7

| RECOMMENDED BATTERIES |  |
| --- | --- |
| AMBIENT TEMPERATURE | BATTERY TYPE |
| > - 15°C | 12V 100 Ah - 800 CCA/SAE |
| -15°C ÷ -25°C | 12V 110 Ah - 950 CCA/SAE |
| < - 25°C | 12V 120 Ah - 1000 CCA/SAE |

## Periodic maintenance

The intervals of preventive maintenance in Tab. 2.8, Tab. 2.9, Tab. 2.10 and Tab. 2.11  refer to the engine operating under normal operating conditions with fuel and oil meeting the recommended specifications.

2.8

| CLEANING AND CHECKING |  |  |  |  |
| --- | --- | --- | --- | --- |
| OPERATION DESCRIPTION | PERIOD (HOURS) |  |  |  |
|  | 100 | 250 | 500 | 5000 |
| Engine oil level (8) |  |  |  |  |
| Coolant level (8) (9) |  |  |  |  |
| Water presence in fuel filter |  |  |  |  |
| Cartridge dry-type air filter (2) |  |  |  |  |
| Radiator heat-exchange surface (2) (8) |  |  |  |  |
| Alternator belt (8) |  |  |  |  |
| Rubber hose (intake air / coolant) |  |  |  |  |
| Fuel hose |  |  |  |  |
| Starter Motor |  |  |  |  |
| Alternator |  |  |  |  |

2.9

| REPLACEMENT |  |  |  |
| --- | --- | --- | --- |
| OPERATION DESCRIPTION |  | PERIOD (HOURS) |  |
|  |  | 500 | 5000 |
| Alternator belt (3) |  |  |  |
| Cartridge dry-type air filter (2) |  |  |  |
| Intake manifold hose (air filter - intake manifold) (7) |  |  |  |
| Coolant hoses (7) |  |  |  |
| Fuel line hose (7) |  |  |  |
| Coolant | OAT |  |  |
|  | HOAT (10) |  |  |

2.10

| ENGINE OIL AND OIL FILTER CARTRIDGE REPLACEMENT |  |  |
| --- | --- | --- |
| ENGINE VERSION | PERIOD (HOURS) |  |
|  | 250 | 500 |
| KDI Mechanical Injection Stage V (1) |  |  |
| KDI Mechanical Injection Tier 4 Final – Stage IIIB (1) |  |  |
| KDI Mechanical Injection Tier 3 – Stage IIIA (1) |  |  |
| KDI Mechanical Injection Uncertified Engines (1) (11) |  |  |

2.11

| FUEL FILTER AND PREFILTER CARTRIDGE REPLACEMENT |  |  |
| --- | --- | --- |
| ENGINE VERSION | PERIOD (HOURS) |  |
|  | 250 | 500 |
| KDI Mechanical Injection Stage V (1) |  |  |
| KDI Mechanical Injection Tier 4 Final – Stage IIIB (1) |  |  |
| KDI Mechanical Injection Tier 3 – Stage IIIA (1) |  |  |
| KDI Mechanical Injection Uncertified Engines (1) |  |  |

(1) - In case of low use: 12 months.

(2) - The period of time that must elapse before checking the filter element depends on the environment in which the engine operates. The air filter must be cleaned and replaced more frequently under very dusty conditions.

(3) - In case of low use: 36 months.

(7) - The replacement interval is only an indication, it strongly depends from environmental condition and hose status detected during regular visual inspection.

(8) - The first check must be done after 10 hours.

(9) - Test the coolant condition annually with coolant test strips.
(10) - It is recommended to have SCA (Supplemental Coolant Additives) added at the first maintenance interval.

(11) - Read Cap. 2.5, "KDI Mechanical Injection Uncertified Engines (no EGR Engines)"

## Fuel system

2.9.1 Supply system Important - The high pressure supply injection system is highly susceptible to damage if the fuel is contaminated. - It is crucial that all components of the injection circuit are thoroughly cleaned before the components are removed. - Thoroughly wash and clean the engine before maintenance. - Contamination in the fuel supply injection system may cause a reduction in effectiveness / operation of engine fault indication. - If the engine is cleaned with high pressure washer, then the nozzle must be kept at a minimum distance of 200mm from the surface, and not directed at electrical components and connectors.

| The fuel supply system is under low pressure from the tank 1 to the high-pressure fuel injection pump 5 . NOTE : The representation of fuel tank is purely indicative. Component not necessarily supplied by KOHLER . Tab 2.12 POS.; DESCRIPTION. 1; Fuel tank. 2; Fuel supply hose from the tank to the injection pump. 3; Fuel filter. 4; Electrical fuel feed pump. 5; Injection pump. 6; Injector high-pressure hose from the injection pump to the injectors. 7; Injectors. | Fig 2.4 |
| --- | --- |
| 2.9.2 Fuel return circuit The fuel return circuit is under low pressure. NOTE : The representation of fuel tank is purely indicative. Component not necessarily supplied by KOHLER . Tab 2.13 POS.; DESCRIPTION. 1; Injectors. 2; Injectors fuel return pipe. 3; Injection pump. 4; Fuel tank. 5; Fuel return pipe to the tank. | Fig 2.5 |

| 2.9.3 Injection pump Pressure into the injection pump must be positive in all operating conditions. The injection pump is operated by means of the pump control gear and sends high-pressure fuel to the injectors. NOTE: In the event of leakage from the high pressure circuit do not intervene with the engine running, but turn it off and wait 5 - 10 minutes before checking the leak. Tab 2.14 POS.; COMPONENTS DESCRIPTION. 1; Accelerator lever. 2; Min Adjustment. 3; Max Adjustment. 4; Torque adjustment. 5; High pressure delivery to injectors. 6; Return to fuel tank. 7; Inlet suction fuel. 8; Cold starting device. 9; Gasket. 10; Shaft. 11; Advance settings (locked). 12; Pump identification label. 13; Air bleeding screw. 14; Pump control shaft blocking device.. | Fig 2.6 Fig 2.7 |
| --- | --- |

| 2.9.4 Injector It is a device used to introduce fuel, in the form of one or more jets that are adequately pulverised and suitably oriented directly into the combustion chamber. They consist of a metallic body that internally provides a mobile element that acts on the needle: this, rising against the action of a calibrated spring, allows the release of fuel under high pressure. Opening pressure: 260-268 bar (3770-3886 PSI) Important - The injectors are calibrated individually. - Fuel contamination causes serious damage to the injection system. Tab 2.15 POS.; COMPONENTS DESCRIPTION. 1; Inlet fuel. 2; Gasket. 3; Gasket. 4; Nozzle. 5; Hole for fuel return to fuel tank. | Fig 2.8 |
| --- | --- |

| 2.9.5 Fuel filter The fuel filter is situated on the crankcase of the engine or it may be assembled on the frame of the vehicle. Tab 2.16 POS.; COMPONENTS DESCRIPTION. 1; Fuel filter support cartridge. 2; Air bleeding screw. 3; Cartridge. 4; Water in fuel sensor. 5; Hole water drainage. Tab 2.17 Cartridge characteristics DESCRIPTION; VALUE. Filtering surface; 2.300 cm 2. Degree of filtration; 5 µm. Max operating pressure; 2.0 Bar. | Fig 2.9 |
| --- | --- |

| 2.9.6 Electric fuel pump (optional) When the electric fuel pump is installed in a diesel engine, one must: - Remove any filters installed on the inlet of the electric fuel pump; - Insert a pre-filter between the tank and the electric pump; - The electric pump must be installed on the application at a height from the minimum tank level such as to generate a MAX . pressure drop equal to a column of 500 mm of fuel; - Insert a shut-off valve to prevent dry operation due to the emptying of the intake manifold; - The electric pump must guarantee a supply pressure at the inlet positive in all conditions. Tab 2.18 POS.; DESCRIPTION. 1; Fuel tank. 2; Arrival pipe from the tank. 3; Prefilter. 4; Flow pipe from pre-filter to electric pump. 5; Electric pump. 6; Flow pipe to the fuel filter. 7; Fuel filter. | Fig 2.10 |
| --- | --- |
| 2.9.7 Guards for fuel injection circuit components High-pressure injection circuit components are particularly sensitive to impurities. To prevent impurities, even microscopic ones, from accessing the fuel input or output unions, you are required to close these accesses by means of specific caps as soon as the various tubes are disassembled and disconnected. Disassembly of any component of the injection circuit must not occur in dusty environments. Cap guards must remain closed in their housing (ST_40) until the moment they are to be used. Pay special attention when using the caps and avoid any contamination of dust or dirt of any kind. Even after using the caps illustrated in this paragraph, all components of the injection circuit must be placed with care in environments that are free of any type of impurity. Fig. 2.11 and 2.12 illustrate the caps that must be used on components of the injection circuit. Cap guards must be accurately washed after use and placed back in their housing (ST_40). Important - It is highly recommended to have this page visible during disassembly operations of the components of the fuel injection circuit. | Fig 2.11 Fig 2.12 |

## Lubrication circuit

| 2.10.1 Lubrication circuit diagram The oil pump is driven by the crankshaft on the timing system side. On the parts of the systems shown in green on In the parts in green, the oil is in intake, in the parts in red, the oil is under pressure and in those in yellow the oil is returning towards the oil sump 2 (not under pressure). Tab 2.19 COLOUR; DESCRIPTION. Oil in intake. Oil under pressure. Oil returning to the oil sump. Tab 2.20 POS.; DESCRIPTION. 1; Oil pump rotors. 2; Oil sump. 3; Crankshaft. 4; Camshaft. 5; Rocker arm pin. 6; Hydraulic tappets. 7; Rocker arm cover. 8; Cylinder head. 9; Upper crankcase. 10; Lower crankcase. 11; Oil filter. 12; PTO 3rd/4th gear housing. | Fig 2.13 Fig 2.14 |
| --- | --- |
| NOTE : Click by side to play the procedure. | https://www.youtube.com/embed/5HuLfSgqz6s?rel=0 |
| 2.10.2 Oil pump The oil pump rotors are trochoidal (with lobes) and are activated from the crankshaft by means of the key. The pump body is situated inside the distribution guard. It is imperative to assemble the rotors with reference A visible by the operator. Tab 2.21 POS.; DESCRIPTION. 1; Internal rotor. 2; External rotor. 3; Oil pump crankcase. 4; Pump control key. 5; Timing system crankcase. 6; Crankshaft. | Fig 2.15 |

| 2.10.3 Oil filter Fig 2.16 |  |
| --- | --- |
| Tab 2.22 DESCRIPTION; VALUE. Filtering surface; 2.000 cm 2. Degree of filtration; 15 µm. Max operating pressure; 7.0 Bar. | Tab 2.23 POS.; DESCRIPTION. 1; Oil cartridge. 2; Filter element. 3; Gasket. 4; Safety valve. 5; Upper cranckcase. 6; Inlet oil. 7; Oil filtering. 8; Output oil (sent to the circuit). |

## Coolant circuit

2.11.1 Coolant circuit diagram

| Tab 2.24 POS.; DESCRIPTION. 1; Coolant pump. 2; Coolant intake. 3; Coolant, cylinder. 4; Coolant, cylinder head. 5; Coolant to radiator. 6; Coolant into radiator. 7; Vent line from radiator (to 8). 8; Compensation tank. 9; Thermostatic valve. 10; Return from compensation tank. | Fig 2.17 |
| --- | --- |
| 2.11.2 Coolant pump Tab 2.25 POS.; DESCRIPTION. 1; Coolant pump control pulley. 2; Coolant intake fitting. | Fig 2.18 |
| 2.11.3 Thermostatic valve Tab 2.26 POS.; DESCRIPTION. 1; Cylinder head. 2; Coolant outlet cover. 3; Thermostatic valve. 4; Gaskets. 5; Air bleeding hole. Opening temperature +79° ± 2°C. | Fig 2.19 |
| 2.11.4 Radiator (optional) NOTE: Component not necessarily supplied by Kohler. Tab 2.27 POS.; DESCRIPTION. 1; Radiator group. 2; Coolant refill cap. 3; Vent tube or excess coolant return. 4; Coolant flow manifold. 5; Coolant intake manifold. 6; Fan. 7; Protective grid. | Fig 2.20 Fig 2.21 |

## Intake and exhaust circuit

Air in intake; Gas in exhaust. Fig 2.22 Fig 2.23

| Important - The air temperature inside the intake manifold must never exceed that of the environment by 10°C. Clean air is sucked by means of an intake manifold and via ducts in the cylinder head, enters the cylinders. Compressed air inside the cylinders and mixed with the fuel transforms into Gas after combustion. Gas is expelled from the cylinders and sent to the exhaust manifold, which expels the gas towards the exhaust muffler. | Tab 2.28 POS.; DESCRIPTION. 1; Air in air filter. 2; Air in intake manifold flow. 3; Air in head intake. 4; Air in cylinder intake. 5; Gas in cylinder outlet. 6; Gas in head outlet. 7; Outlet gas to muffler. A; Inlet manifold. B; Exhaust manifold. C; Crankcase. D; Exhaust muffler (optional). |
| --- | --- |
| 2.12.1 Air filter (optional) NOTE: Component not necessarily supplied by KOHLER. Important - The air filter is a dry-type one, with a replaceable paper filter cartridge - The filter intake must be positioned in a cool area. - The temperature of the aspirated air must never exceed ambient temperature by more than 10°C (if you are using a pipe, its length must not exceed 400 mm and it must be as straight as possible). Fig 2.24 | Tab 2.29 POS.; DESCRIPTION. H; Air filter cartridge. M; Filter cover. N; Filter support. Q; Dust exhaust valve. R; Dust exhaust valve. |

## Electric system

2.13.1 Engine electrical wiring (opzional) NOTE : Component not necessarily supplied by KOHLER . Electrical wiring is supplied upon request, it interfaces with the panel by means of 19-way Deutsch connectors (female on engine panel - male on accessories panel). The connectors are described in Tab. 2.30. Fig 2.25; Tab 2.30 REF.; DESCRIPTION. 1; Engine panel connector interface (Fig. 2.26a). 2; Accessories panel connector interface (Fig. 2.26b). 3; Electrical fuel pump connector. 4; Cold Start Advance connector (on injection pump - Fig. 2.39 ). 5; Fuse connector. 6; Electro-Stop connector (on injection pump). 7; "L" alternator connector (Iskra). 8; "W" alternator connectors (Iskra). 9; Alternator connector without "W" (Chengdu). 10; Alternator connector with "W" (Chengdu). 11; Coolant temperature sensor connector. 12; Coolant temperature sensor connector. 13; Starter motor connector "+ 50". 14; Starter motor connector "+ 30". 15; Air cleaner clogging sensor connector. 16; Earth connector..

| 2.13.1.1 Connector panel on the engine/machine The connector is a female 19-way Deutsch type. There is a list of all PIN connections in Tab. 2.31. Fig 2.26a | Tab. 2.31 PIN.; INLET SIGNALS TO THE PANEL. 1; Oil pressure switch. 2; Alternator indicator light. 3; Coolant temperature warning light. 4; Air cleaner clogging warning light. 7; Air cleaner clogging warning light. 9; Electro-Stop. 13; Alternator (W). 14; Starter motor (+ 30). 15; Inlet indicator general alarm. PIN.; OUTLET SIGNALS FROM THE PANEL. 5; Earth. 6; IG excitation alternator (+15 wrench). 8; Starter motor (+ 50). 10; Grid heater (Relay). 11; Electric pump. 18; Injection pump (Cold Start Advance - Fig. 2.39 ). |
| --- | --- |

| 2.13.1.2 Accessories panel connector The connector is a male 19-way Deutsch type. There is a list of all PIN connections in Tab. 2.32. Fig 2.26b | Tab. 2.32 PIN.; INLET SIGNALS TO THE PANEL. 2; Fuel filter (water detection sensor). 4; Radiator (coolant level sensor). 7; Outlet indicator general alarm. 9; External Stop. 15; Inlet indicator general alarm. 1; Fuel tank (fuel level sensor). PIN.; OUTLET SIGNALS FROM THE PANEL. 5; Earth. 6; Relay with 5A fuse (+ 15 wrench). 10; Grid heater (Relay). 13; Alternator (W). 17; Coolant temperature warning light. |
| --- | --- |

| 2.13.3.1 Wiring disconnection Some sensor connectors and electronic control devices are sealed. This tipe of connectors must be disconnected by means of pressure on tabs A or unblock the retainers B , as illustrated from Fig. 2.26c to Fig. 2.26g. | Fig 2.26c |
| --- | --- |
| Fig 2.26d | Fig 2.26e |
| Fig 2.26f | Fig 2.26g |

## Sensors and switches

| 2.14.1 Fuel filter water detection sensor (optional) The water presence sensor in the fuel filter serves to indicate the presence of water in the fuel. The sensor closes the electrical circuit and the warnin lamp in the panel board switches on the dashboard of the car on which the motor is mounted. Water, if present in the fuel, because of its greater specific weight separates and settles in the lower part of the filter where there is a drain plug. Gently loosen the water drain plug without removing it and spill out the water if present. Re-tighten the water drain plug H as soon as the fuel spills. | Fig 2.27 |
| --- | --- |
| 2.14.2 Oil pressure switch Oil pressure switch N is assembled on the crankcase. It is a N/C sensor, calibrated at 0.6 bar ± 0.1 bar. With oil low pressure the sensor closes the electrical circuit and the warning lamp in the panel board switches on. | Fig 2.28 |

| 2.14.3 Coolant temperature sensor connector The sensor has the dual function of a thermometer and thermal contact. The coolant/thermal contact P temperature probe is applied to the cylinder head on the side of the thermostatic valve. Sensor P1 or P2 (Fig. 2.29) can be assembled on the engine: P1 Characteristics indicated in Tab. 2.33A (blue connector). Thermal contact N/O with closing temperature at +110 °C ±3°C, re-opening +88 °C / +100 °C. P2 Characteristics indicated in Tab. 2.33B (white connector). Thermal contact N/O with closing temperature at +110 °C ±3°C, re-opening +88 °C / +100 °C. NOTE : R indicates the pin where it is possible to measure electrical resistance. Tab 2.33A SENSOR P1 CHARACTERISTICS. Temperatura °C; R min Ω; R max Ω. -35; 53.983; 73.806. -30; 39.229; 52.941. -15; 18.006; 20.825. 0; 7.095; 8.929. 30; 1.717; 2.039. 60; 0.520; 0.589. 90; 0.188; 0.204. 120; 0.076; 0.084. Tab 2.33B SENSOR P2 CHARACTERISTICS. Temperatura °C; R min Ω; R max Ω. -36; 11.835; 15.724. -30; 8.258; 10.834. -16; 3.721; 4.753. 0; 1.611; 2.003. 30; 414,1; 493. 60; 132; 151,7. 90; 50,27; 56,11. 120; 21,6; 24,29. | Fig 2.29 |
| --- | --- |
| 2.14.4 Air cleaner clogging switch NOTE: Component not necessarily supplied by KOHLER . The switch is assembled on the air cleaner. When the filter is clogged, it sends a signal to the panel. Features: - Operating temperature: -30 °C / +100°C - Contact usually open. - Contact closed by vacuum: -50 mbar. | Fig 2.30 |

## Electrical components

| 2.15.1 Alternator (A) Externally controlled by the crankshaft by means of a belt. - Ampere 55 A - Volt 12V | Fig 2.31 |
| --- | --- |
| 2.15.2 Starter Motor (C) - Type Bosch 12 V - Potenza 2 kW - Anticlockwise rotation (seen from timing system side) | Fig 2.32 |
| 2.15.3 Cold starting device (Heater) The cold starting device consists of a resistance, managed by the ECU, which is activated when the ambient temperature is ≤ -16° C. The intake air is heated through the resistor and facilitates starting the engine. Characteristics: - Type Hidria AET 12 V - Power 550 W | Fig 2.33 |
| 2.15.4 Electric pump (optional) NOTE: Component not necessarily supplied by KOHLER. The electric pump is located before the fuel filter. One of the following pumps can be assembled A1 - A2 - A3 - A4. Tab. 2.34 a-b-c-d indicates the pumps' features. Tab 2.34 POS.; DESCRIPTION. 1; Electrical connection. 2; Prefilter pump. IN; Ingoing fitting ( IN ) from tank. OUT; Outgoing fitting ( OUT ) to fuel filter. Tab 2.34a A1; VALUE. Voltage; 12 - 24 V. Delivery; 100 L/h @ 0.44 - 0.56 bar. Tab 2.34b A2; VALUE. Voltage; 12 V. Delivery; 60.56 L/h @ 0.41 bar. Tab 2.34c A3; VALUE. Voltage; 12 V. Delivery; 24 L/h @ 0.1 bar. Tab 2.34d A4; VALUE. Voltage; 12 V. Delivery; 30 L/h @ 0.4 bar. | Fig 2.34 Fig 2.35 Fig 2.36 Fig 2.37 Fig 2.38 |
| 2.15.5 Cold start advance The Cold Start Advance E device is part of injection pump D ; it provides for advance injection modification to enable advance of the engine at low temperatures. The device is controlled by the ECU H . | Fig 2.39 |
| 2.15.6 Electro-Stop The electro-stop F device is part of injection pump D ; it turns off the engine by blocking the flow of fuel into pump D . |  |
| 2.15.7 ECU starting The H device assists cold engine ignition controlling the "cold starting device" ( Heater ) and the "Cold Start Advance" ( CSA ). Tab. 2.35a indicates the activation times based on the ambient temperatur. Tab. 2.35a °C; HEATER; CSA. ≤ 20 ÷ -15; 0''; 120''. - 16; 16''. - 21; 21''. -26; 26''. ≤ -32; 32''. Tab. 2.35b POS.; DESCRIPTION. 1; Heater. 2; 50 - ignition. 3; 15 - ignition. 4; CSA. 5; 30 - battery. 6; .... 7; Earth. 8; Control panel indicator. | Fig 2.40 |
| 2.15.8 Fuse Device G is assembled on cylinder head P (flywheel side); it protects the electrical circuit in the event of an overload or short circuit. NOTE: Component not necessarily supplied by KOHLER. | Fig 2.41 |
| 2.15.9 Control panel (optional) Panel L can be assembled on the engine or machine. The connectors are described in Tab. 2.36 , the main functions are illustrated. NOTE: Component not necessarily supplied by KOHLER. Tab 2.36 POS.; DESCRIPTION. M; Hour-meter indicator. S; Control switch to start the engine. W1; Panel ignition indicator. W2; Warning Light - battery not charging. W3; Warning Light - engine oil not pressurised. W4; Warning Light - high coolant temperature. W5; Warning Light - alarm general indicator. | Fig 2.42 |

## Timing system and tappets

| The timing system is equipped with hydraulic tappets that automatically recover the operation of the rocker rods assembly. No registration is therefore required. 2.16.1 Components identification Fig 2.43 |  |
| --- | --- |
| Tab 2.37 POS.; DESCRIPTION. 1; Crankshaft. 2; Camshaft. 3; Camshaft tappets. 4; Rocker arm control rod. 5; Rocker arms. 6; Valves. 7; Injection pump control gear. 8; Camshaft control gear. 9; Intermediate gear. 10; Intermediate gear pin. 11; Crankshaft gear. 12; Valve control bridge. 13; Articulation control valves. 14; Hydraulic tappets. | Fig 2.44 Fig 2.45 |
| 2.16.2 Rocker arm pin Tab 2.39 POS.; DESCRIPTION. 1; Rocker arm pin. 2; Rocker arm distancing spring. 3; Rocker arm pin support. 4; Exhaust rocker arm. 5; Intake rocker arm. | Fig 2.47 |
| 2.16.3 Rocker arms Tab 2.40 POS.; DESCRIPTION. 1; Rocker arm body. 2; Hydraulic tappet oil refill line. 3; Valve tappet lubrication line. 4; Valve tappet. 5; Hydraulic tappet. 6; Oil flow line. | Fig 2.48 |
| 2.16.4 Hydraulic tappets Tab 2.41 POS.; DESCRIPTION. A; Hydraulic tappets. B; Hight pressure chamber. 1; Hydraulic tappets oil refill pipe. 2; Retaining ring. 3; Piston. 4; Unidirectional valve. 5; Tappet body. 2.16.4.1 Hydraulic tappet operation The operating principle of the hydraulic tappet is based on the incompressibility of the liquids and on controlled leakage. The oil under pressure enters the tappet chamber A , providing a constant supply of oil in the low-pressure chamber. Through the non-return valve, 4 the oil can only access the high-pressure chamber B and exit via the clearance between the piston 3 and the tappet body 5 (controlled leakage). The chamber B is filled when the rocker arm is on the base radius of the cam and the spring 6 keeps the piston 3 against the valve stem, thus eliminating any system play. Thanks to the spring extension, the tappet "extends", creating a small depression in the chamber B , making the non-return valve 4 open, and allowing the oil in the chamber A to pass to chamber B , restoring the proper amount of oil required to eliminate any play in the valves. | Fig 2.49 |

2.16.4.2 Difficult operating conditions For proper operation on the hydraulic tappets it is essentia that the low pressure chamber of the piston 3 is always full of oil. In some conditions this may not occur (due to the fact that the oil leaks away when the engine is switched off, which can also partially drain the tappets). This situation will be the cause of clearances that will result in a characteristic noise similar toa ticking sound. - When the engine is cold, the tappet filling time could be very long if the oil used is not suitable for the specific environmental conditions ( Tab. 2.2 ) - If the engine is very hot: at idle speed, oil pressure may be low, and small air bubbles could form in the circuit. Because of this, this compressing the tappet slightly and producing valve play which is responsible for the ticking sound. On account of this, the tappet compresses slightly giving rise to a valve clearance, thus generating a slight ticking sound, which however disappears rapidly ( MAX 10 seconds) once normal operating conditions have been restored. Anyway the duration of ticking Anyway the duration of ticking sound must be MAX 30 seconds. If not, the problem is surely due to the poor quality of the oil, wear or impurities that, transported by the oil, can infiltrate between the ball valve and its seat inside the piston, compromising the operation of the tappet itself; In these cases, the only solution is to replace the oil or hydraulic tappets. The prolonged persistence of the ticking sound or abnormal noise must be investigating in order to prevent any malfunctions; if necessary, replace the hydraulic tappets and engine oil.

## Components handling

| 2.17.1 Injection pump - Only handle by means of the points marked by Y . - It is forbidden to handle using the points marked by N . | Fig 2.50 |
| --- | --- |
| 2.17.2 Injector - Only handle by means of the points marked by Y . - It is forbidden to handle using the points marked by N . | Fig 2.51 |

## Balancer device (optional)

| The balancer device is composed of a special crankshaft that activates 2 additional shafts (balancers). Rotation of the balancers, which have counterweights that oppose the movement of alternating weights (crankshaft - connecting rods - pistons), reduces vibrations caused by them. The device is developed under the crankshaft, fixed on the crankcase, closed by the oil sump. Tab 2.43 POS.; DESCRIPTION. 1; Crankshaft. 2; Balancer shaft control gear. 3; Balancer shaft support box. 4; Conductor balance shaft. 5; Conducted balance shaft. | Fig 2.67 |
| --- | --- |