General information

KDI-TCP 3404E5 Workshop Manual (Rev. 01.1)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# General information

## Useful information

- This manual contains the instructions needed to carry out  proper use and maintenance of the engine, therefore it must always be available, for future reference when required.

- Information, descriptions and pictures contained in this manual reflect the basic configuration of the engines ( Par. 1.4 and Par. 1.5 ).

- However, the development of engines is continuous. Therefore, the information in this manual is subject to change without notice.

- KOHLER reserves the right to make, at any time, changes on the engines for technical or commercial reasons.

- These changes do not require KOHLER to intervene on the production marketed up to that time and nor to consider this manual as inappropriate.

- The paragraphs, tables and figures are numbered by chapter and followed by the progressive paragraph, table and/or figure number.

Es: Par. 1.3 - chapter 1 paragraph 3 .
Tab. 2.4 - chapter 2 table 4 .
Fig. 4.5 - chapter 4 figure 5 .

NOTE: The paragraphs may contain sub-paragraphs.

- All technical terms, specific components and symbols ( Tab. 15.1 ) that are in the manual are listed and described inside the glossary, which can be consulted in ( Chap. 15 ).

- The references of the objects described in the text and in the figure are indicated by letters and numbers, which are always and only related to the paragraph you are reading unless there are specific references to other figures or paragraphs.

- Reference to values are indicated by letters or numbers.

- Other important references are highlighted in red.

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- Any additional section that KOHLER will deem necessary to supply at a later stage must be kept with the manual and considered as an integral part of it.

- The information contained in this manual is the sole property of KOHLER , therefore no partial or total reproduction or replication is allowed without the express permission of KOHLER .

1.1.1 Useful Information - accident prevention - environmental impact

- Before proceeding repair - handling the motor , read the entire Chap. 3 , which contains important information about the procedures to be followed for safety and environment .

## Manufacturer and engine identification

1.1 Pos.; Description. 1; Production place. 2; Manufacturer identification. 3; Engine model. 4; Engine serial number. 5; Month and year of production. 6; Approval data. 7; Engine version. 8; Data matrix.

## Homologation labels

1.3.1 Label for EPA rules (compilation example) 1.1 POS.; DESCRIPTION. 1; Model year in compliance with the rules. 2; Power category (kW). 3; Engine displacement (L). 4; Particulate emission limit (g/kWh). 5; Engine family ID. 6; Emission Control System = ECS. 7; Fuel with low sulphur content. 8; Injection timing. 9; Electronic injector opening pressure (bar). 10; Production date (example: 2013.JAN). 1.3.2 Label for China Standards (compilation example) 1.2 POS.; DESCRIPTION. 1; Manufacturer. 2; Engine model. 3; Manufactoring date. 4; Certificate N°. 5; Power range (kW). 6; Emission level. 7; Rated power. 8; Aftertreat system. 1.3.3 Label for Korea Standards (compilation example) 1.3 POS.; DESCRIPTION. 1; Tier 4 Final. 2; Engine model. 3; Manufactoring date and manufacturer code. 4; N° Korea emission certificate.

## Identification of the main internal components of the engine and operating reference (BASE CONFIGURATION)

WIEW OF EXHAUST SIDE

Fig 1.5

| The following chapters contain operating references in order to clearly understand the engine. This paragraph illustrates these references that may be recognised by means of some main internal components. Should you need to execute complex operations, always consult this paragraph. NOTE: it is advisable to keepthis page visible during disassembly and assembly operations. | Tab 1.2 REF.; DESCRIPTION. A; View of timing system side (2 nd PTO). B; View of flywheel side (1 nd PTO). C; View of exhaust side. D; View of intake side. 1; Cylinder/Piston N. 1. 2; Cylinder/Piston N. 2. 3; Cylinder/Piston N. 3. 4; Cylinder/Piston N. 4. POS.; DESCRIPTION. 5; Crankshaft pulley (2 nd PTO). 6; Gear timing system. 7; Thermostatic valve. 8; Oil pump. 9; Oil suction hose. 10; Crankshaft. 11; Exhaust manifold. 12; Intake manifold. 13; Camshaft. 14; Gears adaptor for 3 th /4 th PTO (optional). 15; Flywheel (1 st PTO). 16; Gears adaptor for 4th PTO (optional). 17; Balancer shafts. |
| --- | --- |

WIEW OF FLYWHEEL SIDE

Fig 1.6

## Engine component identification

NOTE: Some components are for illustrative purposes only and may vary or they are not supplied by REHLKO.

POS.; DESCRIPTION. 1; ECU. 2; Air filter. 3; ETB. 4; Oil filler cap. 5; High-pressure fuel injection pump. 6; Lub. oil filter. 6a; Oil cooler. 7; Fuel filter. 8; Oil sump. 9; Oil drain plug. 10; Oil dipstick. 11; Radiator/Intercooler. 12; Coolant filler cap. 13; ATS. 14; Turbocharger. 15; Alternator. 16; Starter motor. 17; Oil steam separator. 18; Flywheel. 19; Coolant pump. 20; Thermostatic valve. 21; Coolant temperature sensor. 22; Electronic injector.

## Cooling circuit

NOTE: Some components are for illustrative purposes only and may vary or they are not supplied by REHLKO .

POS.; DESCRIPTION. 1; Radiator. 2; Intake sleeve. 3; Return tube to radiator. 4; Intake return tube. 5; Circuit for DEF heating. 6; DEF tank. 7; Coolant delivery tube in DEF tank. 8; Coolant delivery tube to DEF injector. 9; Solenoid valve. 10; Refrigerant delivery pipe to DEF heating solenoid valve. 11; Coolant to radiator return sleeve. 12; Coolant pump. 13; DEF Dosing Control Unit (DCU). 14; DEF Injector. 15; ATS. 16; Coolant delivery pump to DEF injector (provided if needed).

## DEF circuit

NOTE: Some components are for illustrative purposes only and may vary or they are not supplied by REHLKO .

POS.; DESCRIPTION. 1; DEF Dosing Control Unit (DCU). 2; DEF tank. 3; DEF pump. 4; DEF intake tube. 5; DEF delivery tube to injector. 6; Return tube in DEF tank. 7; DEF injector.

## Intake and exhaust circuit

NOTE: Some components are for illustrative purposes only and may vary or they are not supplied by REHLKO .

POS.; DESCRIPTION. 1; Air filter. 2; Air intake sleeve. 3; Turbocharger. 4; Air delivery tube to Intercooler. 5; Intercooler. 6; Air delivery tube to intake manifold. 7; ETB. 8; Exhaust gas delivery hose to ATS. 9; DEF injector. 10; ATS.

## ATS (After Treatment System)

| ATS with DOC+DPF+SCR filter NOTE: the ATS system with DPF filter is only present for versions compliant with "Stage V" emission regulations. The ATS system can be installed in a different way than shown. | Tab 1.5 POS.; DESCRIZIONE. 1; Turbocharger. 2; Turbine exhaust pipe. 3; DOC. 4; DPF. 5; SCR. 6; ETB. |
| --- | --- |