Fluids filling information

KDI-TCP 3404E5 Workshop Manual (Rev. 01.1)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Fluids filling information

## Engine oil

| Warning - Before proceeding with operation, carefully read Par. 3.3.2 . |  |
| --- | --- |
| - Loosen the oil filler cap A . - Add the type and amount of oil recommended ( Tab. 2.2 ). - Remove the oil dipstick B and check that the level is up to but does not exceed the MAX . Important - Do not use the engine with the level of oil below MIN or above MAX - If the oil level is not at MAX , insert more oil until the MAX level is reached as indicated on the dipstick. - Re-tighten the cap A . | Fig 10.1 |
|  | Fig 10.2 |
| NOTE : Click by side to play the procedure. | https://www.youtube.com/embed/HWCzK41Br1U?showinfo=0&rel=0 |

## Coolant

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Close the exhaust valve B . | Fig 10.3 |
| - Refill the radiator with coolant (refer to Par. 2.6 for the liquid specifications). - Top liquid up until the pipes inside the radiator are covered by about 5 mm. - For engines equipped with separate expansion tank, pour in fluid until reaching the MAX level mark. - Start the engine without the radiator cap A or the expansion tank cap. | Fig 10.4 |
| - Keep it running at idle speed until the cooling liquid level goes down and becomes steady (the waiting times varies according to the ambient temperature). - Stop the engine and allow it to cool. - If there is an expansion tank ( C ) top liquid up to the mark MAX . - Without expansion tank top liquid up until the pipes inside the radiator are covered by 5 mm. Do not overfill the radiator, but leave room for the coolant to expand. - Tighten the radiator cap A or the expansion tank cap. Warning - Before starting make sure that the radiator cap and expansion tank cap, if present, are installed correctly to avoid spillage of liquid or vapour at high temperatures. - After a few hours of operation stop the engine and allow it to cool. Check and top up the coolant liquid. | Fig 10.5 |