Information on adjustments

KDI 2504TCR / KDI 2504TCRE5 Workshop Manual (Rev. 17.8)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information on adjustments

## 'Waste Gate' opening valve regulation

Important

- Before proceeding with operation, read Par. 3.3.2 .

- See service letter 700032 .

- Regulation must not be carried out with the engine running.

- During the procedure in point 5 , pay special attention not to bend rod H .

- Disconnect the hose A from the turbocharger, and connect a pressure gauge B (scale from 0 to 5 bar).

- Connect the gauge B to the network of compressed air, interposing a pressure reducer C .

- Position dial gauge D in such a way that feeler F rests onthe Waste Gate rod control valve extremity H (point E ).

- By using gradually the reduction gear C send the air to the Waste Gate actuator control L in order to move rod H forward by 1 mm (value M to check on dial gauge D). Pressure read on gauge B must be: 1350 mbar for engine model KDI 2504 TCR and 1250 mbar for engine model KDI 1903 TCR.

- If pressure is less or more than the indicated value, proceed as follows: - Undo lock nut G from rod H.

- Remove the retainer cotter pin (point E ) and disconnect rod H from the Waste Gate control lever.
- Tighten (to increase) / or loosen (to decrease) pressure of the ring nut of rod H until reaching the corrected calibration.
- Redo lock nut G .
- Reconnect rod H and assemble the cotter pin point E .

Fig 12.1

## Air filter check

| Important - Before proceeding with operation, read Par. 3.3.2 . - When the cartridge G is dirty, do not clean it but replace cartridges B and G . |  |
| --- | --- |
| - All manifolds connected to the turbo must be fully clean and not damaged. - Clean the inside components A and D with a damp cloth. - Do not use compressed air , repeatedly tap the front side E on a flat surface. | Fig 12.2 |

## Oil steam separator check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Loosen clamp B and remove hose C from hose D . - Start the engine at idle speed or without a load; check if air comes out of hose C . NOTE: If what is described in Point 2 does not occur, proceed with cleaning or replacing oil separator A and accurately clean support flange F , all connecting hoses, and repeat the operation from P oint 2. | Fig 12.3 |

## Rubber hoses and manifolds check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| The check is carried out by applying slight deflection or bending along the tube/hose and next to the hose clamps. Components must be replaced if they have clear signs of cracks, tears, cuts, leaks, or do not retain a certain degree of elasticity. - Check the condition of all rubber hoses A . - Check whether there are any leakages of air, refrigerant, oil or fuel next to their connections. | Fig 12.4 Fig 12.5 |

## Oil leak check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| Check that there are no leakages next to area A . - Start the engine at idle speed or without a load and check whether there are any leakages next to area A. - It is anyhow necessary to also check the seals of all main components and their surface contact, such as: - crankcase and gasket (side 1 a PTO) - oil sump and exhaust caps - cylinder head and its assembled components - rocker arm cover - Timing system carter and gasket (side 2 a PTO) - oil dipstick housing or rod support tube. NOTE: Perform the checks described in Points 1 and 2 periodically and during maintenance procedures. It is also necessary to check for leakages on the components that are not listed. If necessary, disassemble the components that have a leakage and investigate the possible cause. The components must be replaced otherwise they do notguarantee their sealing. | Fig 12.6 Fig 12.7 |

## Oil pressure check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Insert a thermocouple instead of the oil dipstick A . - Unscrew and remove the oil pressure switch and screw on a 10 bar pressure gauge in its seat (Fig. 12.10) . - Start the engine at idle speed and without a load, check the oil pressure value according to the oil temperature (Fig. 12.9 ). NOTE : The graph in Fig. 12.9 illustrates the pressure line with speed of 1000 Rpm. - If the pressure values are below the values indicated in Fig. 12.9 , check to identify the cause of the problem. Fig. 12.9 | Fig. 12.8 Fig. 12.10 |