Information for disassembly

KDI 1903TCR / KDI 1903TCRE5 Workshop manual (Rev. 17.7)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information for disassembly

## Recommendations for disassembly

Important

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- The operator should prepare all equipment and tools in order to enable him to carry out the operations correctly and safely.

- Before disassembly, perform the operation described in Chap. 5 .

- Before proceeding with operation, carefully read Chap. 3 .

- In order to operate safely and easily, we recommend positioning the engine on a rotating stand for engine overhauling.

- Seal all injection component unions as illustrated in Par. 2.9.8 during assembly.

- Protect all disassembled components and coupling surfaces subject to oxidation with lubricant.

- Where necessary, reference to special tools to use during disassembly operations is indicated (es. ST_05 ), identified in Tab. 13.1 - 13.2 - 13.3 .

## EGR circuit disassembly

| 7.2.1 EGR cooler unit - Undo the screws A ( ST_06 ) and B and remove the pipe C with the relative gaskets. | Fig 7.1 |
| --- | --- |
| - Undo the screws D, E ( ST_06 ) and remove pipe F and the relevant gaskets. | Fig 7.2 |
| - Release the clamp M and remove pipe N . - Release the clamp L . - Undo capscrews G and remove EGR Cooler H from hose X ( ST_05 ) . | Fig 7.3 |
| 7.2.2 EGR Valve - Disconnect the connector P . - Undo the screws Q and remove the EGR valve R with the relevant gasket. NOTE: The EGR valve is not a serviceable item, and if damaged/worn, it should be replaced with a new one. | Fig 7.4 |
| - Undo capscrews S and remove flange T with the relative gasket. | Fig 7.5 |

## Electric components disassembly

| 7.3.1 Electric wiring Important - Refer to Par. 2.13 prior to proceeding with disassembly. - Disconnect the connectors A, B and C . - Release the clamp D . | Fig 7.6 |
| --- | --- |
| - Disconnect the connectors E, F and G . - Release the clamps J and H . | Fig 7.7 |
| - Disconnect the connectors K and L . | Fig 7.8 |
| - Disconnect the connectors M and P . - Release the clamp Q . | Fig 7.9 |
| - Disconnect the connectors R . - Undo the screws S and remove the wiring support T ( ST_06 ) . | Fig 7.10 |
| 7.3.2 Starter motor Important - The motor is not repairable. - Undo the screws U and remove the starter motor V . | Fig 7.11 |
| 7.3.3 Belt and alternator - Loosen the screws Z and W . - Push the alternator AA in the direction of the arrow AB . - Remove the belt AC from the pulleys AR . - Undo the screws Z and W and remove the alternator AA . Important - The belt must always be replaced every time it is disassembled, even if it has not reached the scheduled hours for replacement. | Fig 7.12 |
| 7.3.4 Sensors and switches Important - After disassembly, protect the sensors suitably against knocks, dampness and any high temperature sources. - The sensors and switches cannot be repaired, therefore they must be replaced in the event of anomalies. 7.3.4.1 Oil pressure switch ( ) - Unscrew and remove the oil pressure switch AD . | Fig 7.13 |
| 7.3.4.2 Coolant temperature sensor ( ) - Unscrew and remove the coolant temperature sensor AE . | Fig 7.14 |
| 7.3.4.3 Speed sensor ( ) - Undo the screw AF and remove the sensor AG with the relative spacer ( ST_06 ) . - Undo the screws AH and remove the sensor ( ST_06 ) . | Fig 7.15 |
| 7.3.4.4 Camshaft phase sensor - Undo the screw AM and remove the sensor AN with the relative spacer ( ST_06 ). | Fig 7.16 |
| 7.3.4.5 T-MAP Sensor ( ) - Undo the screw AP and remove the sensor AQ ( ST_06 ). | Fig 7.17 |
| 7.3.4.6 Fuel filter water detection sensor ( ) Warning - The fuel filter is not always mounted on the engine. - When disassembling the sensor A , use a suitable container to recover the fuel contained in the cartridge B . - Unscrew the sensor A from the cartridge B . | Fig 7.18 |

## Turbocharger disassembly

| - Release the clamps A and B and remove the manifold C . | Fig 7.19 |
| --- | --- |
| - Unscrew the fittings D and remove the pipe E with the relative gaskets G . - Undo the screws F . - Release clamp M . | Fig 7.20 |
| - Undo the nuts L . - Remove tube N . | Fig 7.21 |

## Exhaust manifold disassembly

| - Undo the nuts A and remove the manifold B and the gaskets C . - Close the openings and manifolds to prevent foreign bodies from entering. | Fig 7.22 |
| --- | --- |

## Coolant recirculation components disassembly

| 7.6.1 Oil Cooler manifold - Release the clamps Z . - Undo the screw W and remove hoses J ( ST_05 ) . | Fig 7.23 |
| --- | --- |
| - Release the clamp K and remove hoses AA . | Fig 7.24 |
| 7.6.2 Coolant pump Important - The pump B is not repairable. - Undo the screws A and remove the water pump unit B with its gasket C . | Fig 7.25 |
| 7.6.3 Thermostatic valve - Undo the screws A and remove the thermostatic valve cover B . - Remove the thermostatic valve C and its gasket. Important - Always replace the gasket D every time it is disassembled. - Check that the air bleeding hole is not clogged or blocked ( Par. 2.11.4 ). | Fig 7.26 |

## Crankshaft and target wheel pulley disassembly

| - Position the crankshaft with the 1st cylinder at TDC,reference H . - Undo the screw C clockwise. - Remove the drive pulley unit and the target wheel D . | Fig 7.27 |
| --- | --- |

## Lubrication circuit disassembly

| 7.8.1 Oil pressure valve( ) - Undo the cap A . - Remove the spring B , check its condition and replace it if broken. - Remove the valve piston C using a magnet. | Fig 7.28 |
| --- | --- |
| 7.8.2 Timing system carter oil filling flange ( ) - Undo the screws D and remove the oil filling flange E ( ST_06 ) . - Remove the gasket F . | Fig 7.29 |
| 7.8.3 Timing system carter - Ensure that the crankshaft with the 1st cylinder is at the TDC (taper pin W must be facing upwards). - Undo the screws G . - Remove the timing system semi-crankcase H . | Fig 7.30 |
| 7.8.4 Oil pump Important - The oil pump is not repairable. - Undo the screws L and remove the pump unit M from the timing system carter H ( ST_06 ) . | Fig 7.31 |
| - Remove the rotors N and P from oil pump carter M . | Fig 7.32 |
| 7.8.5 Oil Cooler unit and lub. oil filter - Screw the lid cartridge holder V . - Undo the screws Q, R and remove Oil Cooler group S . Warning - Use a suitable container to recover any residue oil. - Oil Cooler unit S is not repairable. | Fig 7.33 |
| - Remove the gaskets T and U . NOTE: To replace the oil cartridge, refer to Par. 6.10.2 . | Fig 7.34 |
| 7.8.6 Oil vapour separator unit - Release the clamps AA . - Remove the manifolds AB, AC and AD . - Remove the clamp AE by carefully cutting it at the point indicated and remove the oil separator AF . | Fig 7.35 |
| - Undo the screws AG . - Remove la flangia di supportoRemove flange support AH and the gasket seal AL . | Fig 7.36 |

## Intake manifold disassembly

| - Undo the screws A and remove the semi-manifold B ( ST_05 ) . - Remove the separation plate C and the gaskets H . | Fig 7.37 |
| --- | --- |
| - Undo the screws D ( ST_06 ) and E . - Remove the semi-manifold F and the gasket G . | Fig 7.38 |

## Fuel system disassembly

| 7.10.1 Fuel return pipes - Release the clip A . Warning - After removing the union, the clip A must automatically return to its initial position; otherwise it must be replaced. - Undo the B distributor return fixing capscrew. - Release the clamp D . - Disconnect the pipe E from the fuel return fitting. - Undo and remove the screw G with the relative gaskets and put the cap on the Common Rail pressure relief valve AA . - Disconnect unions M from electronic injectors N . - Remove the fuel return pipes. | Fig 7.39 Fig 7.40 |
| --- | --- |
| 7.10.2 Fuel flow pipes Important - Seal all openings of the inlet and return unions on the injection pump D with the relevant caps, in order to prevent impurities from entering. - Loosen the clamps P . - Remove the pipes Q and R . | Fig 7.41 |
| 7.10.3 High pressure fuel pipes Danger - The fuel injection circuit undergoes high pressure, use safety protections as described in Par 3.4.3 . - Ensure that the Common Rail is not under pressure by slowly and carefully unscrewing one of the nuts S . - Loosen the nuts S and T in sequence. - Fully undo the nuts S and T in sequence, and remove the high pressure pipes U and V . | Fig 7.42 |
| 7.10.4 Common Rail - Undo the screws AB and remove the Common Rail AC . NOTE: Take care to protect te sensor AD from knocks, moisture and any high temperature source. The internal parts of the rail cannot be repaired. Important - Seal all injection component unions as illustrated in Par. 2.9.8 . - Common rail cannot be repaired. | Fig 7.43 |
| 7.10.5 Electronic injectors Important - In the event that the electronic injectors are disassembled (not necessarily replaced), mark them with the relevant cylinder number from which they originate so as not to confuse them during re-assembly (Fig. 7.45) . - The electronic injectors cannot be repaired. - If one or more electronic injectors are to be replaced, the new calibration data must be inserted in the ECU via a specific instrument ( ST_01 ). - Be careful not to damage the gaskets X . - Undo capscrews AE and remove them together with the relative washers AF and then brace AG . - Pull out the electronic injector AH . NOTE: Should you be unable to remove the electronic injector (acting only on point BC ), use an open-ended spanner (Ø 34 mm), by applying small rotations to unblock the component. - Seal all injection component unions as illustrated in Par. 2.9.8 . - Ensure that gasket AL has remained in the correct position (Fig. 7.46) . Otherwise, recover the gasket from inside the electronic injector AM . | Fig 7.44 Fig 7.45 |
| 7.10.6 Fuel filter ( ) - Undo the fuel cartridge AW from support AP . - Undo the screws AN and remove the filter support AP . | Fig 7.46 |
| 7.10.7 High-pressure fuel injection pump Important - Before disassembling, carefully read Par. 2.17 . - The injection pump is not repairable - Should the fuel feeding pump need to be replaced, after assembly, it is necessary to perform the Pump Learning procedure by means of instrument ST_01 . - Undo the nut AQ . - Loosen the screws AR of the injection pump. - Screw the tool ST_04 on the thread of the gear AS . - Tighten the screw AT on the puller to disconnect the gear AS from the injection pump and remove the injection pump control gear AS . - Undo the screws AR . - Remove injection pump AU and the relative gasket AV . - Seal all injection component unions as illustrated in Par. 2.9.8 . | Fig 7.47 Fig 7.48 |

## Timing system gear disassembly

| - Unscrew screws H and remove target wheel G . - Unscrew screws D and remove camshaft gear E . - Remove retainer ring A and the shoulder ring B . - Remove the intermediate gear C . | Fig 7.49 |
| --- | --- |
| - Remove the shoulder ring F . - Unscrew the screws L and remove the intermediate gear support M . | Fig 7.50 |

## Flange unit disassembly

| 7.12.1 Flywheel Important - Leave the special tool ST_34 for blocking the flywheel ( Par. 7.7 ) . - Only undo the screw C located uppermost. - Insert the tool ST_09 in the seat of the screw C tightening it all the way. - Undo the remaining screws D . Danger - The flywheel E is very heavy. Pay the utmost attention while removing it in order to prevent it dropping or falling, as this may have serious consequences for the operative. - Remove the flywheel E . - Remove the tool ST_09 . - Remove the tool ST_34 shown in Fig. 7.11 . | Fig 7.51 |
| --- | --- |
| 7.12.2 Flange housing - Undo the screws A and remove the engine housing B . Danger - The flange housing is very heavy. Pay the utmost attention while removing it in order to prevent it dropping or falling, as this may have serious consequences for the operative. | Fig 7.52 |

## Cylinder head unit disassembly

| 7.13.1 Rocker arms cover - Undo the screws A . - Remove the rocker arms cover B . - Remove the gasket C . | Fig 7.53 |
| --- | --- |
| 7.13.2 Rocker arm pin - Undo the screws D . - Remove the rocker arm pin unit E . | Fig 7.54 |
| 7.13.2.1 Rocker arm ( ) - Remove the retainer ring F . - Remove the shoulder rings G . - Remove the rocker arms H . | Fig 7.55 |
| 7.13.3 Valve rods and bridges - Remove the valve control U-bolts M . - Remove the rocker arm control rods N . | Fig 7.57 |
| 7.13.4 Cylinder head Important - The cylinder head fastening bolts P must be replaced every time they are disassembled. - Undo the bolts P . Important - To lift cylinder head Q , only use both eyebolts AE provided by KOHLER (refer to Fig. 7.66 ). - When removing the cylinder head Q and subsequent disassembly, control, and assembly operations, it is necessary to protect the contact surface W of cylinder head Q and crankcase J against impacts. - Remove the cylinder head Q . - Remove the head gasket R . | Fig 7.58 Fig 7.59 |
| 7.13.4.1 Valves ( ) - Mount the tool ST_07 on the head AF fixing it on one of the holes for fixing the rocker arm cover. NOTE: Change the fixing hole according to the position of the valves to be removed. - Position the tool ST_07 on the valve as shown in the figure. | Fig 7.60 |
| - Push the lever of the tool ST_07 downwards, in order to lower the valve plates S in the direction of the arrow T , remove cotters U using a magnet. NOTE: Repeat all the operations for all the valves concerned. | Fig 7.61 |
| Important - Before removing the valves, make some marks to record their original position, in order to avoid confusing them when reassembling (if they are not replaced). - Remove the valves V . | Fig 7.62 |
| 7.13.4.2 Electronic injector sleeves ( ) - Unscrew and remove the sleeves Z from the head Q . - Remove the gaskets AA and AB . | Fig 7.63 |
| 7.13.4.3 Valve stem gasket ( ) - Remove the gaskets AC . | Fig 7.64 |
| 7.13.4.4 Lifting eyebolts ( ) - Undo the screws AD and remove the eyebolts AE . - Thoroughly wash the cylinder head Q . | Fig 7.65 |

## Oil sump unit disassembly

| 7.14.1 Oil sump - Undo the screws A . - Remove the oil sump B by inserting a plate in the areas indicated by the arrow AA . | Fig 7.66 |
| --- | --- |
| 7.14.2 Oil suction pipe - Undo the screws C and remove the oil pipe D . | Fig 7.67 |
| 7.14.3 Oil vapour pipes ( ) - Unscrew and remove the pipes E . | Fig 7.68 |

## Engine block disassembly

| 7.15.1 Crankshaft gasket flange - Undo the screws A . - Remove the flange B and the gasket C . | Fig 7.69 |
| --- | --- |
| 7.15.2 Piston unit/connecting rod Important - Mark some numerical references (cylinder n°) on the connecting rods, connecting rod caps F1 , pistons and gudgeon pins to prevent unintentionally confusing the components not replaced during assembly. Failure to do this may result in engine malfunctions. - References on connecting rod M and cap F1 must only be carried out on a side in correspondence with K1 and K2 , as illustrated in Fig. 7.70a . - Screw the bolt AM temporarily. - Unsrew bolts E1 and remove the connecting rod caps F1 . | Fig 7.70 |
| NOTE : coupling cap F1 on the connecting rod can be carried out with centring taper pins (Fig. 7.70b) or broken ( Fig. 7.70c - without centring taper pins). Fig. 7.70b Fig. 7.70c | Fig 7.70a |
| - Pull out the connecting rod - piston assembly from position 2 and 3 by manually applying pressure on the connecting rod big end L in the direction of arrow AK . - Couple the connecting rod big end caps L with the relevant piston and connecting rod unit M . - Turn capscrew AM and rotate the crankshaft by 180°. - Repeat points 2 to 5 to disassemble the connecting rod-piston assembly to position 1 and 4 . | Fig 7.71 |
| Warning - The connecting rod half-bearings Z are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.72 |
| 7.15.3 Lower semi-crankcase 3 CYLINDERS - Undo capscrews E and F by following the order indicated in the figure. - Remove the lower semi-crankcase D and store it in a suitable container for washing. | 3 Cylinders Fig 7.73 |
| 4 CYLINDERS - Undo capscrews E and F by following the order indicated in the figure. - Remove the lower semi-crankcase D and store it in a suitable container for washing. | 4 Cylinders Fig 7.74 |
| 7.15.4 Crankshaft Remove: - Crankshaft G . - The shoulder semi-rings H . | Fig 7.75 |
| 7.15.5 7.15.5 Piston ( ) - Remove the retainer ring N . - Remove the pin P to separate the piston Q from the connecting rod R . Important - If they are not replaced, keep the components together (connecting rod - piston - gudgeon pin) by using references in order to prevent them from getting mixed up during assembly. | Fig 7.76 |
| 7.15.5.1 Rings ( ) - Remove the rings S . | Fig 7.77 |
| 7.15.6 Oil spray nozzles ( ) - Undo the screws T and remove the spray nozzles U from the upper semi-crankcase AB . | Fig 7.78 |
| 7.15.7 Camshaft - Remove the lock ring V . - Extract the camshaft W from the upper semi-crankcase AB . | Fig 7.79 |
| 7.15.8 Camshaft tappets - With a magnet, remove the tappets AA from the upper semi-crankcase AB . | Fig 7.80 |
| 7.15.9 Crankshaft bushings - Remove the crankshaft bushings AC from the upper crankcase AB . Important - The crankshaft half-bearings AC are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.81 |
| - Remove the crankshaft bushings AF from the lower semicrankcase AE . | Fig 7.82 |
| 7.15.10 Cover 3 th PTO ( ) - Undo the screws AG . - Remove the cover AH and the gasket AL from the semicrankcase top. | Fig 7.83 |