Information for disassembly

KDI 3404 TM Workshop Manual (Rev. 08.3)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information for disassembly

## Recommendations for disassembly

Important

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- The operator should prepare all equipment and tools in order to enable him to carry out the operations correctly and safely.

- Before disassembly, perform the operation described in Chap. 5 .

- Before proceeding with operation, carefully read Chap. 3 .

- In order to operate safely and easily, we recommend positioning the engine on a rotating stand for engine overhauling.

- Seal all injection component unions as illustrated in Par. 2.9.8 during assembly.

- Protect all disassembled components and coupling surfaces subject to oxidation with lubricant.

- Where necessary, reference to special tools to use during disassembly operations is indicated (es. ST_05 ), identified in Tab. 13.1 - 13.2 - 13.3 .

## Turbocharger disassembly

| - Unscrew the fittings A and remove the pipe B with the relative gaskets C . | Fig 7.1 |
| --- | --- |
| - Undo the screws D and remove pipe E and the relevant gaskets. | Fig 7.2 |
| - Undo the nuts F and remove the turbocharger G . | Fig 7.3 |

## Coolant recirculation components disassembly

| 7.3.1 Oil Cooler manifold - Release the clamps A . - Undo the screw B and remove hoses C . | Fig 7.4 |
| --- | --- |
| - Release the clamp D and remove hoses E . | Fig 7.5 |
| 7.3.2 Coolant pump Important - The pump B is not repairable. - Perform the operations described in Par. 6.2.1 . - Undo capscrews F and remove flange G with the relative gasket. | Fig 7.6 |
| 7.3.3 Thermostatic valve - Undo the screws A and remove the thermostatic valve cover B . - Remove the thermostatic valve C and its gasket. Important - Always replace the gasket D every time it is disassembled. - Check that the air bleeding hole is not clogged or blocked ( Par. 2.11.3 ). | Fig 7.7 |

## Electric components disassembly

| 7.4.1 Starter motor Important - The motor is not repairable. - Perform the operations from point 2 to 3 of Par. 6.1.5 . 7.4.2 Alternator - Undo the screws A1 and B1 and remove the alternator C1 . | Fig 7.8 |
| --- | --- |
| 7.4.2 Sensors and switches Important - After disassembly, protect the sensors suitably against knocks, dampness and any high temperature sources. - The sensors and switches cannot be repaired, therefore they must be replaced in the event of anomalies. 7.4.2.1 Oil pressure switch ( ) - Unscrew and remove the oil pressure switch AD . | Fig 7.9 |
| 7.4.2.2 Coolant temperature sensor ( ) - Unscrew and remove the coolant temperature sensor AE . | Fig 7.10 |

## Exhaust manifold disassembly

| - Remove nuts A , capscrews B and spacers C , manifold D and gaskets E . - Close the openings and manifolds to prevent foreign bodies from entering. | Fig 7.11 |
| --- | --- |

## Fuel system disassembly

| Important - Seal all injection component unions as illustrated in Par. 2.9.8 during disassembly. 7.6.1 Injection fuel pipes - Perform the operations of Par. 6.1.1 . |  |
| --- | --- |
| 7.6.2 Rocker arms cover - Perform the operations of Par. 6.1.1 . |  |
| 7.6.3 Fuel return pipes - Perform the operations of Par. 6.1.3 . - Perform the operations of point 18 of Par. 6.1.5 . - Loosen union A from cylinder head B and remove return line C . | Fig. 7.12 |
| 7.6.4 Injectors - Perform the operations of Par. 6.1.4 . |  |
| 7.6.5 Injection pump - Perform the operations of Par. 6.1.5 . |  |
| 7.6.6 Fuel filter - Perform the operations of Par. 6.5.1 . |  |

## Crankshaft pulley disassembly

| - Undo the screws A and remove the pulley B . | Fig 7.13 |
| --- | --- |

## Flange unit disassembly

| 7.8.1 Flywheel Danger - The flywheel A is very heavy. Pay the utmost attention while removing it in order to prevent it dropping or falling, as this may have serious consequences for the operative. - Undo the screws B and remove the flywheel A by means of tool ST_43 . - Secure tool ST_41 onto gear C by means of capscrews B . | Fig 7.14 |
| --- | --- |
| 7.8.2 Flange housing Danger - The flange housing D is very heavy. Pay the utmost attention while removing it in order to prevent it dropping or falling, as this may have serious consequences for the operative - Undo capscrews E by following the order indicated in the figure. - Remove the engine housing D by means of tool ST_44 . | Fig 7.15 |

## Lubrication circuit disassembly

| 7.9.1 Oil pump Important - The oil pump is not repairable. - Undo the screws A and remove the pump unit B . | Fig 7.16 |
| --- | --- |
| 7.9.2 Oil pressure valve ( ) - Remove cotter pin C . - Remove disk D , spring E , piston valve F using a magnet. | Fig 7.17 |

## Cylinder head unit disassembly

| 7.10.1 Rocker arm pin - Undo the screws D . - Remove the rocker arm pin unit E . | Fig 7.18 |
| --- | --- |
| 7.10.1.1 Rocker arm ( ) - Remove the retainer ring F . - Remove the shoulder rings G . - Remove the rocker arms H . | Fig 7.19 |
| 7.10.3 Valve rods and bridges - Remove the valve control U-bolts M . - Remove the rocker arm control rods N . | Fig 7.20 |
| 7.10.4 Cylinder head Important - The capscrews P must be replaced every time they are disassembled. - Do NOT remove the capscrews completely, first loosen them by turning them a whole cycle following the order shown in the figure. - Loosen fastening screws P , turning them by one turn following the order shown in the figure. - Undo capscrews P by following the order indicated in the figure. Important - To lift cylinder head Q , only use both eyebolts AE provided by KOHLER (refer to Fig. 7.28 ). - When removing the cylinder head Q and subsequent disassembly, control, and assembly operations, it is necessary to protect the contact surface W of cylinder head Q and crankcase J against impacts. - Remove the cylinder head Q . - Remove the head gasket R . | Fig 7.21 Fig 7.22 |
| 7.10.4.1 Valves ( ) - Mount the tool ST_07 on the head Q fixing it on one of the holes for fixing the rocker arm cover. NOTE: Change the fixing hole according to the position of the valves to be removed. - Position the tool ST_07 on the valve as shown in the figure. | Fig 7.23 |
| - Push the lever of the tool ST_07 downwards, in order to lower the valve plates S in the direction of the arrow T , remove cotters U using a magnet. NOTE: Repeat all the operations for all the valves concerned. | Fig 7.24 |
| Important - Before removing the valves, make some marks to record their original position, in order to avoid confusing them when reassembling (if they are not replaced). - Remove the valves V . | Fig 7.25 |
| 7.10.4.2 injector sleeves ( ) - Unscrew and remove the sleeves Z from the head Q . - Remove the gaskets J, K . | Fig 7.26 |
| 7.10.4.3 Valve stem gasket ( ) - Remove the gaskets W . | Fig 7.27 |
| 7.10.4.4 Lifting eyebolts ( ) - Undo the screws X and remove the eyebolts Y . - Thoroughly wash the cylinder head Q . | Fig 7.28 |

## Oil sump unit disassembly

| 7.11.1 Oil sump - Undo the screws A . - Remove the oil sump B by inserting a plate between surface C of crankcase D and oil sump B . - Remove the oil dipstick E . | Fig 7.29 |
| --- | --- |
| 7.11.2 Oil suction pipe - Undo the screws F and remove the oil pipe G . | Fig 7.30 |
| 7.11.3 Oil drain pipe ( ) - Undo the screws H and remove the pipe L . | Fig 7.31 |

## Engine block disassembly

| 7.12.1 Piston unit/connecting rod Important - Mark some numerical references (cylinder n°) on the connecting rods, connecting rod caps N , pistons and gudgeon pins to prevent unintentionally confusing the components not replaced during assembly. Failure to do this may result in engine malfunctions. - References on connecting rod M and cap N must only be carried out on a side in correspondence with K1 and K2 , as illustrated in Fig. 7.35 . - Screw the bolt M temporarily. | Fig 7.32 |
| --- | --- |
| NOTE : coupling cap N on the connecting rod can be carried out with centring taper pins (Fig. 7.33) or broken ( Fig. 7.34 - without centring taper pins). Fig. 7.33 Fig. 7.34 | Fig 7.35 |
| - Pull out the connecting rod - piston assembly from position 2 and 3 by manually applying pressure on the connecting rod big end M in the direction of arrow AK . - Couple the connecting rod big end caps L with the relevant piston and connecting rod unit M . - Rotate the crankshaft by 180°. - Repeat points 2 to 5 to disassemble the connecting rod-piston assembly to position 1 and 4 . | Fig 7.36 |
| Warning - The connecting rod half-bearings Z are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.37 |
| 7.12.2 Timing system gear disassembly - Unscrew screws A and remove the gear B . - Remove the gear C . - Unscrew screws D and remove the gear E . | Fig 7.38 Fig 7.39 |
| 7.12.3 Lower semi-crankcase Important - The capscrews Q must be replaced every time they are disassembled. - Do NOT remove the capscrews completely, first loosen them by turning them a whole cycle following the order shown in the figure. - Loosen fastening screws Q , turning them by one turn following the order shown in the figure. - Undo capscrews Q by following the order indicated in the figure. | Fig 7.40 |
| Important - The capscrews R must be replaced every time they are disassembled. - Do NOT remove the capscrews completely, first loosen them by turning them a whole cycle following the order shown in the figure. - Loosen fastening screws R , turning them by one turn following the order shown in the figure. - Undo capscrews R by following the order indicated in the figure. - Remove the lower semi-crankcase D1 and store it in a suitable container for washing. | Fig 7.41 |
| 7.12.4 Crankshaft Remove: - Crankshaft S . - The shoulder semi-rings T . - gasket U from crankshaft S . | Fig 7.42 |
| 7.12.5 Piston ( ) - Remove the retainer ring V . - Remove the pin Z to separate the piston J from the connecting rod L . Important - If they are not replaced, keep the components together (connecting rod - piston - gudgeon pin) by using references in order to prevent them from getting mixed up during assembly. | Fig 7.43 |
| 7.12.5.1 Rings ( ) - Remove the rings K . | Fig 7.44 |
| 7.12.6 Oil spray nozzles ( ) - Undo the screws W and remove the spray nozzles X from the upper semi-crankcase D2 . | Fig 7.45 |
| 7.12.7 Camshaft - Remove the lock ring C . - Extract the camshaft F from the upper semi-crankcase D2 . | Fig 7.46 |
| 7.12.8 Camshaft tappets - With a magnet, remove the tappets Y from the upper semi-crankcase D2 . | Fig 7.72 |
| 7.12.9 Crankshaft bushings - Remove the crankshaft bushings A1 from the upper crankcase D2 . Important - The crankshaft half-bearings A1, B1 are made of special material. Therefore, they must be replaced every time they are removed to prevent seizures. | Fig 7.73 |
| - Remove the crankshaft bushings B1 from the lower semicrankcase D2 . | Fig 7.74 |