Information for replacing the functional units

KDI 3404 TM Workshop Manual (Rev. 08.4)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information for replacing the functional units

## Injector and injection pump replacement

| Important - Before proceeding with operation, carefully read Par. 3.3.2 . - Replace the high pressure pipes after two disassemblies. - Seal all injection component unions as illustrated in Par. 2.9.7 during disassembly. - Always replace the gaskets after each disassembly. - Handle the components as described in Par. 2.18 . - Refer to Par. 1.3 for operating references when assembling and disassembling. - When repaired, RSN-A injectors must be certified by a Stanadyne centre to check their correct operation - check the type of engine mounted injectors on the spare parts list ( RSN-A is specified in the description). | Fig 6.1 |
| --- | --- |
| 6.1.1 Injection fuel pipes disassembly (injection pump/injectors) - Undo the screw s A . - Undo the screws B . - Remove quick fitting C . - Undo the screws D and remove the manifold E . | Fig 6.2 |
| - Undo the screw H1 and remove the clamp H2 and remove rubber. - Undo the nuts F . - Undo the nuts G . - Remove the tube H . | Fig 6.3 |
| 6.1.2 Rocker arms cover disassembly - Undo the screw L . - Undo the screw M. - Undo the screws N and remove the rocker arm cap P . | Fig 6.4 |
| 6.1.3 Fuel return pipes disassembly - Undo the screws Q and remove hose R . | Fig 6.5 |
| 6.1.4 Injectors disassembly - Undo the screw J and remove washer K and then bracket X . - Remove the injector Z . NOTE: Should you be unable to remove the electronic injector (acting only on point BC), use an open-ended spanner (Ø 11 mm), by applying small rotations to unblock the component. - Seal all injection component unions as illustrated in Par. 2.9.7 . - Ensure that gasket S has remained in the correct position ( Fig. 6.7 ). Otherwise, recover the gasket from inside the electronic injector V manifold. | Fig 6.6 Fig 6.7 |
| NOTE: Click on the right to play the procedure. | https://www.youtube.com/embed/mt-Dsw4A81A?rel=0 |
| 6.1.5 Injection pump disassembly Importante - Before proceeding with the disassembly, identify the pump code from its identifying name plate ( Pos. 12 - Tab. 2.12 ) and remove the cylinder injector 1 ( Par. 1.4 - 6.1.1 - 6.1.2 - 6.1.3 - 6.1.4 ). - Alternatively, you can identify the pump from the online spare parts catalogue ( https://partners.lombardini.it/App/SparepartCatalogue/Default/Catalogue.aspx ) - Insert the tool ST_30 into the injector N°1 and fix it with the fixing brace X , capscrew J and washer K . NOTE : Do not tighten the capscrew J . | Fig 6.8 |
| - Disassemble the starter motor. - Mount the tool ST_34 in the seat of the starter motor Y and fit it with the two starter motor fixing screws. - Rotate the crankshaft clockwise (Rif. A Par. 1.4 ) through the ST_34 tool bringing reference X upwards. NOTE : When positioning reference X , make sure cylinder N° 1 is in compression phase (the valves on piston N° 1 must all be closed). | Fig 6.9 |

| - With reference X pointed upwards, find the TDC through tool ST_30 , then bring the dial gauge indicator to 0. | Fig 6.10 |
| --- | --- |
| - By means of the identified pump code, refer to Tab. 6. 1 to know the advance degrees and the corresponding value to lower the piston. - Having identified the value to lower the piston, rotate the crankshaft anti-clockwise by going beyond the value described in Tab. 6.1 , once again, rotate the crankshaft clockwise, stopping at the correct advance value by using tool ST_30 ( Rif. A Par. 1.4 ). NOTE: The value indicated in Tab. 6.1 must be reached by rotating the shaft with the piston in compression phase. Use the ST_34 tool to totate the crankshaft. | Pump code; Piston lowering (mm); Advance. ED0065905290-S; 0,733 (0.562 - 0.927); 8° (± 1°). ED0065905430-S; 0,562 (0.413 - 0.733); 7° (± 1°). ED0065905440-S; 1,033 (0.828 - 1.261); 9,5° (± 1°). ED0065905690-S; 0,645 (0.485 - 0.828); 7.5° (± 1°). Tab. 6.1 |
| - Lock the ST_34 tool through J screws and ensure that the crankshaft does not rotate, which would alter the correct advance value. If this happens, repeat the instructions described in points 4 , 5 , 6 , 7 and 8 . - Undo the screws A1 , remove the plate B1 . | Fig 6.11 |
| - Undo and remove the nut C1 fixing the injection pump control gear D1 . Important - After removing the nut C1 , ensure that the correct advance value has remained unchanged on ST_30 . - Be careful that the nut C1 does not fall into the timing cover. | Fig 6.12 |
| - Undo the capscrew E1 and shift the slotted plate F1 in the direction of arrow G1 . - Tighten screw E1 to block the pump (tightening torque to 12 Nm ). - Screw the tool ST_13 on the gear D1 . | Fig 6.13 |
| - Perform the operations of point 1 of Par. 5.2 . - Remove quick fitting N1 . - Undo the screws K1 and K2 and remove the Oil Cooler group L1 from the crankcase M1 . - Loosen the screws J2 . | Fig 6.14 |
| - Undo the screw P1 . - Tighten the screw of tool ST_13 to disconnect the injection pump J1 from the high pressure pump control gear D1 . - Svitare le viti J2 ed estrarre la pompa iniezione J1 . - DO NOT remove the tool ST_13 . | Fig 6.15 |
| NOTE: Click on the right to play the procedure. | https://www.youtube.com/embed/lll9hIO0pXM?rel=0 |
| 6.1.6 Injection pump assembly Important - Before assembling the new pump J1 , make sure that plate F1 can move freely and that fastening capscrews E1 are not loose (the pump sold as a spare part is, upplied with the cylinder injection timing blocked N° 1 ). - Ensure that the coupling surfaces on shaft Q1 and gear D1 are free from impurities and lubrication residues. - Remove the guard cap only when the pipes are reconnected. - Do not remove the tool ST_30 . - Mount the injection pump J1 , inserting the shaft Q1 in the gear D1 . Important - Always change screws J2 with new ones or apply Loctite 2701 to the threads. - Clamp the screws J2 on the crankcase M1 (tightening torque at 25 Nm ). - Remove the tool ST_13 . - Ensure that the correct advance value has remained unchanged, tighten nut C1 on shaft Q1 (as shown in Fig. 6.17, you are allowed to use a screwdriver to guide nut C1 onto shaft Q1 in order to prevent it from accidentally falling inside carter S1 - tightening torque at see service letter 710018 ). | Fig 6.16 Fig 6.17 |
| - Undo the capscrew E1 and shift the slotted plate F1 in the direction of arrow G2 . - Tighten screw E1 (tightening torque to 5.5 Nm ). The injection pump is unlocked. - Remove the tool ST_30 and ST_34 . | Fig 6.18 |
| - Assemble Oil Cooler L1 onto crankcase M1 by means of capscrews K1 and K2 . NOTE: Always replace the gasket R1 , R2 at each assembly. | Fig 6.19 |
| - Secure tube U1 by means of capscrew P1 , inserting gasket T1 . - Fit quick coupling N1 onto pump J1 . | Fig 6.20 |
| - Secure plate B1 by means of capscrews A1 , inserting gasket V1 onto carter S1 (tightening torque at 10 Nm ). | Fig 6.21 |
| NOTE: Click on the right to play the procedure. | https://www.youtube.com/embed/xAUa9IQBmpU?rel=0 |
| 6.1.7 Injector assembly Important - To prevent damaging the injection system, the protection caps ( Par. 2.9.7 ) must be removed during assembly. - Lubricate the gaskets U, T, S , and fit them on the injector Z . | Fig 6.22 |
| - Fit the injector Z in the sleeve V . | Fig 6.23 |
| - Assemble the parts P, Q, R . and fit the parts so assembled on the injector Z . | Fig 6.24 |
| - Insert tool ST_52 on the injectors junctions Z (detail X2 ). - Tighten the screw J (tightening torque to 20 Nm - Fig. 6.24 ). | Fig 6.25 |
| 6.1.8 Assembly of the injector return pipes - Position the tube N on the injectors Z and tighten screws M (tightening torque to 14 Nm ). | Fig 6.26 |
| 6.1.9 Assembly Rocker arm cover Important - The gasket Z1 between the rocker arm cover and the cylinder head must always be replaced every time it is disassembled. - Position tool ST_17 onto the head in correspondence with the two fastening holes 9 and 10 . - Position gasket Z1 and cap P on cylinder head A2 matching the holes of fastening capscrews N with the aid of the gudgeon guides ST_17 . - Attach the rocker arm cover P on the head A2 with the screw N adhering to the tightening sequence shown in Fig. 6.28 (tightening torque to 10 Nm ). - Secure the hose M2 with the screw L (tightening torque to 10 Nm ). - Clamp the fitting M3 with the screw M (tightening torque to 25 Nm ) inserting the gasket B2 . | Fig 6.27 Fig 6.28 |
| 6.1.10 Installation of the fuel injector pipes (pump injector/injectors) - Position pipes D on the injectors and on the injector pump. Important - Tighten the nuts F and G manually, without clamping them. - Tighten the nuts F and G (tightening torque at 25 Nm ). - Secure tubes H by means of clamps H2 , assembling: - H3 rubber element; - clamp H2 on element H3 ; - secure clamp H2 by means of capscrew H4 and nut H5 (tightening torque at 10 Nm ). - Secure manifold E onto cylinder head A2 by means of capscrews D , inserting gasket C2 . - Secure suction line E2 onto manifold E by means of capscrews A , inserting gasket D2 . - Fit quick coupling C onto manifold E . - Fasten the pipe H6 on the manifold E with the screws B. | Fig 6.29 |
| Fig. 6.30/6.31 |  |

## Coolant pump replacement

| 6.2.1 Disassembly NOTE: Perform the operations described in Par. 5.1 . Important - Before proceeding with operation, read Par. 3.3.2 . - The coolant pump is not repairable. - Loosen the screws A and B . - Loosen capscrew C and disconnect voltage from belt D and remove belt D . - Undo the screws E and remove the pulley F . | Fig 6.32 Fig 6.33 |
| --- | --- |
| - Undo the screws G and remove the pump H with the relevant gasket. | Fig 6.34 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/FdI56hBo_R0?rel=0 |
| 6.2.2 Assembly Important - Always replace the gaskets J , after each disassembly. - Always replace the belt D after each assembly. - To handling components refer to Par. 2.17 - Always replace the gaskets (where are provided) after each disassembly - Fit the coolant pump G with the screws H interposing the new gasket J (tightening torque at 25 Nm ). | Fig 6.35 |
| - By means of capscrews E , secure pulley F to crankcase K ( tightening torque at 25 Nm). | Fig 6.36 |
| - Insert the belt D on the pulleys M . - Tighten screw C and bring block L at 10 mm from bracket N (value C1 ). - Fit the screw A (tightening torque at 25 Nm ). - Fit the screw B (Fig. 6.32 - tightening torque at see service letter 710007 ). - Start the engine and run it for some minutes, then turn off it, and let it cool down at ambient temperature. Check by the appropriate tool that at point P the tension value is between 135 and 178 Hz. NOTE: If the poly-v belt tension results out of the above mentioned values, proceed with the replacement. | Fig 6.37 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/edCJrMN0G5M?rel=0 |

## Oil vapour separator replacement

| 6.7.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Remove quick fitting A . - Release the clamps B and C . - Remove hose D from breather body E . | Fig 6.38 |
| --- | --- |
| - Remove capscrews F and remove breather body E . | Fig 6.39 |
| 6.7.2 Assembly - Secure breather body E by means of capscrews F (tightening torque at 22 Nm ). - Fit hose D onto breather body E (Fig. 6.38) . - Secure the clamps B and C (Fig. 6.38). | Fig 6.40 |

## Oil cooler unit and oil filter replacement

| 6.4.1 Oil Cooler unit disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Perform the operations described in Par 5.1 and Par 5.2 . - Oil Cooler unit E is not repairable. - Release the clamps A . - Remove the manifolds B out of the Oil Cooler unit E . | Fig 6.41 |
| --- | --- |
| Warning - Electric/pneumatic screwdrivers are forbidden. - Use a suitable container to recover any residue oil. - Unscrew cartridge holder cover H by performing three complete turns and wait 1 minute. NOTE : this operation allows to oil contained in the support E to flow into the oil sump in the correct way. - Unscrew cartridge holder cover H and check that the oil in the lub. oil filter support E has flowed towards the oil sump. - Remove quick fitting N1 . | Fig 6.42 |
| - Undo the screws C and D and remove the Oil Cooler unit E . - Remove the gaskets F and G from the Oil Cooler unit E . | Fig 6.43 |
| 6.4.2 Oil filter cartridge replacement - Remove gaskets L, M and N from element holder cover H . - Remove cartridge P from element holder cover H . | Fig 6.44 |
| - Lubricate and insert gaskets L, M and N in the L1, M1 and N1 seats of element holder cover H . - Insert element P into element holder cover H . | Fig 6.45 |
| 6.4.3 Oil Cooler unit assembly Important - In the event of assembly of union U on crankcase S , manual tightening torque with Loctite 2701 on the thread). - Check that the surface Q on the support E and on the crankcase S are free from impurities. - Lubricate and insert the gasket T on the fitting U . - Lubricate and insert the gaskets on the support E : F in seat F1 ; G in seat G1 . - Fit the support R with the screws C and D (tightening torque at 10 Nm ). - Insert and tighten the cartridge support H on the filter support E (tightening torque at 25 Nm ). - Fit the hoses B on the support E and secure the hoses B with the clamps A . - Fit quick coupling N1 onto pump J1 . | Fig 6.46 Fig 6.47 |

## Fuel filter replacement

| 6.5.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . Warning - The fuel filter is not always mounted in the engine. - When disassembling, use a suitable container to recover the fuel contained in the cartridge F . - Remove quick fitting N1 . - Release the clamps A and pull the pipes B out of the support H . - Unscrew the cartridge F from the support H . - Undo the screws C and remove the support H . | Fig 6.48 Fig 6.49 |
| --- | --- |
| Warning - Check that the fuel supply pump filter is present, and replace if necessary. - Release the clamp D . - Demount the hose E . - Unscrew the filter G from the pump Q . |  |
| - Screw the new filter G onto the pump Q (tightening torque 20 Nm ). - Connect the hose E to the filter G and fasten with the clamp D . |  |
| 6.5.2 Assembly - Secure fuel filter bracket H with capscrews C onto crankcase M , inserting spacer L between M and H and washer P between H and C (tightening torque at 25 Nm ). - Fit the pipes B on the support H . - Secure the pipes B with the clamps A . | Fig 6.50 |
| - Lubricate the gasket N with fuel. - Tighten the cartridge F on the support H (tightening torque at 17 Nm ). - Fit quick coupling N1 onto support H . | Fig 6.51 |