Information for replacing the functional units

KDI 1903TCR / KDI 1903TCRE5 Workshop manual (Rev. 17.8)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information for replacing the functional units

## Electronic injector replacement

| Important - Before proceeding with operation, read Par. 3.3.2 . - In the event of the electronic injectors being disassembled (not necessarily replaced) their position with respect to individual cylinders must not be changed when re-assembled. Refer to the reference between each injector and respective cylinder number. - Seal all injection component unions as illustrated in Par. 2.9.8 during disassembly. - Handle the components as described in Par. 2.17 . - Replace all seal gaskets after each assembly for all components on which they are provided. - The high pressure pipes must be replaced every time they are disassembled. - Before disassembling the electronic injectors, make sure the new high pressure pipes are available. - If a new (or different) electronic injector is fitted on the engine, the new calibration data must be entered in the ECU through a specific instrument ( ST_01 ) . - Electronic injectors are not repairable. - This procedure may be performed on one or more electronic injectors. NOTE: In the event of a leak upon replacement (oil - coolant - fuel - air), do not intervene with the engine running, but stop it and wait for 5/10 minutes before checking and solving the problem. | Fig 6.1 |
| --- | --- |
| 6.1.1 Fuel return pipes disassembly (Common Rail/electronic injectors) - Disconnect the connector C . | Fig 6.2 |
| - Remove clips E from the electronic injector F . - Disconnect the junction G from the electronic injector F . Warning - After removing the fittings, the clips E must automatically return to their initial position; otherwise they must be replaced. - Seal all injection component unions as illustrated in Par. 2.9.8 . | Fig 6.3 |
| 6.1.2 High pressure fuel pipes disassembly (Common Rail/electronic injectors) Danger - The fuel injection circuit is under high pressure, use safety protections as described in Par 3.4.3 . - Ensure that the Common Rail is not under pressure by slowly and carefully unscrewing one of the nuts H . - Undo the nut H on the Common Rail L and then the nut M on the electronic injector F and remove the pipe N . Important - In the event that the electronic injectors are disassembled (not necessarily replaced), mark them with the relevant cylinder number from which they originate so as not to confuse them during re-assembly. - Seal all injection component unions as illustrated in Par. 2.9.8 . | Fig 6.4 |
| 6.1.3 Electronic injectors disassembly - Undo and remove the screw P with the washer R e and then the bracket Q . Important - Be careful not to damage the gaskets X . - Replace rings X , if damaged. 2. Pull out the electronic injector F . NOTE: Should you be unable to remove the electronic injector (acting only on point BC ), use an open-ended spanner (Ø 34 mm), by applying small rotations to unblock the component. 3. Seal all injection component unions as illustrated in Par. 2.9.8 . 4. Ensure that gasket S has remained in the correct position (Fig. 6.6) . Otherwise, recover the gasket from inside the electronic injector T manifold. | Fig 6.5 Fig 6.6 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/QQZtx2i75AY?rel=0 |
| 6.1.4 Electronic injector assembly Important - Always replace and lubricate the gaskets AA and S of the electronic injectors F with fuel, every time they are replaced. - Reposition the electronic injectors (not replaced) by following the references made for disassembly, as indicated in Par. 6.1.2. - If the engine is painted or protected with clear paint, clean the paint off the diesel injector F near to the part in contact with the gasket AB . - Insert the gasket S on the electronic injectors F (Fig. 6.7) . - Insert electronic injector F into manifold T , being extra careful not to damage gasket AB and direct it as indicated in Fig. 6.7 . | Fig 6.7 |
| 6.1.5 High pressure fuel pipes assembly Important - Always replace the pipes N after each assembly. - If the engine is painted or protected with clear paint, replace the fastening screws P to ensure the gaskets are sealed properly. - Position tube N in the Common Rail seat of the electronic injector; correct the position of the electronic injector by means of the entrance of the electronic injector unions F and Common Rail L . - Apply the nuts H and M by hand without tightening them. - Position the fastening brace of electronic injectors Q on capscrew surface AD , insert capscrews P in brace Q inserting washer R . | Fig 6.8 |
| Important - Ensure that brace S is perfectly positioned onto the electronic injector. 4. Tighten the fixing screws P of the electronic injector bracket (tightening torque at 20 Nm ). 5. Tighten the nut M (tightening torque at 25 Nm ). 6. Tighten the nut H (tightening torque at 30 Nm ). | Fig 6.9 |
| 6.1.6 Fuel return pipes assembly - Check the condition of the gaskets AE . | Fig 6.10 |
| - Insert unions AF onto electronic injectors F and block them with clips E . - Mount the connectors C on the electronic injectors F . Warning - Slightly move the wiring support to check that the electrical wire of connector C is not strained in correspondence with the outlet hole AF . | Fig 6.11 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/ArOgFV739EU?rel=0 |

## High-pressure fuel injection pump replacement

| Danger - The fuel injection circuit undergoes high pressure, use safety protections as described in Par 3.4.3 . - Ensure that the Common Rail is not under pressure by slowly and carefully unscrewing nut A . Important - Before proceeding with operation, read Par. 3.3.2 . - Always replace the high pressure pipes after each disassembly. - Before disassembling the injection pump, make sure the new high-pressure pipe is available. - The injection pump is not repairable. - Should the fuel feeding pump need to be replaced, after assembly, it is necessary to perform the Pump Learning procedure by means of instrument ST_01 . - Seal all injection component unions as illustrated in Par. 2.9.8. - To handling components refer to Par. 2.17. - Always replace the gaskets (where are provided) after each disassembly. | Fig 6.12 |
| --- | --- |
| 6.2.1 High-pressure fuel line disassembly (from the injection pump to the Common Rail). - Undo the nut A . | Fig 6.13 |
| - Undo nut D from Common Rail E . | Fig 6.14 |
| - Undo the nut B on the intake manifold C and remove the pipe F . | Fig 6.15 |
| 6.2.2 Timing system carter oil filling flange disassembly - Remove starter motor ( Par. 7.3.2 ) and assemble special tool ST_34 ( Par. 7.7 point 2 ) . - Undo the screws G ( ST_06 ) ,remove the clamp E and the flange H . | Fig 6.16 |
| 6.2.3 High-pressure fuel injection pump disassembly - Undo and remove nut L fixing the fuel feeding pump control gear M . Important - Be careful that the nut L does not fall into the timing cover. - Tighten tool ST_04 on the gear M . | Fig 6.17 |
| Important - Do NOT use the cylinder connection pipe W as a handle, to prevent damage or fuel leaks. - Before disassembling, carefully read Par. 2.17 . - Seal all injection component unions as illustrated in Par. 2.9.8. - Release the clamps N on the return pipe P and on the fuel inlet pipe Q . - Disconnect tubes P and Q from fuel feeding pump R . - Disconnect connectors S and T . - Loosen the screws U . - Redo the capscrew of tool ST_04 to disconnect injection pump R from gear M . - Undo capscrews U and extract injection pump R with the relevant gasket V . - Undo and remove the tool ST_04 . | Fig 6.18 Fig 6.19 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/UaZgKyWrP48?rel=0 |
| 6.2.4 High-pressure fuel injection pump assembly Important - Before assembling, carefully read Par. 2.17 - Always replace the gasket V after each assembly. The gasket V can only be fitted in one direction. - Remove the tool ST_04 from the pump control gear ( Ref. M of Par. 6.2.3 ) if applicable. - Do NOT use the cylinder connection pipe W as a handle, to prevent damage or fuel leaks. - Remove the protection caps only when reconnecting the hoses. | Fig 6.20 |
| - Check that the contact surfaces AA are free from impurities. - Insert the reference key K in the seat of the shaft Z . - Assemble the new gasket V on injection pump R . Insert injection pump R in its housing on crankcase AA making key K coincide with key seat AH of gear M . | Fig 6.21 |
| - Fully tighten the nut L on the shaft Z of the injection pump. Important - Apply nut L by hand, but do not tighten. | Fig 6.22 |
| Important - It is mandatory to replace the screws U or apply a few drops of Loctite 270 . - Clamp the screws U on the crankcase AB (tightening torque at 25 Nm ). - Clamp the nut L (Fig. 6.22) (tightening torque at 65 Nm ). | Fig 6.23 |
| - Fit the connector T on the sensor J . - Fit the connector S on the sensor Y . - Remove the protection caps. - Fit the pipe Q on the fitting AA . - Fit the pipe P on the fitting AB . - Hook the clamps N on the hoses Q and P . | Fig 6.24 |
| 6.2.5 High-pressure line assembly (injection pump / Common Rail) - Remove the protection cap. - Position the pipe F . Important - Manually tighten the nut A . | Fig 6.25 |
| - Manually tighten the nut D . - Fix clamp AC by means of capscrew B on intake manifold C (tightening torque 10 Nm - ST_06 ). - Clamp nut D (tightening torque at 30 Nm ) and A (tightening torque at 25 Nm ) in sequence. | Fig 6.26 |
| 6.2.6 Timing system carter oil filling flange assembly NOTE: Always replace the gasket AE after each assembly. - Position the gasket AE in the set on the flange H . - Fix the flange H on the crankcase AF with the screws AG (tightening torque at 10 Nm - ST_06 ). - Fit the clamp E on the flange H . - Disassemble the special tool ST_34 and assemble the starter motor (tightening torque 45 Nm ). | Fig 6.27 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/o3h6Say9sc4?rel=0 |

## Unit EGR Cooler replacement

| 6.3.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - To handling components refer to Par. 2.17. - Always replace the gaskets (where are provided) after each disassembly. NOTE: Perform the operations described in Par. 5.1 . - Undo the screws B of pipe C . | Fig 6.28 |
| --- | --- |
| - Undo screws D and E . - Remove pipe F and the relevant metal gaskets. - Release the clamps G and remove the sleeve M . | Fig 6.29 |
| - Undo the screws H and remove the EGR Cooler L from the sleeve N ( ST_05 ). - Should the passage ducts of the gas exhaust be clogged by soot or carbon, replace EGR Cooler L . | Fig 6.30 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/xGWUnc-V1YY?rel=0 |
| 6.3.2 Assembly - Insert the fitting U in the manifold M of the EGR valve unit. - Fit the EGR Cooler L with the screws H on the intake manifold S (tightening torque at 22 Nm - ST_05 ) - Insert the hose G on the fitting V . - Secure the clamps F . | Fig 6.31 |
| - Insert the gasket N between the hose B and the EGR Cooler L and fix the screws A (tightening torque at 25 Nm ) | Fig 6.32 |
| - Insert the hose E in its housing on the manifold S inserting the gasket R . - Insert the gasket T between the hose E and the EGR Cooler L and fit the screws C (tightening torque at 25 Nm ). - Fit the screws D (tightening torque at 22 Nm - ST_05 ). NOTE: Perform the operations described in Par. 10.2 . | Fig 6.33 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/XSTfzyJa-9Q?rel=0 |

## EGR valve replacement

| 6.4.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . NOTE: Perform the operations described in Par. 5.1 . - Disconnect the connector A from the valve C . - Undo the screws B and remove the EGR valve C with the relevant gasket. | Fig 6.34 |
| --- | --- |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/lZlk78GFzsg?rel=0 |
| 6.4.2 Assembly Important - Always replace gasket D after each assembly. - The EGR valve is not a serviceable item, and if faulty / worn out, should be replaced with a new one. - Movimentare i componenti come descritto nel Par. 2.17 . - Mount the new gasket D on the valve C . - Fit the valve C on the flange E with screws B (tightening torque at 10 Nm ). | Fig 6.35 |
| - Fit the connector A on the valve C . NOTE: Perform the operations described in Par. 10.2 | Fig 6.36 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/KGHm0dnsQdc?rel=0 |

## Coolant pump replacement

| 6.5.1 Disassembly NOTE: Perform the operations described in Par. 5.1 . Important - Before proceeding with operation, read Par. 3.3.2 . - If the engine is fitted with the Poly-V belt, perform the operations described in Par. 11.3 . - Loosen the screws A and B . - Push the alternator C in the direction of the arrow D and remove the belt E . - Release clamp F and disconnect tube G from the coolant pump N . NOTE : If union R , is disassembled, replace it or alternatively apply Loctite 2701 on the thread when assembling on pump G (tightening torque of 20 Nm ). | Fig 6.37 Fig 6.38 |
| --- | --- |
| - Release the clamp M from the coolant pump N . - Undo the screws H and remove the pump N with the relevant gasket L . | Fig 6.39 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/_QESHZf50PU?rel=0 |
| 6.5.2 Assembly Important - Always replace the gaskets L , after each disassembly. - Always replace the belt E after each assembly. - If the engine is fitted with the Poly-V belt, perform the operations described in Par. 11.3 . - To handling components refer to Par. 2.17. - Always replace the gaskets (where are provided) after each disassembly - Fit the coolant pump N with the screws H interposing the new gasket L (tightening torque at 25 Nm ). | Fig 6.40 |
| - Reinsert the pipe G and hook the clamp F ( Fig. 6.38 ). - Rehook the clamp M on the pump N (Fig. 6.39) . - Push the alternator C in the direction of the arrow D . - Insert the belt E on the pulleys P . | Fig 6.41 |
| - Pull the alternator C in the direction of the arrow Q . - While tensioning the alternator C , first clamp screw A (tightening torque at 25 Nm ) and then screw B (tightening torque at 69 Nm [thread M10] - 40 Nm [thread M8] ). - Check the tension of the belt E with the instrument ( DENSO BTG-2 ), positioning it in point P (the tension must be between 350 and 450 Nm ). - If the tension values do not correspond, tighten screws A and B , then repeat operations 6, 7 and 8 . | Fig 6.42 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/GbvNS15R9SQ?rel=0 |

## Target wheel replacement

| 6.6.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Position the crankshaft with the 1st cylinder in TDC, reference A upwards. - Remove the alternator belt following steps 1 and 2 ( Par. 6.5.1 ) . | Fig 6.43 |
| --- | --- |
| - Disassemble the starting motor. - Mount the tool ST_34 in the seat of the starter motor C and fit it with the two starter motor fixing screws. | Fig 6.44 |
| - Disconnect the connector L . - Undo the screw M and remove the revolution sensor N and its respective spacer. | Fig 6.45 |
| - Undo the screw P (clockwise) and remove the pulley/targetwheel unit Q . | Fig 6.46 |
| - Undo the screws R and remove the target wheel S with the relevant sound-absorption disk T . | Fig 6.47 |
| 6.6.2 Assembly - Check that the pin U is mounted properly on the pulley V . - Insert the disk T on the pulley V respecting the reference of the pin U . - Position the target wheel S on the pulley V respecting the reference of the pin U . - Fit the target wheel S with the screws W (tightening torque 10 Nm ). - Perform the operations described in Par. 6.7.7 and then the operations between point 2 and 9 of Par. 6.5.2 . | Fig 6.48 |

## Oil pump replacement

Important

- Before proceeding with operations, read  Par. 3.3.2 .

- The oil pump is not repairable.

| 6.7.1 Refrigerant pump disassembly - Perform the operations described in Par 6.5.1 . 6.7.2 Crankshaft and target wheel pulley disassembly - Perform the operations described in Par 6.6.1 . - Disconnect the connector AE from the sensor S . | Fig 6.49 |
| --- | --- |
| 6.7.3 Timing system crankcase disassembly Important - Perform the operations described in Par. 5.2 . - Make sure that the reference pin A is facing upwards. - Undo the screw H ( ST_06 ) and remove the sensor S . - Undo the screws B and remove the timing system crankcase C . | Fig 6.50 |
| 6.7.4 Oil pump disassembly - Undo the screws D ( ST_06 ) and remove the group pump E from the timing system crankcase C . - Remove the rotors F and G from the oil pump crankcase E . | Fig 6.51 Fig 6.52 |
| 6.7.5 Oil pump assembly NOTE: Carry out the checks described in Par. 8.7 prior to assembly. - Check that all surfaces in contact between F, G, H, E and C are free from impurities - scratches - dents. - When assembling, do not use any type of gasket between E and C . - Thoroughly lubricate the seat of the rotors H on the oil pump crankcase E and the two rotors F and G . - Within housing H insert the 2 rotors (in sequence) G and F , observing the references BP as described in figure (or refer to Par. 2.10.2 ). - Check that the 2 pins L are inserted properly in the timing system crankcase C . - Position the oil pump carter E using the reference pins L . - Clamp the oil pump carter E with the screws D (tightening torque 10 Nm - ( ST_06 ) ). | Fig 6.53 Fig 6.54 |
| 6.7.6 Timing system crankcase assembly Important - Always replace the oil seal J after each assembly. - Always replace the gasket P after each assembly. - To prepare the surface of the K plane for the new application of the sealant, it must be cleaned through the use of: - initially Loctite SF 7200 - subsequently Loctite SF 7063 Avoid any contact with the K plane and be careful not to compromise the cleaning performed. - Lubricate the lip of the oil seal J . - Apply a coating of Loctite 5188 around 1mm thick on the surfaces K of the crankcase C . - Make sure that the key M (Fig. 6.56) is inserted properly on the crankshaft and that it is facing upwards. - Check that the 2 pins N are inserted properly in the timing system crankcase C . | Fig 6.55 |
| - Lubricate and insert the gasket P in the seat of the oil pump Q . - Tighten the tool ST_10 on the crankshaft. - Position the crankcase C on the base, using the reference pins N , inserting the oil pump Q on the crankshaft. | Fig 6.56 |
| - Fit the timing system crankcase C with the screws R observing the indicated clamping sequence (tightening torque at 25 Nm ). | Fig 6.57 |
| - Assemble sensor S by means of capscrew T on carter C inserting gasket U (tightening torque at 10 Nm ST_06 ). | Fig 6.58 |
| 6.7.7 Crankshaft and target wheel pulley assembly - Leave the tool ST_34 mounted ( Fig. 6.44 ) . - Check that the pin A is mounted properly on the crankshaft Z . - Position the pulley unit W on the crankshaft Z respecting the reference with the pin A . - Apply Molyslip grease on the screw thread Y . - Clamp the pulley unit W with the screw Y (tightening torque at 360 Nm ). - Remove the special tool ST_34 ( Fig. 6.44 ) . | Fig 6.59 |
| - Mount the bracket Z with the screws AA (tightening torque at 10 Nm ). - Perform the operations described in Par. 9.12 . - Insert the shim AB on the sensor AC . - Clamp the sensor AC on the bracket Z with the screw AD (tightening torque at 10 Nm ). 6.7.8 Coolant pump assembly - Perform the operations described in Par 6.5.2 . | Fig 6.60 |

## Oil pressure valve replacement

| 6.8.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Undo cap A . - Remove spring B , check its condition and replace it if broken. - Remove the valve piston C using a magnet. | Fig 6.61 |
| --- | --- |
| 6.8.2 Assembly - Lubricate the piston C and fully insert it in the seat E . - Insert the spring B in the piston. NOTE: Always replace the gasket F after each assembly. - Mount the gasket F on cap A . - Clamp the cap A on the crankcase D (tightening torque at 50 Nm ). | Fig 6.62 |

## Oil vapour separator replacement

| 6.9.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Release the clamp AA and remove the pipe D . - Release the clamps F . - Remove the clamp P cutting it in the point indicated and remove the separator body C removing it from the hose AG and G . | Fig 6.63 |
| --- | --- |
| - Release the clamp F . - Remove the pipes G and AG . - Remove the clamp J . - Undo the screws B . - Release the clamp S from the sleeve K . - Pull the flange H out of the manifold K and remove the relevant gasket, being careful not to bend the pipe E . | Fig 6.64 |
| 6.9.2 Assembly Warning - Always carefully inspect the condition of the tubes, and replace them if there is any doubt regarding their integrity. - Always replace the gasket M after each assembly. - Check that the contact surface L is free from impurities. - Position flange H inserting hose K onto the flange union H , being careful not to bend tube E . - Insert the gasket M between the flange H and the crankcase N . - Secure the flange H using the screws B on the crankcase N (tightening torque at 10 Nm ). - Secure the clamp S on the manifold K . - Fit the pipes G and AG on the flange H . - Fit the breather body C on the pipes G and D and fit the pipe G with the clamps F and the pipe D with the clamp AA . - Fit the breather body C on the support flange H with the new clamp P . | Fig 6.65 Fig 6.66 |

## Oil cooler unit and oil filter replacement

| 6.10.1 Oil Cooler unit disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Perform the operations described in Par 5.1 and Par 5.2 . - Oil Cooler unit E is not repairable. - Release the clamps A . - Remove the pipes B out of the Oil Cooler unit E . | Fig 6.67 |
| --- | --- |
| Warning - Electric/pneumatic screwdrivers are forbidden. - Use a suitable container to recover any residue oil. - Unscrew cartridge holder cover H by performing three complete turns and wait 1 minute. NOTE : this operation allows to oil contained in the support E to flow into the oil sump in the correct way. - Unscrew cartridge holder cover H and check that the oil in the lub. oil filter support E has flowed towards the oil sump. - Undo the screws C and D and remove the Oil Cooler unit E . | Fig 6.68 |
| - Remove the gaskets F and G from the Oil Cooler unit E . | Fig 6.69 |
| 6.10.2 Oil filter cartridge replacement - Remove gaskets L, M and N from element holder cover H . - Remove cartridge P from element holder cover H . | Fig 6.70 |
| - Lubricate and insert gaskets L, M and N in the L1, M1 and N1 seats of element holder cover H . - Insert element P into element holder cover H . | Fig 6.71 |
| 6.10.3 Oil Cooler unit assembly Important - In the event of assembly of union U on crankcase S , manual tightening torque with Loctite 2701 on the thread). - Check that the surface Q on the support E and on the crankcase S are free from impurities. - Lubricate and insert the gasket T on the fitting U . - Lubricate and insert the gaskets on the support E : F in seat F1 ; G in seat G1 . - Fit the support R with the screws C and D (tightening torque at 10 Nm ). - Insert and tighten the cartridge support H on the filter support E (tightening torque at 25 Nm ). - Fit the pipes B on the support E and secure the pipes B with the clamps A . | Fig 6.72 Fig 6.73 |

## Fuel filter replacement

| 6.11.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . Warning - The fuel filter is not always mounted in the engine. - When disassembling the sensor E , use a suitable container to recover the fuel contained in the cartridge F . - Release the clamps A and pull the pipes B out of the support H . - Unscrew the sensor E from the cartridge F . - Unscrew the cartridge F from the support H . - Undo the screws C and remove the support H . | Fig 6.74 Fig 6.75 |
| --- | --- |
| Warning - Check that the fuel supply pump filter is present, and replace if necessary. - Release the clamp D . - Demount the hose E . - Unscrew the filter G from the pump Q . |  |
| - Screw the new filter G onto the pump Q (tightening torque 20 Nm ). - Connect the hose E to the filter G and fasten with the clamp D . |  |
| 6.11.2 Assembly - Clamp the fuel filter support H with the screws C on the crankcase M (tightening torque at 25 Nm ). - Fit the pipes B on the support H . - Secure the pipes B with the clamps A . | Fig 6.76 |
| - Lubricate the gasket N with fuel. - Tighten the cartridge F on the support H (tightening torque at 17 Nm ). - Assemble gasket J onto sensor E and lubricate with fuel. - Tighten the sensor E on the cartridge F (tightening torque at 5 Nm ). | Fig 6.77 |

## Replacement of SCV valve

| Important - Before starting any replacement operations, make sure the work area is free from dust (part X of valve B is extremely sensitive to micro-dust). - Pay the utmost attention to cleaning in order to prevent any type of contamination during replacement operations - - Before proceeding with the replacement, clean the outer part of pump A thoroughly - Avoid any type of contact with part X of the valve during replacement. - Lubricate part X of valve B with oil spray. - Before starting any replacement operations, make sure that the key on the vehicle’s panel is OFF . - Assemble the new valve in the same position as the previous one. | Fig. 6.78 |
| --- | --- |
| 6.12.1 Disassembly 1 - Disconnect connector C from valve B . 2 - Loosen screws D . 3 - Remove valve B from pump A . | Fig. 6.79 |
| 6.12.2 Assembly 1 - Insert studs E supplied with valve B in the fastening holes of pump A and insert gasket F in the seat of pump A . 2 - Assemble valve B on pump A using studs E as positioning guides. 3 - Remove studs E and secure valve B with screws D (tightening torque of 6 Nm). 4 - Fasten valve B by means of screws D (tightening torque of 10 Nm). | Fig. 6.80 |
| Fig. 6.81 | Fig. 6.82 |