Information about optional components

KDI 3404 TCR-SCR Workshop Manual (Rev. 10.4)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about optional components

## Heater (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.1.1 Disassembly - Undo the screws A . - Remove the flange C . - Remove the heater E and the relevant gaskets F . | Fig 11.1 |
| 11.1.2 Assembly Important - Always replace gaskets F , with each assembly. - In sequence, fit the manifold G with the gasket F , the new heater E , the second gasket F , the flange C , the washers H , the screws A and the cable B . - Secure the flange C with the screws A (tightening torque at 22 Nm ). | Fig 11.2 |

## Idler gear (for 3rd / 4th PTO)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.2.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Loosen capscrews A and remove plate B1 . - Extract gear C . - Remove gudgeon D together with plate B2 . | Fig 11.3 |
|  | Fig 11.4 |
| 11.2.2 Assembly Important - Make sure that gudgeon D has no impurities inside. - On capscrew A , assemble: - plate B1 - gudgeon D - gear C - plate B2 . | Fig 11.5 |

| - Position gear unit C1 onto crankcase E , complying with reference J1 , J2 with gear F . - Secure unit C1 by means of capscrew A (tightening torque 25 Nm ). NOTE: the reference J1 can have 2 different configurations for the gear C , Fig. 11.6b shows the correct position of the reference J1 for both configurations. | Fig 11.6 Fig 11.6b |
| --- | --- |

## 3rd PTO (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.3.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Extract gear A . | Fig 11.7 |
| 11.3.2 Assembly - Insert gear A into the seat of crankcase B , fitting the shaft of pump C in gear A . | Fig 11.8 |

## 4th PTO (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.4.1 Disassembly - Perform the operations of point 7 of Par. 7.4.1 . - Perform the operations from point 2 to 3 of Par. 6.6.1 . - Perform the operations of point 3 of Par. 7.8.1 . - Perform the operations of Par. 7.8.2 . - Extract gear A . | Fig 11.9 |
| 11.4.2 Assembly - Insert gear A into the seat of crankcase B , fitting the shaft of pump C in gear A . | Fig 11.10 |

## Balancer device (replacement)

| Importante - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.5.1 Disassembly - Perform the operations described in Par. 11.2.1 . - Extract shaft A1 e A2. | Fig 11.11 |
| 11.5.2 Assembly - Lubricate gudgeon C of shaft A1 and A2 with oil. - Insert shaft A1 into seat B1 of the crankcase, complying with reference D of gear E . - Insert shaft A2 into seat B2 of the crankcase. - Perform the operations described in Par. 11.2.2 . NOTE: the shafts A1 and A2 are different and it is important not to invert the assembly position, the reference D is specific for the shaft A1 and is timed with the gear E , the reference J2 is specific for the shaft A2 and is timed with the idler gear F (see the Fig. 11.12b ). Fig 11.12 Fig 11.12b |  |

## ETB (replacement)

| - Unscrew screws A and remove the ETB valve B with its gasket C . Important - Always replace the gasket C at each assembly. | Fig. 11.13 |
| --- | --- |
| - Fix the ETB valve B and its gasket C by means of screws A (tightening torque 10 Nm ) | Fig. 11.14 |