Information for replacing the functional units

KDI 3404 TCR-SCR Workshop Manual (Rev. 10.4)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information for replacing the functional units

## Electronic injector replacement

| Important - Before proceeding with operation, read Par. 3.3.2 . - In the event of the electronic injectors being disassembled (not necessarily replaced) their position with respect to individual cylinders must not be changed when re-assembled. Refer to the reference between each injector and respective cylinder number. - Seal all injection component unions as illustrated in Par. 2.9.8 during disassembly. - Handle the components as described in Par. 2.17 . - Replace all seal gaskets after each assembly for all components on which they are provided. - The high pressure pipes must be replaced every time they are disassembled. - Before disassembling the electronic injectors, make sure the new high pressure pipes are available. - If a new (or different) electronic injector is fitted on the engine, the new calibration data must be entered in the ECU through a specific instrument ( ST_01 ) . - Electronic injectors are not repairable. - This procedure may be performed on one or more electronic injectors. NOTE: In the event of a leak upon replacement (oil - coolant - fuel - air), do not intervene with the engine running, but stop it and wait for 5/10 minutes before checking and solving the problem. | Fig 6.1 |
| --- | --- |
| 6.1.1 Fuel return pipes disassembly (Common Rail/electronic injectors) - Disconnect the connector C . | Fig 6.2 |
| - Remove clips E from the electronic injector F . - Disconnect the junction G from the electronic injector F . Warning - After removing the fittings, the clips E must automatically return to their initial position; otherwise they must be replaced. - Seal all injection component unions as illustrated in Par. 2.9.8 . | Fig 6.3 |
| 6.1.2 High pressure fuel pipes disassembly (Common Rail/electronic injectors) Danger - The fuel injection circuit is under high pressure, use safety protections as described in Par 3.4.3 . - Ensure that the Common Rail is not under pressure by slowly and carefully unscrewing one of the nuts H . - Undo the nut H on the Common Rail L and then the nut M on the electronic injector F and remove the pipe N . Important - In the event that the electronic injectors are disassembled (not necessarily replaced), mark them with the relevant cylinder number from which they originate so as not to confuse them during re-assembly. - Seal all injection component unions as illustrated in Par. 2.9.8 . | Fig 6.4 |
| 6.1.3 Electronic injectors disassembly - Undo and remove the screw P with the washer R e and then the bracket Q . Important - Be careful not to damage the gaskets X . - Replace rings X , if damaged. 2. Pull out the electronic injector F . NOTE: Should you be unable to remove the electronic injector (acting only on point BC ), use an open-ended spanner (Ø 34 mm), by applying small rotations to unblock the component. 3. Seal all injection component unions as illustrated in Par. 2.9.8 . 4. Ensure that gasket S has remained in the correct position (Fig. 6.6) . Otherwise, recover the gasket from inside the electronic injector T manifold. | Fig 6.5 Fig 6.6 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/slELtJW2bFE?showinfo=0&rel=0 |
| 6.1.4 Electronic injector assembly Important - Always replace and lubricate the gaskets AA and S of the electronic injectors F with fuel, every time they are replaced. - Reposition the electronic injectors (not replaced) by following the references made for disassembly, as indicated in Par. 6.1.2. - If the engine is painted or protected with clear paint, clean the paint off the diesel injector F near to the part in contact with the gasket AB . - Insert the gasket S on the electronic injectors F (Fig. 6.7) . - Insert electronic injector F into manifold T , being extra careful not to damage gasket AB and direct it as indicated in Fig. 6.7 . NOTE : to replace gaskets AB , follow the operations in Para. 7.12.1 and 9.5.9 . | Fig 6.7 |
| 6.1.5 High pressure fuel pipes assembly Important - Always replace the pipes N after each assembly. - If the engine is painted or protected with clear paint, replace the fastening screws P to ensure the gaskets are sealed properly. - Position tube N in the Common Rail seat of the electronic injector; correct the position of the electronic injector by means of the entrance of the electronic injector unions F and Common Rail L . - Apply the nuts H and M by hand without tightening them. - Position the fastening brace of electronic injectors Q on capscrew surface AD , insert capscrews P in brace Q inserting washer R . | Fig 6.8 |
| Important - Ensure that brace S is perfectly positioned onto the electronic injector. 4. Tighten the fixing screws P of the electronic injector bracket (tightening torque at 20 Nm ). 5. Tighten the nut M (tightening torque at 25 Nm ). 6. Tighten the nut H (tightening torque at 30 Nm ). Important - Replace the pipes N (Fig. 6.8) if the screws P are stiff when tightened. | Fig 6.9 |
| 6.1.6 Fuel return pipes assembly - Check the condition of the gaskets AE . | Fig 6.10 |

| - Insert unions G onto electronic injectors F and block them with clips E . - Mount the connectors C on the electronic injectors F . Warning - Slightly move the wiring support to check that there is no voltage in the electrical wire of connector C in correspondence with the outlet hole AF . | Fig 6.11 |
| --- | --- |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/IVoumDwS7oY?showinfo=0&rel=0 |

## High-pressure fuel injection pump replacement

| Danger - The fuel injection circuit undergoes high pressure, use safety protections as described in Par 3.4.3 . - Ensure that the Common Rail is not under pressure by slowly and carefully unscrewing nut A . Important - Before proceeding with operation, read Par. 3.3.2 . - Always replace the high pressure pipes after each disassembly. - Before disassembling the injection pump, make sure the new high-pressure pipe is available. - The injection pump is not repairable. - Should the fuel feeding pump need to be replaced, after assembly, it is necessary to perform the Pump Learning procedure by means of instrument ST_01 . . - Seal all injection component unions as illustrated in Par. 2.9.8 - To handling components refer to Par. 2.17 - Always replace the gaskets (where are provided) after each disassembly. | Fig 6.12 |
| --- | --- |
| 6.2.1 High-pressure fuel line disassembly (from the injection pump to the Common Rail). - Undo the nut A . | Fig 6.13 |
| - Undo nut D from Common Rail E . | Fig 6.14 |
| - Undo the nut B1 on the intake manifold C. - Undo the nut B2 on the intake manifold C and remove the pipe F (Fig. 6.16) . | Fig 6.15 |
| 6.2.2 Timing system carter oil filling flange disassembly - Remove starter motor ( Par. 6.6.1 point 2 ) and assemble special tool ST_34 ( Par. 6.6.1 point 3 ) . - Undo the screws G ,remove the plate H . | Fig 6.16 |
| - Undo and remove nut L fixing the fuel feeding pump control gear M . Important - Be careful that the nut L does not fall into the timing cover. - Tighten tool ST_13 on the gear M . | Fig 6.17 |
| Important - Do NOT use the cylinder connection pipe W as a handle, to prevent damage or fuel leaks. - Before disassembling, care read Par. 2.17 . - Seal all injection component unions as illustrated in Par. 2.9.8 - Disconnect tubes P and Q from fuel feeding pump R . - Disconnect connectors S and T . - Loosen and distance capscrews U . - Redo the capscrew of tool ST_13 to disconnect injection pump R from gear M . - Undo capscrews U and extract injection pump R with the relevant gasket V . Important - Do NOT remove tool ST_13 in order to prevent gear M inside the distribution carter from falling. | Fig 6.18 Fig 6.19 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/jPnRSYu0sKM?showinfo=0&rel=0 |
| 6.2.3 High-pressure fuel injection pump assembly Important - Before assembling, carefully read Par. 2.17 - Always replace the gasket V after each assembly. The gasket V can only be fitted in one direction. - Do NOT use the cylinder connection pipe W as a handle, to prevent damage or fuel leaks. - Remove the protection caps only when reconnecting the hoses. | Fig 6.20 |
| - Check that the contact surfaces AA are free from impurities. - Make sure reference key K is properly inserted into the Z shaft seat. - Assemble the new gasket V on injection pump R . Insert injection pump R in its housing on crankcase AA making key K coincide with key seat AH of gear M . | Fig 6.21 |
| - Remove the tool ST_13 from the pump control gear ( Ref. M of Par. 6.2.3 ) if applicable. Important - Pay the utmost attention when executing the operation in point 5 so as to prevent nut L from falling inside the distribution carter. - Fully tighten the nut L on the shaft Z of the injection pump. Important - Apply nut L by hand, but do not tighten. - It is mandatory to replace the screws U or apply a few drops of Loctite 2701 . | Fig 6.22 |
| - Clamp the screws U on the crankcase AB (tightening torque at 25 Nm ). - Clamp the nut L (Fig. 6.22) (tightening torque at 70 Nm ). - Disassemble the special tool ST_34 and assemble the starter motor (tightening torque 45 Nm ). NOTE: Always replace the gasket AE after each assembly. - Position the gasket AE in the set on the plate H . - Fix the plate H on the crankcase AF with the screws G (tightening torque at 10 Nm ). | Fig 6.23 |
| - Fit the connector T on the sensor J . - Fit the connector S on the sensor Y . - Remove the protection caps. - Fit the pipe Q on the fitting AA . - Fit the pipe P on the fitting AB . | Fig 6.24 |
| 6.2.4 High-pressure line assembly (injection pump / Common Rail) - Remove the protection cap. - Position the pipe F . - Manually tighten the nut A . | Fig 6.25 |
| - Manually tighten the nut D . | Fig 6.26 |
| - Secure clamps F1 and F2 by means of capscrews B1 and B2 onto intake manifold C (tightening torque 10 Nm ). - In order, tighten nut D (tightening torque 30 Nm ) and A (tightening torque 25 Nm ). | Fig 6.27 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/3ULD_PiHEaw?showinfo=0&rel=0 |

## Unit EGR Cooler replacement

| 6.3.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - To handling components refer to Par. 2.17 - Always replace the gaskets (where are provided) after each disassembly. NOTE: Perform the operations described in Par. 5.1 . - Undo the screws B of pipe C . - Release the clamps F and remove the hose M . | Fig 6.28 |
| --- | --- |
| - Undo the screws C of pipe E . - Release the clamp F and remove the hose G. | Fig 6.29 |
| - Undo the screws H and remove the EGR Cooler L and the relevant metal gaskets ( ST_05 ). - Should the passage ducts of the gas exhaust be clogged by soot or carbon, replace EGR Cooler L . | Fig 6.30 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/A8fU76g4nUQ?showinfo=0&rel=0 |
| 6.3.2 Assembly - Fit the EGR Cooler L with the screws H on the intake manifold S (tightening torque at 22 Nm - ST_05 ). | Fig 6.31 |
| - Insert the gasket N between the hoses B-E and the EGR Cooler L . | Fig 6.32 |
| - Fit the screws A and C (tightening torque at 25 Nm ). - Insert the hose M on the fitting V1 and G on V2 . - Secure the clamps F . NOTE: Perform the operations described in Par. 10.2 . | Fig 6.33 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/vTWVObqWIGE?showinfo=0&rel=0 |

## EGR valve replacement

| 6.4.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . NOTE: Perform the operations described in Par. 5.1 . - Disconnect the connector A from the valve C . - Undo the screws B and remove the EGR valve C with the relevant gasket. | Fig 6.34 |
| --- | --- |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/JZWXxa3UssY?showinfo=0&rel=0 |
| 6.4.2 Assembly Important - Always replace gasket D after each assembly. - The EGR valve is not a serviceable item, and if faulty / worn out, should be replaced with a new one. - Handle the components as described in Par. 2.17 . - Mount the new gasket D on the valve C . - Fit the valve C on the flange E with screws B (tightening torque at 10 Nm ). | Fig 6.35 |
| - Fit the connector A on the valve C . NOTE: Perform the operations described in Par. 10.2 . | Fig 6.36 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/JZWXxa3UssY?showinfo=0&rel=0 |

## Coolant pump and Poly-V belt replacement

| 6.5.1 Disassembly NOTE: Perform the operations described in Par. 5.1 . Important - Before proceeding with operation, read Par. 3.3.2 . - Loosen the screws A and B . - Loosen capscrew C and disconnect voltage from belt D and remove belt D . - Undo the screws E and remove the pulley F . | Fig 6.37 Fig 6.38 |
| --- | --- |
| - Undo the screws G and remove the pump H with the relevant gasket. | Fig 6.39 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/tgDL1w2AUd0?showinfo=0&rel=0 |
| 6.5.2 Assembly Important - Always replace the gaskets J , after each disassembly. - Always replace the belt D after each assembly. - To handling components refer to Par. 2.17 - Always replace the gaskets (where are provided) after each disassembly - Fit the coolant pump G with the screws H interposing the new gasket J (tightening torque at 25 Nm ). | Fig 6.40 |
| - By means of capscrews E , secure pulley F to crankcase K . | Fig 6.41 |
| - Insert the belt D on the pulleys M . - Tighten screw C and bring block L at 10 mm from bracket N (value C1 ). - Fit the screw A (tightening torque at 25 Nm ). - Fit the screw B (Fig. 6.37 - tightening torque at see service letter 710007 ). - Start the engine and run it for some minutes, then turn off it, and let it cool down at ambient temperature. Check by the appropriate tool that at point p the tension value is between 135 and 178 Hz. NOTE: If the poly-v belt tension results out of the above mentioned values, proceed with the replacement. | Fig 6.42 |
| NOTE: Click by side to play the procedure. | https://www.youtube.com/embed/Zrhc5qTwPRM?showinfo=0&rel=0 |

## Target wheel replacement

| 6.6.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Remove the alternator belt following steps 1 and 2 ( Par. 6.5.1 ) . - Undo the screws N and Remove starter motor A . - Mount the tool ST_34 in the seat of the starter motor B and fit it with the two starter motor fixing screws. | Fig 6.43 |
| --- | --- |
| Important - Pay the utmost attention when executing the operation in point 4 so as to prevent knocking speed sensor E . - Undo the screws C and remove the pulley D . | Fig 6.44 |
| - Undo the screws F and remove the target wheel G . | Fig 6.45 |
| 6.6.2 Assembly - Check that the pin H is mounted properly on the pulley D . - Position the phonic wheel G on the pulley D respecting the reference of the pin H . - Fit the phonic wheel G with the screws F (tightening torque 10 Nm ). | Fig 6.46 |
| - Check that the pin L is mounted properly on the crankshaft M . Important - Pay the utmost attention when executing the operation in point 4 so as to prevent knocking speed sensor E . - Position the pulley unit D on the crankshaft M respecting the reference with the pin L . - Apply Molyslip grease onto the thread and under the head of capscrew C . - Clamp the pulley unit D with the screw C (tightening torque at 100 Nm ). - Perform the operations from point 3 to 7 of Par. 6.5.2. | Fig 6.47 |
| - Undo the screws N and remove special tool ST_34 . - Secure motor A by means of capscrews N (tightening torque at 45 Nm ). | Fig 6.48 |

## Oil vapour separator replacement

| 6.7.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Remove quick fitting A . - Release the clamps B and C . - Remove hose D from breather body E . | Fig 6.49 |
| --- | --- |
| - Remove capscrews F and remove breather body E . | Fig 6.50 |
| 6.7.2 Assembly Warning - Always carefully inspect the condition of the tubes, and replace them if there is any doubt regarding their integrity. - Always replace the gasket M after each assembly. - Secure breather body E by means of capscrews F . NOTE : Insert clamp G between capscrew F and breather body E . - Fit hose D onto breather body E (Fig. 6.49) . - Secure the clamps B and C (Fig. 6.49). | Fig 6.51 |

## Oil cooler unit and oil filter replacement

| 6.8.1 Oil Cooler unit disassembly Important - Before proceeding with operation, read Par. 3.3.2 . - Perform the operations described in Par 5.1 and Par 5.2 . - Oil Cooler unit E is not repairable. - Release the clamps A . - Remove the manifolds B out of the Oil Cooler unit E . | Fig 6.52 |
| --- | --- |
| Warning - Electric/pneumatic screwdrivers are forbidden. - Use a suitable container to recover any residue oil. - Unscrew cartridge holder cover H by performing three complete turns and wait 1 minute. NOTE : this operation allows to oil contained in the support E to flow into the oil sump in the correct way. - Unscrew cartridge holder cover H and check that the oil in the lub. oil filter support E has flowed towards the oil sump. - Undo the screws C and D and remove the Oil Cooler unit E . | Fig 6.53 |
| - Remove the gaskets F and G from the Oil Cooler unit E . | Fig 6.54 |
| 6.8.2 Oil filter cartridge replacement - Remove gaskets L, M and N from element holder cover H . - Remove cartridge P from element holder cover H . | Fig 6.55 |
| - Lubricate and insert gaskets L, M and N in the L1, M1 and N1 seats of element holder cover H . - Insert element P into element holder cover H . | Fig 6.56 |
| 6.8.3 Oil Cooler unit assembly Important - In the event of assembly of union U on crankcase S , manual tightening torque with Loctite 2701 on the thread). - Check that the surface Q on the support E and on the crankcase S are free from impurities. - Lubricate and insert the gasket T on the fitting U . - Lubricate and insert the gaskets on the support E : F in seat F1 ; G in seat G1 . - Fit the support R with the screws C and D (tightening torque at 10 Nm ). - Insert and tighten the cartridge support H on the filter support E (tightening torque at 25 Nm ). - Fit the hoses B on the support E and secure the hoses B with the clamps A . | Fig 6.57 Fig 6.58 |

## Fuel filter replacement

| 6.9.1 Disassembly Important - Before proceeding with operation, read Par. 3.3.2 . Warning - The fuel filter is not always mounted in the engine. - When disassembling the sensor E , use a suitable container to recover the fuel contained in the cartridge F . - Release the clamps A and pull the pipes B out of the support H . - Unscrew the sensor E from the cartridge F . - Unscrew the cartridge F from the support H . - Undo the screws C and remove the support H . | Fig 6.59 Fig 6.60 |
| --- | --- |
| Warning - Check that the fuel supply pump filter is present, and replace if necessary. - Release the clamp D . - Demount the hose E . - Unscrew the filter G from the pump Q . |  |
| - Screw the new filter G onto the pump Q (tightening torque 20 Nm ). - Connect the hose E to the filter G and fasten with the clamp D . |  |
| 6.9.2 Assembly - Secure the fuel filter support H with the screws C on the crankcase M (tightening torque at 25 Nm ). - Fit the pipes B on the support H . - Secure the pipes B with the clamps A . | Fig 6.61 |
| - Lubricate the gasket N with fuel. - Tighten the cartridge F on the support H (tightening torque at 17 Nm ). - Assemble gasket J onto sensor E and lubricate with fuel. - Tighten the sensor E on the cartridge F (tightening torque at 5 Nm ). | Fig 6.62 |

## Replacement of SCV valve

| Warning - Before starting any replacement operations, make sure the work area is free from dust (part X of valve B is extremely sensitive to micro-dust). - Pay the utmost attention to cleaning in order to prevent any type of contamination during replacement operations - - Before proceeding with the replacement, clean the outer part of pump A thoroughly - Avoid any type of contact with part X of the valve during replacement. - Lubricate part X of valve B with oil spray. - Before starting any replacement operations, make sure that the key on the vehicle’s panel is OFF . - Assemble the new valve in the same position as the previous one. | Fig. 6.60 |
| --- | --- |
| 6. 10 .1 Disassembly 1 - Disconnect connector C from valve B . 2 - Loosen screws D . 3 - Remove valve B from pump A . | Fig. 6.61 |
| 6. 10 .2 Assembly 1 - Insert studs E supplied with valve B in the fastening holes of pump A and insert gasket F in the seat of pump A . 2 - Assemble valve B on pump A using studs E as positioning guides. 3 - Remove studs E and secure valve B with screws D (tightening torque of 6 Nm). 4 - Fasten valve B by means of screws D (tightening torque of 10 Nm). | Fig. 6.62 |
| Fig. 6.63 | Fig. 6.64 |

## Replacement of DEF filter (SCR versions only)

| Warning - Do not lubricate gaskets A with oil or fuel. - Filter D includes gaskets A in the package. - Avoid any type of contamination during replacement. - Before starting any operations, make sure the key on the machine’s panel is OFF and the DEF pump has executed the circuit emptying operation. |  |
| --- | --- |
| 6.11.1 Disassembly 1 - Loosen cap B . 2 - Remove cap B and extract filter bracket C . 3 - Extract filter D . 4 - Use warm DEF to clean the seat of filter D on pump E if impurities are detected. | Fig. 6.65 |
| 6.11.2 Assembly 1 - Lubricate the A gaskets with DEF or distilled water. 2 - Place the filter bracket C together with filter D inside pump E . 3 - Tighten cap B (tightening torque of 20 Nm). | Fig.6.66 |

## Replacement of DEF pump inlet filter (SCR versions only)

| Warning - Do not lubricate union C or connector A with oil or fuel. - Union C comes with gasket E in the package. - Avoid any type of contamination during replacement. - Before starting any replacement operations, make sure the key on the machine’s panel is OFF and the DEF pump has executed the circuit emptying operation. | Fig. 6.67 |
| --- | --- |
| 6.12.1 Disassembly 1 - Disconnect tube A . 2 - Remove lock ring B . 3 - Remove union C . | Fig. 6.68 |
| 6.12.2 Assembly Warning - Make sure gasket E is on union C . 1 - Insert union C in pump D . 2 - Insert lock ring B on pump D and union C . 3 - Fit tube A on the union C . | Fig. 6.69 |

## Replacement of DEF injector (SCR versions only)

| Warning - If injector E is not replaced, gasket F must be replaced with every disassembly. - Injector E cannot be repaired. - It is forbidden to remove injector E1 from injector body E2 . - Gasket F is pre-assembled on the new component E. - Before starting any operations, make sure the key on the machine’s panel is OFF and the DEF pump has executed the circuit emptying operation. - The connectors on the SCR wiring for the connection of the DEF injector ( C ) and SCR-T are identical, mark the connectors in order to tell them apart and avoid inverting the connection upon assembly. | Fig. 6.70 |
| --- | --- |
| 6.13.1 Disassembly - Remove pipes A and B . - Disconnect connector C . - Loosen screws D and remove injector E . 6.13.2 Assembly - Place injector E on support G of SCR H . - Fasten injector E by means of screws D (tightening torque of 8 Nm). - Fit connector C . - Fit tubes A and B . | Fig.6.71 |

## Replacement of DEF tank inlet filter (SCR versions only)

| Warning Avoid any type of contamination during replacement. This information applies if the DEF tank is supplied by KOHLER. |  |
| --- | --- |
| 6.14.1 Disassembly 1 - Release and remove cap A of tank B. | Fig. 6.72 |
| 2 - Press release tabs C1 and C2 and remove lock ring C from tank B. | Fig. 6.73 |
| 3 - Remove filter D. | Fig. 6.74 |
| 6.14.2 Assembly 1 - Insert filter D inside the seat of tank B (Fig. 6.74). 2 - Insert lock ring C inside the seat of tank B and push it until it locks tabs C1 and C2 (Fig. 6.73). 3 - Assemble cap A on tank B securing it in a locked position | Fig. 6.75 |

## SCR sensor replacement (SCR VERSIONS ONLY)

| Warning - The NOx sensors are identical, the Upstream and Downstream sensors0 can be distinguished by their assembly position on SCR ( Upstream = SCR Input ¦ Downstream = SCR Output ). This position allows distinguishing also the connection of the SCR wiring ( D1 > SCR Input = 5-pin connector ¦ D2 > SCR Output = 4-pin connector ). - The NOx sensors can be without Upstream and Downstream identification label (SCR Input/Output). - The NOx sensors supplied as spare parts are without Upstream and Downstream identification label (SCR Input/Output). After assembly, mark them for a future identification ( GREEN = SCR Input ¦ YELLOW = SCR Output ). - The connectors on the SCR wiring for the connection of the AdBlue® injector and SCR-T are identical, mark the connectors in order to tell them apart and avoid inverting the connection upon assembly. |  |
| --- | --- |
| 6.15.1 SCR-T 6.15.1.1 Disassembly 1 - Disengage connector A . 2 - Unscrew and remove SCR-T sensor B . 6.15.1.2 Assembly 1 - Screw SCR-T sensor B on SCR C . 2 - Tighten SCR-T sensor B to SCR C (tightening torque of 45 Nm ). 3 - Fit connector A to SCR-T sensor B . | Fig. 6.76 |
| 6.15.2 NO x 6.15.2.1 Disassembly 1 - Disengage connectors D1 and D2 . 2 - Loosen screws E and remove SCUs F . 3 - Loosen and remove NOx sensors G from SCR C . 6.15.2.2 Assembly 1 - Screw NOx sensor G on SCR C . 2 - Tighten NOx sensors G to SCR C (tightening torque of 60 Nm ). 3 - Fasten SCUs F with screws E (tightening torque of 3 Nm ). 4 - Fit connector D1 and D2 as described in the connection plates ( D1 > Upstream = 5-pin connector ¦ D2 > SCR Downstream = 4-pin connector ). | Fig. 6.77 Fig. 6.78 Fig.6.79 |

## SCR Replacement

| 6.16.1 Disassembly 1 - Perform the operations indicated in Par. 6.12.1 point 3, 6.15.1.1 and 6.15.2.1. 2 - Loosen clamp B through screws A . 3 - Disengage SCR C from exhaust line D . | Fig. 6.80 |
| --- | --- |
| Warning - Avoid tensioning the assembly of the SCR C on the exhaust line, the MAX misalignment allowed on the junction point is of 1° . 6.16.2 Assembly 1 - Fit SCR C on the exhaust line. 2 - Insert clamps B on the junctions and tighten screw A (tightening torque of 12 Nm ). 3 - Perform the operations indicated in Par. 6.12.2 point 1 and 2, 6.15.1.2 and 6.15.2.2 . | Fig. 6.81 |