Assembly information

KDI 3404 TCR-SCR Workshop Manual (Rev. 10.4)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Assembly information

## Information on engine configuration

- In this chapter, the engine is represented as "BASE CONFIGURATION" (refer to Par. 1.4 - 1.5 ).

- For the assembly of components not described in this chapter refer to Chap. 11 .

- The following are the components described in Chap. 11 .

11.1 Heater (reaplacement)
11.2 Idler gear (for III/IV PTO)
11.3 III PTO (replacement)
11.4 IV PTO (replacement)
11.5 Balancer shafts (replacement)

11. 6 ETB (replacement)

## Assembly recommendations

- The information is laid out in sequence, the intervention methods have been selected, tested and approved by the manufacturer's technicians.

- This chapter describes the installation procedures for the assemblies and/ or individual components which have already been checked, overhauled or possibly replaced with original spare parts.

- Where necessary, reference to special tools during assembly operations is indicated and identified in Tab 13.1 , hereinafter in Tab. 9.1 an example of a special tool ( ST_05 ).

Tab. 9.1

| SPECIAL TOOLS |  |  |  |
| --- | --- | --- | --- |
| "ST" Code | Picture /draw | DESCRIPTION | PART NUMBER |
| ST_05 |  | Six nicks Key SN 8 | ED0014603650-S |

Important

- Before proceeding with operations, read Par. 3.3.2 .

- To easily locate specific topics, the reader should refer to the analytical index or chapter index .

- The operator must check that:

  - the components, the assemblies, the coupling surfaces of the parts are washed, clean and thoroughly dried;

  - the coupling surfaces are undamaged;

  - the equipment and tools are ready so that all work can be carried out correctly and safely;

  - ensure that the working environment is safe.

- The operator must:

  - carry out the procedures smoothly and safely. It is thus recommended to install the engine on a special rotating stand used when servicing engines to ensure the safety of the operator and the other individuals involved;

  - tighten the assemblies and / or components in a criss-cross or alternating pattern, initially with a value lower than that preset, and then subsequently, with the tightening torque specified in the procedure;

  - replace all seal gaskets after each assembly for all components on which they are provided.

## Engine block assembly

| 9.3.1 Crankshaft bushings Important - Execute the procedure in Par. 8.2.1 and 8.2.2 , before proceeding with assembly. - The crankshaft half-bearings are made of special material. Therefore, they must be replaced every time they are assembled to prevent seizures. - Fit the new half-bearings A1 onto the crankcase upper half B1 adhering to the reference notches C . Importante - After the half-bearings are fitted, check that the lubrication holes D correspond with the crankcase grooves B1 . - The lower and upper half bearings CANNOT be singularly replaced, and both halves must be replaced together. - Fit the new half-bearings A2 onto the lower crankcase B2 using the reference notches C . - Lubricate the half-bearings A1 and A2 with oil. | Fig 9.1 Fig 9.2 |
| --- | --- |
| 9.3.2 Tappets - Lubricate the tappets E with oil. - Insert the tappets E into the housings F of the upper crankcase B1 . | Fig 9.3 |
| 9.3.3 Oil spray nozzles - Insert the sprayers G onto the upper crankcase B1 manually screwing the screw fittings H . - Ensure that the spray nozzles G are inserted correctly in their seat, as shown in detail L and tighten the capscrews of union H (tightening torque of 10 Nm ). | Fig 9.4 |
| 9.3.4 Crankshaft Important - Carry out the checks described in Par. 8.4.1 and Par. 8.4.2 . - Check that the crankshaft half-bearings A1 are mounted correctly on the upper crankcase B1 . - Lubricate the main journal and crankpin J , with oil. - Insert the crankshaft M into its seat on the upper crankcase B1 . - Insert the 2 shoulder half-rings N1 , between the crankshaft M and the upper crankcase B1 ( Q detail). | Fig 9.5 |
| 9.3.5 Lower semi-crankcase - Check that the coupling surfaces P are free from dirt and grit. - Spread a bead of Loctite 5660 of approx 1,5 mm thickness on the surface P of the upper crankshaft half B1 being careful not to block the oil feed grooves X and the return oil sump Y . - Insert gasket S into the seat of crankcase B1 . Note :alternatively apply Loctite 5699 . | Fig 9.6 |

| - Check that the crankshaft half-bearings A2 are mounted correctly on the lower crankcase B2 . - Assemble the 2 shoulder half-rings N2 onto the lower crankcase B2 applying two drops of grease to keep them in their seat. - Join the two crankshaft halves B1 and B2 observing the guide pins T . |  |
| --- | --- |
| Fig 9.7 - F ig 9.8 |  |

| Fig 9.9 | Fig 9.10 |
| --- | --- |
| Tab 9.2 CYCLE; SCREWS; TORQUE. 1; J - Torx M14x1,5; 60 Nm. 2; K - Torx M10x1.25; 30 Nm. 3; J - Torx M14x1,5; 45°. 4; J - Torx M14x1,5; 45°. Important - The fastening bolts J , K must be replaced every time they are assembled. - Failure to adhere to the bolt fixing procedures may compromise the functionality of the engine, and also may cause damage to persons and property. - Tighten capscrews J , K observing the cycles, tightening, and subsequent rotation as indicated in Tab. 9.2 . - Apply " Molyslip AS COMPOUND 40 " on the threads and under the head of capscrews J and K and manually tighten them until their stop. - Tightening the screws J , K strictly following the sequence indicated in the Fig. 9.9 or Fig. 9.10 and the tightening torque indicated in the Tab. 9.2 . - Check that crankshaft M rotates smoothly. - Insert gasket W into the seat of crankcase B ( ST_47 ) . | Fig 9.11 |
| 9.3.6 Camshaft - Check that the pin P1 is correctly fitted on the crankshaft M . - Position the gear M1 on the crankshaft M respecting the reference with pin P1 . - Fully tighten the screw N1 . - Lubricate the pins S2 , the cams S3 of the camshaft S1 , all the housing Q1 with oil. - Insert the camshaft S1 all the way into its housing Q1 . - Position the gear R1 observing all the marks T1 of the gears M1 . Important - Failure to comply with the marks T1 on the gears M1 and R1 causes engine malfunction and serious damage. 7. Check that crankshaft M rotates smoothly. | Fig 9.12 Fig 9.13 |
| 9.3.7 Piston rings - Perform the operations described in Par. 8.5.3 . - Put the scraper ring Z3 onto the piston Z . - Put the 2° seal ring Z2 on the piston Z . - Put the 1° seal ring Z1 onto the piston Z . - Perform the operations described in Par. 8.5.4 . - Position the segment openings with a 120° angle between them ( Y ). NOTE: do not use the segment opening with the pin hole ( N ) - Lubricate the piston skirt and piston rings with oil. | Fig 9.14 Fig 9.15 |
| 9.3.8 Piston Importante - The fastening bolts E1 must be replaced every time they are assembled. - Before proceeding to the assembly of the piston and connecting rod, carry out the checks described in Par. 8.5.1 . - Always replace the bearings D1 after each assembly. - Mate components respecting references at Par. 7.15.5 . - Loosen the screws E1 and remove the connecting rod cap F1 . - Insert the connecting rod F2 into the piston Z and align the seats G1 . - Insert the gudgeon pin H1 into the seat G1 for the assembly of the connecting rod and piston unit. - Insert the lock rings L1 inside the seat G2 of the piston Z to lock the gudgeon pin H1 . | Fig 9.16 Fig 9.17 |

| 9.3.9 Piston and connecting rod assembly Important - Before assembling the piston and connecting rod assemblies, execute the controls described in Par. 8.5.5 . - Rotate the crankshaft M by moving the crankpin J1 to a TDC position of the affected cylinder. | Fig 9.18 |
| --- | --- |
| - Lubricate the piston skirt and rings Z . - Check that the half-bearing U1 is mounted correctly and lubricate it thoroughly. - Using the piston ring compression pliers, insert the piston inside the cylinder W1 by around 10mm (height T2 ). Important - Make sure you are at the stage described in Point 1 . - Piston Z must be assembled with notch K1 on the side of the skirt facing oil spray nozzles G . - Rotate the piston Z by 10° counter-clockwise with respect to its correct assembly position (Fig. 9.20 - height T3 ). NOTE: Doing this prevents the impact between the connecting rod F2 and the sprayer G . | Fig 9.19 Fig 9.20 Fig 9.21 Fig 9.22 |
| Important - Leave the ring compressor assembled on the piston. - Push piston Z downwards without introducing the segments in the cylinder, rotate piston Z by 10° in a clockwise direction (value T3 – correct assembly position). | Fig 9.23 |

| - Push the piston Z downwards by centering the crankpin J1 with the connecting rod F2 . - Rotate the crankshaft M by moving the crankpin J1 to a BDC position of the affected cylinder. - Push the piston Z downwards by centering the crankpin J1 with the connecting rod F2 . - Turn the crankcase on support to assemble the con rod capp F1 . - Check that the half-bearing U1 is mounted correctly on the connecting rod cap F1 . Important - Check that the break levels of connecting rod cap F1 coincide perfectly onto connecting rod F2 before screwing on and tightening capscrews E1 . - Couple the connecting rod cap F1 to the connecting rod F2 using the marks made at disassembly ( Par. 7.15.2 and 7.15.5 ). - Apply "Molyslip AS COMPOUND 40" on the threads and under the head of capscrew E1 and manually tighten them until their stop. Important - Failure to adhere to the assembly procedures may compromise the functionality of the engine, and also cause damage to persons and property. - Tighten the screws E1 , alternately, strictly following the tightening torques indicated ( Tab. 9.3 ). - Repeat the operations from 1 to 14 for each cylinder. - Check that the connecting rods have axial play and the crankshaft M rotates smoothly. NOTE: After the check carried out at point 16, position the shaft M with the first cylinder to TDC. | Fig 9.24 Fig 9.25 Fig 9.26 |
| --- | --- |
| Tab 9.3 CYCLE; SCREWS; TORQUE. 1; E1; 28 Nm. 2; E1; 30°. 3; E1; 30°. |
| NOTE : Click by side to play the procedure. | https://www.youtube.com/embed/V4aXYc_0x8U?showinfo=0&rel=0 |

## Oil sump unit assembly

| 9.4.1 Oil drain pipe Important - It is mandatory to replace the gasket D after each assembly. - Always replace capscrews B with new ones or alternatively apply Loctite 2701 . - Secure the hose A on the crankcase C with the screws B inserting the gasket D (tightening torque 10 Nm ). | Fig 9.27 |
| --- | --- |
| 9.4.2 Oil suction pipe Important - It is mandatory to replace the gasket F after each assembly. - Always replace capscrews B with new ones or alternatively apply Loctite 2701 . - Secure the hose E on the crankcase C with the screws B (tightening torque 10 Nm ) fitting the gasket F . | Fig 9.28 |
| 9.4.3 Oil Sump - Ensure that the contact surfaces G of the oil sump H and the crankcase C are completely clean. - Apply a bead of approx. 2.5 mm of sealant ( Loctite 5660 ) on the surface G of the crankcase C . Note : alternatively apply Loctite 5699 . | Fig 9.29 |
| - Position the oil sump H on the crankcase C in line with the fastening holes (use the aid of tool ST_18 ). | Fig 9.30 |
| Important - Tighten the screws L , strictly following the sequence and tightening torque indicated. - Secure oil sump H by means of capscrews L . - After tightening of the screw n° 10 , loosen screw n°1 and re-tighten it to the torque value specified in step 4 . | Fig 9.31 |

## Cylinder head unit assembly

| 9.5.1 Valve stem gasket Important - Carry out the checks described in Par. 8.6.4 before proceeding with the following operations. - Always replace gasket A with every assembly. - Lubricate the oil seals A on the inside. - Fit the oil seals A on the valve guides B using the tool ST_08 . | Fig 9.32 |
| --- | --- |
| 9.5.2 Electronic injector sleeves ( ) - Insert the seals C in the seats of the sleeve D . - Insert the seal E with the convex side facing upward at the base of the sleeve D . - Lubricate the gaskets C . - Insert and carefully screw the sleeve D into the seat of the head F . NOTE: The sleeve D must not protrude above the surface of the head G . - Clamp the sleeve D (tightening torque at 30 Nm ). | Fig 9.33 |
| 9.5.3 Electronic injectors projection - Perform the operations of point 1 and 2 Par. 6.1.4. - Perform the operations of point 3 and 4 Par. 6.1.5. - Check using ST_03 tool (Fig. 9.35) , the projection of the injector, which must range between 1.68 ÷ 2.42 mm. NOTE : if the value detected does not correspond, replace gasket Q with a different thickness. | Fig 9.34 - Fig. 9.35 |
| 9.5.4 Valves - Pre-lubricate and insert the valves X into the head F taking care to fit them in the original positions as per the reference marks made in Par. 7.12.4.1 . - Position the spring Y on the seat of the head F . - Position the disk S on the spring Y centering the valve X . - Mount the tool ST_07 on the head F fixing it on one of the holes for securing the rocker arm cover. NOTE: Change the fixing hole according to the position of the valves to be fitted. - Position the tool ST_07 on the valve as shown in the Fig. 9.37 . - Push the lever of the tool ST_07 downwards, in order to lower the valve disks S in the direction of the arrow AK , and insert the valve cotters AJ inside the disk S . - Check that the valve cotters AJ are properly mounted on the valve seats X and release the tool ST_07 . NOTE: repeat all the steps for the relevant valves and remove the tool ST_07 . | Fig 9.36 |
|  | Fig 9.37 |
|  | Fig 9.38 |
| 9.5.5 Cylinder head - Fix the eyebolts AW with the screws AX onto the head F (tightening torque of 80 Nm ). - Position the piston P at the TDC. - Position the tool ST_03 on the crankcase surface of the head and measure the piston protrusion P from head level K in 4 diametrically opposed points R. Repeat the operation for all pistons P and take note of the highest average value, determining valu e S (Tab. 9.4) . Tab. 9.42 S (mm); Hole number. 0.030 - 0.126; 1. 0.127 - 0.250; 2. 0.251 - 0.375; 3. - Based on the value detected at point 3 , select the relevant gasket T as shown in the Tab. 9.4 (Fig. 9.41 detail U ). - Check that the crankcase surface K and the gasket T are completely free of dirt and grit. Important - The head gasket must be replaced for each assembly. - Position the gasket T on the surface K with reference to the centering bushings J . | Fig 9.39 Fig 9.40 Fig 9.41 |
| - Check that the surface head W is free from impurities. - Position the head F on the crankcase Z with reference to the centering bushings J . Important - The fastening bolts V must be replaced every time they are assembled. Modified component, see service letter 710009. - Failure to adhere to the bolt fixing procedures may compromise the functionality of the engine, and also may cause damage to persons and property. - Tighten capscrews V observing the cycles, tightening, and subsequent rotation as indicated in Tab. 9.5 . - Secure the head F by tightening the screws V strictly following the sequence indicated in the Fig. 9.43 and the tightening torque and pauses between cycles indicated in the Tab. 9.5 . | Fig 9.42 |
| Tab. 9.5 CYCLE; TORQUE; PAUSE. 1; 75 Nm; 3min. 2; 90°; 3min. 3; 90°; 3min. 4; 90°; ---. | Fig 9.44 |
| 9.5.6 Rods and valve bridges - Insert the rocker control rods AA into the niches of the head F . Important - Properly centre the rods AA into the spherical housing of the camshaft tappets AB . - Mount the valve bridge AC on to the pairs of discharge and suction valves. | Fig 9.45 |
|  | Fig 9.46 |
| 9.5.7 Rocker arms Important - The discharge rocker arm AT is shorter than the suction arm AR . - Fit the lock ring AM into the seat AN of the rocker arm pin AH . - Position the pin AH with the screw support surface AP facing upwards and insert the 2 shoulder rings AQ . - Insert in sequence the suction rocker arm AR , the holder AS and the discharge rocker arm AT in the pin AH . - Insert the spring AU in the pin AH . - Repeat points 3, 4 for all the rocker arms. NOTE: Support AV , which contains taper pin BV , must be assembled in correspondence with cylinder n° 3 . - Insert 2 shoulder rings AQ and the lock ring AN to lock all the components inserted in the pin AH . NOTE : The spring AU ensures that the supports AS and AV are kept in place. | Fig 9.47 Fig 9.58 |
| 9.5.8 Rocker arm pin assembly Important - Position the rocker arm pin assembly BB on a level to align all the support surfaces. - Check that the pistons are positioned half way between the TDC and BDC. As seen from A ⇒ ( Par. 1.4 ) turn the crankshaft anticlockwise by 90°, complying with TDC of the 1st cylinder , positioning taper pin BP of the crankshaft as shown in Fig. 9.48 . - If the engine is painted or protected with clear paint, replace the fastening screws BE . - Position rocker arm shaft unit BB on cylinder head F , complying with the taper pin BC reference with hole BF of cylinder head F . - Check the correct positioning of all the rocker arms and the u-bolt control valves (detail BD ). House the tappet in the seat of the rocker arms control rod. - Secure the rocker arm pin BB tightening the screws BE (tightening torque to 40 Nm ). Adhere to the screw tightening sequence BE as shown in Fig. 9.50 . | Fig 9.48 Fig 9.49 |
|  | Fig 9.50 |
| 9.5.9 Assembly Rocker arm cover Important - Replace gasket BF, BL and BM with each assembly ( ST_11 - ST_12 apply Loctite 480 to the seats of cap BN before assembling the gaskets). Modified component, see service letter 710017. - Observe the order of tightening illustrated in Fig. 9.52 . - Position tool ST_17 onto the head in correspondence with the two fastening holes 9 and 10 . - With vaseline lubricate the gaskets BL in the upper part, and the gaskets BM in the lower part. - Position gasket BF and the rocker arm cover BN on cylinder head F using tool ST_17 as a guide. - Attach the rocker arm cover BN on the head F with the screw BG (tightening torque to 10 Nm ). | Fig 9.51 |
|  | Fig 9.52 |

## Intake manifold assembly

| 9.6.1 Internal half-manifold Important - Check that the contact surfaces between the semi-collector C and the head D are free from impurities. - Insert the special tool ST_18 into indicated point. - Position gasket B and manifold C on cylinder head D . - Secure the semi-collector C with the screws A on the head D (tightening torque of 25 Nm ). | Fig 9.53 |
| --- | --- |
| 9.6.2 External half-manifold Important - Check that the contact surfaces between the two semi collectors C and D are free from impurities. - Insert the special tool ST_18 into indicated point. - Position gasket N , panel P and semi-manifold M onto semi-manifold C . - Fit the semi-collector M on the semi-collector C with the screws L (tightening torque of 22 Nm - ST_05 ). | Fig 9.54 |

## Fuel system assembly

| Important - Do NOT mount new or different injectors without the required tool ( Chap. 13 ). - Remove the protective caps from all the components of the fuel circuit just before assembly just before assembly ( Par. 2.9.8 ). |  |
| --- | --- |
| 9.7.1 High-pressure injection pump - Check that the surface A is free from impurities ( Fig. 9.56 ). Important - Always replace gasket B with every assembly. - The seal gasket B can only be fitted in one direction ( Fig. 9.55 ). - Always replace capscrews C with new ones or alternatively apply Loctite 2701 ( Fig. 9.55 ). - Fit the new gasket B on the injection pump D ( Fig. 9.56 ). - Fix the pump D into the housing A1 together with the gasket B by the screws C ( Fig. 9. 56 - tightening torque at 25 Nm ). - Check the correct fitting of the key E on the shaft F of the injection pump D ( Fig. 9.57 ). - Place the gear G on the shaft F of the pump D respecting the reference to the key E and the reference H of the gear L ( Fig. 9.57 ). - Tighten the nut M (tightening torque at 140 Nm ). | Fig 9.55 |
|  | Fig 9.56 |
|  | Fig 9.57 |
| 9.7.2 Fuel filter - Secure the fuel filter holder N with the screws P on the crankcase Q (tightening torque of 25 Nm ). NOTE : For the assembly of the fuel cartridge, refer to operations 4 and 5 of Par. 6.9.2 . | Fig 9.58 |
| 9.7.3 Electronic injectors Importante - Always replace and lubricate the gaskets R of the electronic injectors S with fuel, every time they are assembled. - Pay attention when repositioning the electronic injectors, using the marks as described in Par. 7.10.5 . - If a new (or different) electronic injector is fitted on the engine, you are required to prepare tool ST_01 . - If the engine is painted or protected with clear paint, clean the paint off the diesel injector S near to the part in contact with the gasket AB . - Assemble gasket T onto electronic injector S . - Insert the electronic injectors S inside the rocker arm cover U being extra careful not to damage gasket AB and direct it as indicated in Fig. 9.59. | Fig 9.59 |
|  | Fig 9.60 |
| 9.7.4 Common Rail - Secure the rail W on the intake manifold X with the screws V (tightening torque at 25 Nm ). | Fig 9.61 |
| 9.7.5 High pressure fuel pipes Important - Always replace the pipes Y and tube Z after each assembly. - Position the pipes Y on the Common Rail W and on the electronic injectors S , adjust the position of electronic injectors S via the fitting inlets with the pipes Y . Important - Tighten the nuts J and K manually, without clamping them. - If the engine is painted or protected with clear paint, replace the fastening screws B1 . - Position the injector fastening brackets A1 and the screws B1 , inserting the washer C1 . Important - Replace the pipes Y ( Fig. 9.62 ) if the screws B1 do not rotate freely. - Tighten all the nuts K (tightening torque at 30 Nm ). - Tighten the nuts J (tightening torque at 25 Nm ). - Make sure that the mounting brackets A1 are positioned correctly on electroinjectors S and on fixing screws of the rocker arm assembly D1 . - Tighten the fixing screws B1 of the injector mounting bracket (tightening torque of 20 Nm ). - Position the pipe Z screwing the nuts J and K . Important - Screw the nuts J and K manually without tightening them. - Tighten the nut K (tightening torque of 30 Nm ). - Tighten the nut J (tightening torque of 25 Nm ). - Fasten the clamp E1 with the screw F1 (tightening torque of 10 Nm ). | Fig 9.62 |
|  | Fig 9.63 |
|  | Fig 9.64 |
| 9.7.6 Fuel flow pipes - Insert the pipes G1 on the fitting coming out of the filter holder N and on the fuel inlet fitting of the injection pump D . | Fig 9.65 |
| 9.7.7 Fuel return pipes - Check the gaskets H1 on the fittings J1 . NOTE : Do not disconnect the pipes from the distributor K1 . - Position the return pipes and fitting the distributor K1 with the screw L1 on the intake manifold X ( Fig. 9.67 - tightening torque of 10 Nm ). - Mount the fittings J1 ( Fig. 9.67 ) on the injectors S and lock them with the clips M1 . - Insert the pipe N1 on the fitting P1 . - Fit the gaskets Q1 and the fitting R1 on the screw S1 . - Tighten capscrews S1 on Common Rail W (tightening torque 15 Nm ) with the opening of union R1 facing upwards. | Fig 9.66 |
|  | Fig 9.67 |
|  | Fig 9.68 |

## Assembly lubrication circuit

| 9.8.1 Assembly oil mist separator unit Important - Always carefully inspect the condition of the pipes, and replace them if there is any doubt regarding the integrity of their seal. - Secure the plate A using the screws B (tightening torque of 10 Nm - ST_05 ). - Assemble clamps C onto tube D . - Secure tube D by means of fastening clamp C with capscrews E , inserting clamp F. - Fit hose G onto union H . - Secure tube J by means of capscrew K , inserting gasket L . - Secure the clamp M . - Perform the operations of Par. 6.7.2 . | Fig 9.69 |
| --- | --- |
|  | Fig 9.70 |
|  | Fig 9.71 |
| 9. 8 .2 Oil Cooler and oil filter Unit Assembly - Perform the operations of Par. 6.8.3 . NOTE :To replace the oil cartridge, refer to operations of Par. 6.8.2 . |  |
| 9. 8 .3 Oil pressure relief valve - Lubricate the piston N and fully insert it in the seat P . - Insert the spring Q in the piston N . - Insert disk R onto spring Q . - Insert cotter pin S in the provided seat of oil pump T to lock components N, Q and R . | Fig 9.72 |
| 9. 8 .4 Oil pump NOTE: Carry out the checks described in Par. 8.7 before proceeding with the following operations. - Check that all contact surfaces between T, V are free of impurities – scratches - dents. - When assembling, do not use any type of gasket between T and V . - Thoroughly lubricate the seat of the rotors on oil pump T . - Make sure the external rotor is assembled correctly with Ref. U visible, as shown in the picture (or refer to Par. 2.10.2 ). - Fasten the oil pump cover T on the crankcase V with the screws X (tightening torque 10 Nm ). | Fig 9.73 |
|  | Fig 9.74 |

## Flange unit assembly

| 9.9.1 Bell housing Danger - Bell A is very heavy; pay special attention during assembly operations to avoid dropping and causing serious risks to the operator. Important - Failure to adhere to the assembly procedures may compromise the functionality of the engine, and also cause damage to persons and property. - Always replace and lubricate gasket C with oil with each assembly (gasket C is to be mounted after the operation in point 5 ( ST_47 ) ). - To assemble components P, Q, R, S , and T , proceed with the operations described in Par. 11.2.2 - 11.3.2 - 11.4.2 - 11.5.2 . - Apply a bead of approx. 2.5 mm of sealant ( Loctite 5188 ) on the surface B of the bell A . - Ensure that bearing J is correctly assembled on camshaft K . - Assemble bell A onto crankcase D , complying with reference taper pins E ( ST_45 ) . - Apply the screws F by hand without tightening them. - Tighten the screws F following the tightening sequence indicated (tightening torque 75 Nm ). | Fig 9.75 Fig 9.76A Fig 9.76B |
| --- | --- |

| 9.9.2 Flywheel Danger - Flywheel F is very heavy; pay special attention during assembly operations to avoid dropping and causing serious risks to the operator. - Loosen capscrews G and remove tool ST_41 . - Position flywheel H onto crankshaft L by means of tool ST_43 - ST_46 . - Apply " Molyslip AS COMPOUND 40 " on the threads and under the head of capscrews G and manually tighten them until their stop. - Secure flywheel H with capscrews G (tightening torque 60 Nm ). - Once again, tighten capscrews G (2 cycles with tightening torque 130 Nm ). | Fig 9.77 |
| --- | --- |

## Exhaust manifold assembly

| Important - Replace the metal gaskets A, B every time they are assembled. - In the event of mounting the studs C , fix ( 25 Nm tightening torque) with Loctite 2701 on the thread. - Gasket B must be assembled with the wording " TOP " visible and facing upwards. - Check that the contact surfaces D are free from impurities. - Insert the gasket B on the studs C . - Position manifold E onto cylinder head G by manually tightening capscrews F , inserting: - gaskets A between cylinder head G and manifold E ; - spacers H between capscrews F and manifold E . - Secure manifold E onto cylinder head G by means of capscrews F (tightening torque 25 Nm ). - Clamp the nuts L ( 25 Nm tightening torque). | Fig 9.78 |
| --- | --- |

## Crankshaft pulley assembly

| - Perform the operations from point 1 to 7 of Par. 6.6.2 . |  |
| --- | --- |

## Turbocharger Assembly

| Importante - Before proceeding, perform the operation described in Par. 2.18 . - Ensure that tube C is not clogged. - Always replace the gaskets A, B, Q at each assembly. - Remove the plastic or foam caps from the turbo compressor before assembling. - Check that the contact surfaces D are free from impurities deformations or cracks, otherwise replace the damaged component. - Position the turbo-compressor E on the bolts F on the manifold G . - Fasten the turbo-compressor E with the nuts H (tightening torque of 25 Nm ). - Fasten the pipe L with the screws M to the turbo-compressor E . - Fasten the pipe G with the screws N on the crankcase P . Importante - Always replace the gasket Q after each assembly. - Before assembly of the tube R , perform the operation described in Par. 2.18.2 - Point 2 . - Ensure that tube R is not clogged. - Fasten the pipe R with the fittings S on the turbo-compressor E and on the crankcase P (tightening torque of 15 Nm ). Insert the gaskets Q between: - S and R; - E and R; - P and R. | Fig 9.79 Fig 9.80 Fig 9.81 |
| --- | --- |

## Electric component assembly

| 9.13.1 Sensors and switches |  |
| --- | --- |
| 9.13.1.1 T-MAP Sensor - Fasten the sensor A with the screws B on the manifold C (tightening torque of 10 Nm - ST_06 ). | Fig 9.82 |
| 9.13.1.2 Coolant temperature sensor - Secure the sensor D onto the head E (tightening torque of 20 Nm ). | Fig 9.83 |
| 9.13.1.3 Oil Pressure Switch - Clamp the oil pressure switch F on the crankcase G (tightening torque at 35 Nm ). | Fig 9.84 |
| 9.13.1.4 Camshaft phase sensor disassembly - Measure the distance between the coupling surface P1 to gear teeth G1 (X1). - Measure the distance between the coupling surface P1 and the sensor surface S7 (Y1). - The difference between the 2 measurements determines the air gap value (Z1). The value (Z1) permitted must be a MIN of 0.2 mm and a MAX of 1.2 mm. Insert one spacers K2 based on the value (Z1) detected. NOTE : The calibrated spacers K1 have a thickness of 0.2mm. - Insert the shim K1 on the sensor S10. - Secure phase sensor S10 onto crankcase L using capscrew R2 (tightening torque 10 Nm). | Fig 9.85 |
| Fig 9.85A |  |
| 9.13.1.5 Speed sensor - Measure the distance from the coupling surface J to the external diameter of the phonic wheel ( X2 ). - Measure the distance between the coupling surface J and the sensor surface H ( Y2 ). - The difference between the 2 measurements determines the air gap value ( Z2 ). The value ( Z2 ) permitted must be a MIN of 0.2 mm and a MAX of 1.2 mm . Insert one or two spacers K based on the value ( Z2 ) detected. NOTE : The calibrated spacers K have a thickness of 0.2mm. - Secure the bracket M with the screws N inserting the washer S (tightening torque at 10 Nm ). - Insert the shim K on the sensor H . - Clamp the sensor H on the bracket M with the screw R (tightening torque at 10 Nm - ST_06 ). | Fig 9.86 Fig 9.87 |
| 9.13.1.6 Fuel filter water detection sensor - Lubricate and insert the gasket V on the fitting W . - Tighten the sensor W onto the cartridge Z (tightening torque of 5 Nm ). | Fig 9.88 |
| 9.13.2 EGR valve Important - Check that the contact surfaces between flange B and the head D are free from impurities. - Always replace the gasket A after each assembly. - Position gasket A onto cylinder head D . - Secure the flange B with the screws C on the head D (tightening torque of 10 Nm ). - Perform the operations of Par. 6.4.2 . | Fig 9.89 |
| 9.13.3 Alternator - Insert the washer E onto the screw F . - Insert the screw F onto the alternator G . - Secure the bracket H and the alternator G using the screws L1, F onto the crankcase L . 9.13.4 Starter Motor - Perform the operations to point 10 of Par. 6.6.2 . | Fig 9.90 |
| 9.13.5 Electric cabling - Position the cable holder N together with the cabling P on the rocker cap Q . - Screw the wiring holder N on the rocker cap Q with the screws R (tightening torque of 10 Nm - ST_06 ). - Mount the connectors C1 on the electronic injectors S1 . Warning - Slightly move wiring support N to check that there is no voltage in the electrical wire of connector C1 in correspondence with the outlet hole N1 . | Fig 9.91 |
| - Fit the connector C2 on the sensor S2 . | Fig 9.92 |
| - Fit the connector C3 on the sensor S3 . | Fig 9.93 |
| - Insert the connector C4 on the fuel intake valve S4 . - Insert the connector C5 on fuel temperature sensor S5 . | Fig 9.94 |
| - Fit the connector C6 on the sensor S6 . | Fig 9.95 |
| - Fit the connector C7 on the sensor S7 . - Insert the clamp P1 onto the crankcase M . - Fasten the clamp P2 with the screw T onto the crankcase M (tightening torque of 10 Nm ). | Fig 9.96 |
| - Fit the connector C8 on the valve S8 . - Fit the connector C9 on the sensor S9 . | Fig 9.97 |
| - Secure cable X on motor V by means of nut J . - Secure cable Y on alternator W by means of nut K . | Fig 9.98 |

## Coolant circuit assembly

| 9.14.1 Thermostatic valve Important - Always replace the gasket A after each assembly. - Check the condition of the seal gasket A and fit it on the thermostatic valve B . - Position the thermostatic valve B in the seat on the head C (detail D ). - Secure the cover E with the screws F on the head C (tightening torque of 10 Nm ). | Fig 9.99 |
| --- | --- |
| 9.14.2 Coolant pump Important - Always replace the gasket L every time it is assembled. - Secure the flange G with the screws H interposing the gasket L onto the crankcase M (tightening torque of 25 Nm ). - Perform the operations of Par. 6.5.2 . | Fig 9.100 |
| 9.14.3 Oil Cooler hoses - Secure hose N on Oil Cooler P and on crankcase M by means of clamps Q . - Position and secure hose R by means of clamp S on Oil Cooler P and on crankcase M . - Secure clamps T on manifold U by means of capscrews V in points X (tightening torque 10 Nm - ST_06 ). | Fig 9.101 |
| Fig 9.102 |  |

## EGR Circuit Assembly

| 9.15.1 EGR Cooler - Insert the fitting A1 of EGR Cooler B in the sleeve C of the EGR valve unit. - Position EGR Cooler B on the intake manifold D with the screws E ( ST_05 ). - Secure the fitting A1 with the clamp F1 to the sleeve C . - Secure hose G onto union A2 of EGR Cooler B by means of clamp F2 . | Fig 9.103 |
| --- | --- |
| - Fasten the pipe H with the screws L on the EGR valve unit M inserting the gasket N (tightening torque of 22 Nm - ST_05 ). - Fasten the pipe H with the screws P on EGR Cooler B inserting the gasket Q (tightening torque of 25 Nm ). | Fig 9.104 |
| - Fasten the pipe J on the intake manifold D with the screws R (tightening torque of 22 Nm - ST_05 ) inserting the gasket S . - Fasten the pipe J on the EGR Cooler B with the screws T (tightening torque of 25 Nm ) inserting the gasket U . - Fit the EGR Cooler B on the intake manifold D with the screws E (tightening torque of 22 Nm - ST_05 - Fig. 9.103 ). | Fig 9.105 |

## Tightening torques and the use of sealants

Tab. 9.4 - *Alternatively to the capscrew replacements, with "Dri-loc"

| BASE CONFIGURATION |  |  |  |
| --- | --- | --- | --- |
| SHORT BLOCK |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Oil sprays fastening capscrew | M6x1 | 10 |  |
| Lower crankcase fastening capscrew | M14x1.25 | 3 Torque cycles |  |
| 1st Cycle |  | 60 |  |
| 2nd Cycle |  | +45° |  |
| 3rd Cycle |  | +45° |  |
| Lower crankcase fastening capscrew | M10x1.25 | 30 |  |
| Connecting rod screw | M11x1 | 3 Torque cycles |  |
| 1st Cycle |  | 28 |  |
| 2nd Cycle |  | +30° |  |
| 3rd Cycle |  | +30° |  |
| Coolant drain hole closing cap | M16x1.5 | 50 |  |
| Main oil delivery line closing plate | M6x1 | 15 |  |
| Intermediate idle gear cap fastening screw | M8x1 | 25 |  |
| OIL SUMP ASSEMBLY |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Oil suction hose fastening capscrew | M6x1 | 10 | Loctite 2701* |
| Oil return pipe fastening screw | M6x1 | 10 | Loctite 2701* |
| Oil sump fastening capscrew | M8x1 | 25 |  |
| Oil drain cap | M18x1.5 | 30 |  |
| FLANGE ASSEMBLY (1st PTO) |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Flange bell fastening capscrew | M12x1,75 | 75 |  |
| Flywheel fastening capscrew | M12x1,25 | 3 Torque cycles |  |
| 1st Cycle |  | 60 |  |
| 2nd Cycle |  | 130 |  |
| 3rd Cycle |  | 130 |  |
| ENGINE CYLINDER HEAD ASSEMBLY |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Air bleeding cap (Rev. 00) | M6x1 | 6 |  |
| Air bleeding cap (Rev. 01) | M14x1,5 | 50 |  |
| Lifting brace fastening capscrew | M8x1.25 | 80 |  |
| Injector manifold | M12x1 | 30 |  |
| Cylinder head fastening capscrew | M12x1.25 | 4 Torque cycles |  |
| 1 st Cycle |  | 75 |  |
| 2 nd Cycle |  | +90° |  |
| 3 rd Cycle |  | +90° |  |
| 4 th Cycle |  | +90° |  |
| Rocker arm gudgeon fastening capscrew | M8x1,25 | 40 |  |
| Rocker arm cover fastening capscrew | M6x1 | 10 |  |
| INJECTION SYSTEM |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Fuel filter fastening capscrew | M8x1.25 | 25 |  |
| Fuel cartridge fastening | ... | 17 |  |
| Common rail fastening capscrew | M8x1.25 | 25 |  |
| Electronic injector brace fastening capscrew | M8x1.25 | 20 |  |
| Distributor fastening capscrew | M8x1.25 | 25 |  |
| Waste line fastening drilled capscrew on common rail | M10x1 | 30 |  |
| Injector side injection tube nuts | M12x1.5 | 25 |  |
| Injection pump side injection tubes nuts | M12x1.5 | 25 |  |
| Common Rail side injection tubes nuts | M14x1.5 | 30 |  |
| Injection pump fastening capscrew | M8x1.25 | 25 | Loctite 2701* |
| Gear fastening nut on high-pressure fuel injection pump | M14X1.5 | 140 |  |
| Screw for cover over injection pump shaft nut (on bell housing) | M6x1 | 10 |  |
| INTAKE MANIFOLD |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Internal semi-manifold fastening capscrew (on cylinder head) | M8x1.25 | 25 |  |
| External semi-manifold fastening capscrew | TG8 | 22 |  |
| Intake flange fastening capscrew | TG8 | 22 |  |
| EXHAUST MANIFOLD |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Exhaust manifold fastening screw | M10x1.5 | 50 |  |
| Exhaust manifold fastening nut | M10x1.5 | 50 |  |
| Exhaust manifold fastening stud | M10x1.5 | 2 Torque Cycles |  |
| 1 st Cycle |  | 40 |  |
| 2 nd Cycle |  | 80 |  |
| LUBRICATION CIRCUIT |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Oil vapour separator support plate fastening capscrew | TG8 | 22 |  |
| Oil vapour separator support fastening capscrew (on crankcase) | M6x1 | 12 |  |
| Oil steam separator return tube drilled fastening screw (on cranckase) | M16x1.5 |  |  |
| Oil filter fastening union | M20x1.5 | 15 | Loctite 2701* |
| Oil cooler fastening capscrew | M6x1 | 10 |  |
| Cartridge-holder cover | ... | 25 |  |
| Oil pump fastening screw | M6x1 | 10 |  |
| CRANKSHAFT AND TARGET WHEEL PULLEY ASSEMBLY (2nd PTO) |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Phonic wheel fastening capscrew (on crankshaft pulley) | M6x1 | 10 |  |
| Crankshaft pulley fastening capscrew | M12x1.75 | 100 | Molyslip |
| COOLANT CIRCUIT |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Coolant tube clamp fastening capscrew (Oil Cooler return) | TG8 | 22 |  |
| Thermostatic valve cover fastening capscrew | M6x1 | 10 |  |
| Coolant pump fastening capscrew | M8x1.25 | 25 |  |
| Blower fastening capscrew | M8x1.25 | 25 |  |
| TURBOCHARGER |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Oil return tube fastening capscrew | M6x1 | 10 |  |
| Oil supply tube fastening capscrew | M10x1 | 15 |  |
| Turbine fastening stud (on manifold) | M10x1.5 | 25 |  |
| Exhaust flange fastening stud (on turbine) | M8x1.25 | 25 |  |
| Turbine fastening stud | M10x1.5 | 30 |  |
| Exhaust flange fastening nut (on turbine) | M8x1.25 | 25 |  |
| ELECTRICAL COMPONENTS |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| MAP sensor fastening capscrew | M6x1 | 10 |  |
| Coolant temperature sensor | M12x1.5 | 20 max. |  |
| Oil pressure switch | M12x1.5 | 35 |  |
| Phase sensor fastening capscrew | M6x1 | 10 |  |
| Speed sensor fastening capscrew | M6x1 | 10 |  |
| Sensor for water presence in fuel |  | 5 |  |
| Alternator fastening capscrew | M10x1.5 | 45 |  |
| Alternator fastening capscrew | M8x1.25 | 25 |  |
| Alternator brace fastening capscrew | M12x1.75 | 75 |  |
| Starter motor fastening capscrew | M10x1.5 | 45 |  |
| Supply cable fastening nut (starter motor) | M10x1.5 | 15 |  |
| EGR CIRCUIT |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Flange EGR valve fastening capscrew | M8x1.25 | 25 |  |
| EGR valve fastening capscrew | M6x1 | 10 |  |
| EGR Cooler tube fastening capscrew (on flange EGR valve) | TG8 | 22 |  |
| EGR Cooler fastening capscrew | TG8 | 22 |  |
| Tube fastening capscrew on EGR Cooler | M8x1.25 | 25 |  |
| Tube fastening capscrew on intake manifold | TG8 | 22 |  |
| SCR CIRCUIT |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| DCU fastening screw | M6 | 8 |  |
| ETB fastening screw | M6 | 10 |  |
| DEF heating valve fastening screw | M6 | 10 |  |
| DEF injector fastening screw | M6 | 8 |  |
| DEF pump fastening screw | M8 | 19 |  |
| Ambient temperature sensor | M12x1.5 | 20 |  |
| SCR temperature sensor | M14x1.5 | 45 |  |
| DEF draining screw (tank supplied by KOHLER) | .... | 20 |  |
| NOx Sensor | M20x1.5 | 60 | Castrol Optimol Paste MF o Bostik Never-Seez Grade |
| NOx control unit | ... | 3 |  |
| clamp fastening capscrew SCR | ... | 12 |  |

* Alternatively to the capscrew replacements, with "Dri-loc"

| OPTIONAL COMPONENTS (Chap. 11) |  |  |  |
| --- | --- | --- | --- |
| HEATER |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Flange intake with heater fastening capscrew | M8x1.25 | 25 |  |
| IDLE GEAR (FOR 3 rd ) |  |  |  |
| Component | Thread (mm) | Torque (Nm) | Sealer |
| Gear fastening capscrew | M8x1 | 25 |  |

* Alternatively to the capscrew replacements, with "Dri-loc"