Fluids filling information

KDI 1903TCR / KDI 1903TCRE5 Workshop manual (Rev. 17.8)

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Fluids filling information

## Engine oil

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Loosen the oil filler cap A or the oil filler cap C if the cap A is not accessible. - Add the type and amount of oil recommended ( Tab. 2.2 ). | Fig 10.1 |
| - Remove the oil dipstick B and check that the level is up to but does not exceed the MAX . - If the oil level is not at MAX , insert more oil until the MAX level is reached as indicated on the dipstick. - Re-tighten the cap A or C . NOTE: See the Par. 11.1 for the various configurations of the oil dipstick. | Fig 10.2 |
| NOTE : Click by side to play the procedure. | https://www.youtube.com/embed/cVpoy_m253A?showinfo=0&rel=0 |

## Coolant

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Tighten the cap G , replacing the copper gasket (Tightening torque of 50 Nm ). | Fig 10.3 |
| NOTE: A sealing plug or connecting pipe to the expansion tank may be present on the junction B , both should be secured with a strap. - Reinsert on the junction B the sealing plug or a connecting pipe to the expansion tank. | Fig 10.4 |
| Warning - Presence of steam pressurized coolant danger of burns. - Refill the radiator with coolant (refer to Par. 2.6 for the liquid specifications). - Top liquid up until the pipes inside the radiator are covered by about 5 mm. - For engines equipped with separate expansion tank, pour in fluid until reaching the max level mark. - Loosen the screw F on the head H , release any air and tighten the screw F (Tightening torque of 8 Nm - Fig. 10.7 ). - Start the engine without the radiator cap A or the expansion tank (C) cap B. - Keep the engine at idle speed or without a load until the coolant level goes down and becomes steady (the waiting times varies according to the ambient temperature). - Turn off the engine and allow it to cool. - If there is an expansion tank (C) top liquid up to the mark MAX . - Without expansion tank top liquid up until the pipes inside the radiator are covered by 5 mm. Do not overfill the radiator, but leave room for the coolant to expand. - Tighten the radiator cap A or the expansion tank (C) cap B. Warning - Before starting make sure that the radiator cap and expansion tank cap, if present, are installed correctly to avoid loss of liquid or vapour at high temperatures. - After a few hours of operation stop the engine and allow it to cool. Check and top up the coolant. | Fig 10.5 Fig 10.6 |
| Fig. 10.7 |  |
| NOTE : Click by side to play the procedure. | https://www.youtube.com/embed/S79xPhTZMps?showinfo=0&rel=0 |