Information about optional components

KDI 2504TCR / KDI 2504TCRE5 Workshop Manual (Rev. 17.8)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about optional components

## Oil dipstick in cylinder head

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.1.1 Check - Pull out the dipstick B in the direction of the arrow A . - Check that the mark left by the oil on the dipstick is between the MIN and MAX notches. | Fig 11.1 |
| 11.1.2 Replacement 11.1.2.1 Disassembly - Undo the screw D . - Pull out the oil dipstick hose E in the direction of the arrow F | Fig 11.2 |
| 11.1.2.2 Assembly Important - Always replace the gasket G every time it is disassembled. - Insert the gasket G in the seat K of the hose E . - Insert the hose E in the crankcase H . | Fig 11.3 |
| - Secure the oil dipstick hose E using the screw D on the manifold L (tightening torque at 10 Nm ). | Fig 11.4 |
| NOTE: Check the integrity of the gaskets J . - Insert the dipstick B inside the hose E . | Fig 11.5 |

## Heater (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.2.1 Disassembly - Undo the screws A and the relevant washers and remove the earth cable B . - Remove the flange C and the manifold D . - Remove the heater E and the relevant gaskets F . | Fig 11.6 |
| 11.2.2 Assembly Important - Always replace gaskets F , with each assembly. - In sequence, fit the manifold G with the gasket F , the new heater E , the second gasket F , the flange C , the washers H , the screws A and the cable B . - Secure the flange H with the screws A (tightening torque at 22 Nm ). - Secure the earth cable B with the nut J and the relevant washer on the heater E . | Fig 11.7 |

## Poly-V alternator belt (replacement and adjustment)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Loosen the nut B and manually tighten the screw C until it just touches the pulley pin D (Fig. 11.9) . | Fig 11.8 |
| - Untighten the screw E by around 32mm (A) . - Untighten the screw C . NOTE: The belt tensioner pulley F should move towards the arrow G . If it does not, please move it manually. | Fig 11.9 |
| - Remove the V-Belt H and install the new one. NOTE: Ensure that the internal profile of belt H is properly inserted into the grooves of the pulley A (as illustrated in D1 e D2 ). | Fig 11.10 |
| - Tighten capscrew C , to shift gudgeon D fully to the bottom of the grooved guide. - Tighten capscrew E (tightening torque at 45Nm ). - Hold the screw C still with a key, and tighten the screw B on the plate L to secure the screw C (tightening torque at 45Nm ). - Check, in point P (Fig. 11.8) ,the tension of the belt. Check by the appropriate tool that at point p the tension value is between 149 and 196 Hz . NOTE: After the engine has been in operation for around 15 minutes, repeat point 8 . | Fig 11.11 |

## Tightening pulley and alternator for Poly-V belt

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.4.1 Disassembly - Perform the operations from point 1 to 3 of Par. 11.3 . - Remove the belt H ( Fig. 11.10 ) . - Undo and remove the screw A . - Fully undo the screw B and remove the tightening pulley C . | Fig 11.12 |
| - Undo the screws D and remove the plate E and the pin F . | Fig 11.13 |
| - Undo the screws G and H remove the alternator L . | Fig 11.14 |
| - Undo the screws M and remove the bracket N . | Fig 11.15 |
| 11.4.2 Assembly - Secure the bracket N using the screws M on the cylinder head P (tightening torque at 25 Nm ). | Fig 11.16 |
| - Insert the screw H into the fixing hole on the alternator L . - Insert the spacer R on the screw H (between the alternator and crankcase). - Tighten the screw manually H onto the crankcase Q . - Orientate the second fixing hole of the alternator L with the hole of the bracket N , secure the alternator L using the screw G (tightening torque at 25 Nm ) onto the bracket N and then the screw H (tightening torque at 25 Nm ). | Fig 11.17 |
| - Insert the pin F in the plate slot E . - Orientate the pin F with the surface S (support for screw A ) upwards. - Secure the plate E using the screws D on the bracket N (tightening torque at 25 Nm ). | Fig 11.18 |
| - Insert the screw B in the plate C1 and pulley C . - Manually tighten the screw B onto the pin F up to the stop; Undo the screw B again by one turn. NOTE: The screw B must protrude by about 32 mm (A) from the surface of the tightening pulley C (see detail X ). - Install the new belt H (Fig. 11.10) . - Tighten the screw A onto the plate E up to the stop on the pin F . - Perform the operations from point 6 to 8 of Par. 11.3 . | Fig 11.19 |

## Idler gear (for 3rd / 4th PTO)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.5.1 Disassembly - Undo the screw A and remove the gear unit B . | Fig 11.20 |
| - Remove the retainer ring C from the seat of the pin D . - Remove the shoulder washer E , the gear B , the shoulder ring F and the bushing G from the pin G . 11.5.2 Assembly - Insert gudgeon D : - shoulder ring F (minimum shim) - gear B - shoulder ring E - retainer ring C . - Insert the bushing G on the crankcase L . | Fig 11.21 Fig 11.22 |
| Important - Always replace the washer H every time it is disassembled. Modified component, see service letter 700019 - 700021 - 700028 . - Check that the perforated screw A is free from impurities inside it. - Position the gear unit B on the hole J using the bushing G to centre. - Secure the gear unit B using the screw A inserting the washer H (tightening torque at see service letter 700019 - 700021 - 700028 ). | Fig 11.23 |

## 3rd PTO (replacement)

Fig 11.12

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.4.1 Disassembly - Undo the screws A and remove the pump B . | Fig 11.13 |
| - Remove the centring ring C and the relative gaskets. - Undo the screws N . | Fig 11.14 |
| - Remove the flange F with the components D, E, G and H in the direction of the arrow P . - Remove the gasket J . | Fig 11.15 |
| - Remove the retainer ring D and the shoulder washer E . - Remove the gear H and the shoulder ring G from the flange F in the direction of the arrow Q . | Fig 11.16 |
| 11.4.2 Assembly Important - Always replace the gasket J after each assembly. - Lubricate the gear H with oil. - Insert the gear H in the flange F in the direction of the arrow R inserting the shoulder ring G . - Insert the shoulder ring E on the flange F and clamp the gear H using the retainer ring D . - Position flange F on the crankcase K inserting the gasket J , and inser gear H in crankcase K . | Fig 11.17 Fig 11.18 |
| - Secure the flange F using the screws N (tightening torque at 25 Nm ). | Fig 1 |
| Important - Always replace rings P and Q after each assembly. 5. Insert the centring ring C in the flange F up to the stop. 6. Position the pump B on the flange F engaging the gear H . 7. Secure the pump B using the screws A on the flange F (tightening torque at 25 Nm ). | Fig 11.20 |

## 4th PTO (replacement)

Fig 11.21

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.7.1 Disassembly - Undo the screws A and remove the pump B . | Fig 11.22 |
| - Undo the screws C and remove the flange D . | Fig 11.23 |
| - Undo the screws E and remove the cover F . | Fig 11.24 |
| - Undo the screws G and remove the flange K with the components H, J, M, N and P . | Fig 11.25 |
| - Remove the retainer ring H and the shoulder ring J from the flange K . - Remove the gear N and the shoulder ring M from the flange K . | Fig 11.26 |
| 11..2 Assembly Important - Always replace the gasket J after each assembly. - Lubricate the gear H with oil. - Insert the gear N in the flange K in the direction of the arrow W inserting the shoulder ring M . - Insert the shoulder ring J on the flange K and clamp the gear N using the retainer ring H . | Fig 11.27 |
| - Position flange K onto crankcase Q inserting gasket P and insert gear N in crankcase Q . Important - Always change capscrews G with new ones or alternatively apply Loctite 2701 . - Secure the flange K using the screws G (tightening torque at 25 Nm ). | Fig 11.28 |
| Important - Always replace the gasket V after each assembly. - Insert gasket V on cover F , insert and position the cover F on flange K . - Secure the cover F using the screws E (tightening torque at 25 Nm ) on the flange K . | Fig 11.29 |
| Important - Always replace the gasket T after each assembly. - Position and tighten flange D by means of capscrews C on carter S (tightening torque 10 Nm - ST_06 ). | Fig 11.30 |
| Important - Always replace the gasket U after each assembly. - Position the gasket U on the flange D . - Secure the pump B using the screws A (tightening torque at 25 Nm ) on the flange D . | Fig 11.31 |

## 3rd + 4th PTO (configurations)

Fig. 11.45

| 11.8.1 Information Hydraulic pumps on the 3rd and 4th PTO can be installed at the same time. In some configurations, there is also the centering ring C on the 4th PTO. Important - For disassembly or installation, refer to Par. 11.5 , Par. 11.6 e Par. 11.7 . - Always replace the gasket of the rings B and C and flanges D and K at each assembly. - Lubricate the gear H with oil. | Fig. 11.46 |
| --- | --- |

## Balancer device (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.9.1 Disassembly - Perform the operations described in Par. 5.2 . - Undo the screws A and remove the oil sump B . | Fig 11.47 |
| - Undo the screws C and remove the hose D . | Fig 11.48 |
| - Undo the screws E and remove the shaft support box F . | Fig 11.49 |
| - Undo the screws G and remove the plate H . | Fig 11.50 |
| - Remove the shafts J and K in the direction of the arrow L from box F . | Fig 11.51 |
| 11.9.2 Assembly - Lubricate the bushings V with Molikote grease. - Insert the shafts J and K inside the box F in the direction of the arrow M . | Fig 11.52 |
| - Make sure that the shafts J and K i nside the box F observe the marks N and that the shaft J with the gear indicated by letter " S " stamped on it is on the left with respect to the box F . | Fig 11.53 |
| - Secure the plate H using the screws G on the box F (tightening torque at 8 Nm ). | Fig 11.54 |
| - Manually tighten the retainer screw ST_15 on the box F by slightly rotating the shaft K , centring the hole on it using the ST_15 , to lock the device. | Fig 11.55 |
| - Rotate the crankshaft and clamp it on the TDC (Ref. P upwards) using the tool ST_34 secured in place of the starter motor (detail Q ). | Fig 11.56 |
| - Position the box F on the surface of crankcase R observing the reference bushings. - Secure housing F using capscrews E and insert washers U (tightening torque at 50 Nm ). - Remove the retainer screw ST_15 from the box F . | Fig 11.57 |
| Important - Check that the retainer capscrew ST_15 (Fig. 11.55) is not present in point X on housing F . - Always replace the gasket W after each assembly. - Lubricate the gasket W with oil before assembling it. - Insert the gasket W in the seat on the flange of the oil hose D . - Secure the oil intake hose D using the screws C . - Perform the operations described in Par. 9.4.3 to assemble the oil sump. | Fig 11.58 |

## Air filter (cartridge replacement)

| - Release the two hooks A and remove the cover B from the body C . - Remove the cartridges D and E . Important - Before proceeding with operation, read Par. 3.3.2 . - The safety cartridge E (if present) must always be replaced if it is dirty or damaged. - Insert the new cartridge E inside the new cartridge D and both of them inside the filter body C . - Secure the cover B via the hooks A . | Fig 11.59 |
| --- | --- |

## Remote oil filter (disassembly and assembly)

| 11.11.1 Option A Important - Modified component, see service letter 700018 . 11.11.1.1 Disassembly - Perform the operations described in Par. 5.2 . Important - Before proceeding with operation, read Par. 3.3.2 . - For the replace the cartridge, please refer to operation n° 4 (Par. 11.11.1) and operation n° 7 (Par. 11.11.2) . - For the disassembly of the pipes B and C , lock with a tool the fittings K, H (Fig. 11.61) and L (Fig. 11.62) in order to prevent their lose together with the nuts A , with the consequent of oil leakage. - The threads of unions L and H have different features - Before removing the unions L and H , apply a distinguishing mark on them in order to reposition them correctly on the support M or on the head J during the assembly phase. - The threads of nuts A have different features - Before removing the tubes B and C, apply a distinguishing mark on them in order to correctly screw the nuts A to the unions L and H during the assembly phase. - Undo the nuts A and remove the hoses B and C . - Unscrew the fittings L and remove the copper gaskets from the support M . - Unscrew the cartridge N with gasket from the support M . | Fig 11.60 Fig 11.61 |
| --- | --- |
| - Release the clamps D and remove the hoses E and F from Oil Cooler G . - Unscrew and remove the fitting H with its copper gasket from the oil filter head J . - Unscrew and remove: - the fitting K with the copper gasket; - Oil Cooler G and the relative gasketsi; - the oil filter head J . | Fig 11.62 |
| 11.11.1.2 Assembly Important - Always replace the gaskets P, Q, R and W at each assembly. - Lubricate the gaskets P, Q and R with oil before assembling them. - Insert the gasket P on the seat of the fitting K . - Insert flange head J on the fitting K and the gasket Q in the seat of head J . - Insert the Oil Cooler G on the fitting K and the gasket R in the seat of Oil Cooler G . - Onto crankcase S apply Oil Cooler G and flange J by means of union K (tightening torque at 25 Nm + Loctite 2701 on thread) as positioned in Fig. 11.64. - Clamp union H on flange J inserting gasket U (tightening torque at 40 Nm ). | Fig 11.63 Fig 11.64 |
| Important - Always replace the gaskets V after each assembly. - Clamp unions L on support M inserting gasket V (tightening torque at 40 Nm ). - Lubricate gasket W and clamp cartridge N on support M (tightening torque at 20 Nm ). | Fig 11.65 |
| 8. Connect tube B to the central fitting of support M and of head J . 9. Connect tube C to the side fitting of support M and of head J . 10. Clamp the nuts A on the head J (tightening torque at 30 Nm ). 11. Clamp the nuts A on support M (tightening torque at 35 Nm ). Important - Check the tightening of the fittings K, H (Fig. 11.64) and L (Fig. 11.65) (tightening torque at 40 Nm ). | Fig 11.66 |
| 11.11.2 Option B 11.11.2.1 Disassembly - Perform the operations described in Par. 5.2 . - Perform the operations indicated in point 1 of Par. 7.3.2 . Important - Before proceeding with operation, read Par. 3.3.2 . - The oil filter cartridge may not be supplied by KOHLER (in such cases, refer to the machine’s documentation) - To replace only the cartridge, refer to operations 5 ( Par. 11.11.1 ) and 7 ( Par. 11.11.2 ). - to remove the tubes B and C , use a wrench to lock the unions L ( Fig. 11.70 ) and H ( Fig. 11.72 ) in order to prevent them from being loosened and removed together with the nuts A , with subsequent oil leaks. - The threads of unions L and H have different features - Before removing the unions L and H , apply a distinguishing mark on them in order to reposition them correctly on the support M or on the head J during the assembly phase. - The threads of nuts A have different features - Before removing the tubes B and C , apply a distinguishing mark on them in order to correctly screw the nuts A to the unions L and H during the assembly phase. | Fig 11.67 |
| - Undo the nuts A and remove the hoses B and C . - Unscrew the fittings L and remove the copper gaskets from the support M . - Unscrew the cartridge N with gasket from the support M . | Fig 11.68 Fig 11.69 Fig 11.70 |
| - Release the clamps D and remove the hoses E and F from Oil Cooler G . - Unscrew and remove the unions H together with the relevant copper gasket from the head J . - Unscrew and remove: - the union K with its gasket; - the head J ; - the Oil Cooler G with relevant gaskets; - the screws X and Y ; - the support T ; | Fig 11.71 Fig 11.72 |
| 11.11.2.2 Assembly Importante - Replace the gaskets H1 , J1 , J2 , K1 , L1 , T1 and Z1 at every assembly. - Lubricate the gaskets J1 , J2 , K1 , T1 and Z1 with oil before performing the assembly. - Insert the gasket K1 on the seat of the union K . - Insert the gaskets J1 and J2 on the seats of the union J . - Assemble on the support T : Oil Cooler G head J - Fix the union K to the support T , aiming the Oil Cooler G and the head J correctly (tightening torque of 25 Nm + Loctite 2701 on the thread). | Fig 11.73 |
| - Insert the gasket Z1 on the seat of the union Z . - Insert the gasket T1 on the seat of the support T . - Assemble on the support T on the crankcase S and fix it using the screws X and Y (tightening torque of 10 Nm ). - Fit the tubes E and F on the Oil Cooler G and fix them using the clamps D ( Fig. 11.71 ). | Fig 11.74 Fig 11.75 |
| - Fix the unions H to the head J placing the gasket H1 in-between (tightening torque of 40 Nm ). | Fig 11.76 |
| - Fix the unions L to the support M placing the gaskets L1 in-between (tightening torque of 40 Nm ). - Lubricate the gasket N1 and fix the cartridge N to the support M (tightening torque of 20 Nm ). | Fig 11.77 |
| - Connect the tubes B and C onto the unions L of the support M and H of the head J . - hten the nuts A on the head J (tightening torque 30 Nm ). - Tighten the nuts A on the support M (tightening torque 35 Nm ). - Perform the operations indicated in point 1 of Par. 9.15.3 .* | Fig 11.78 Fig 11.79 |

## Oil sump with supporting structure

| 11.12.1 Flywheel (J) disassembly - Execute the operations described in Par. 7.12.1 . 11.12.2 Plate/flange housing (L) disassembly - Loosen supplementary capscrews A and B . - Execute the operations described in Par. 7.12.2 . - Remove housing or plate L . | Fig. 11.80 |
| --- | --- |

| 11.12.3 Oil sump disassembly - Execute the operations described in Par. 5.2 . - Loosen capscrews C and remove bypass tube D . - Loosen capscrews E and remove oil sump F . | Fig. 11.81 |
| --- | --- |

| 11.12.4 Oil sump assembly - Make sure contact surfaces G of oil sump F and crankcase H have no impurities. - Apply a sealing bead of approximately 2.5 mm ( Loctite 5660 ) onto surface G of crankcase H . - Place oil sump F onto crankcase H in correspondence with the fastening holes (use tool ST_18 ). | Fig. 11.82 |
| --- | --- |

| Fig. 11.83 |  |
| --- | --- |

| - Apply capscrews E into the fastening holes and use torque at 10 Nm . - Loosen capscrews E , leaving approximately 1 mm leeway ( position A ) between the neck surface of capscrews E and oil sump F . - Place flange housing or plate L onto crankcase H , complying with centring tap pins M . - Using 2 capscrews A , fasten housing or plate L onto crankcase H (tightening torque at 20 Nm ). - Using 2 capscrews A , fasten housing or plate L onto oil sump F (tightening torque at 20 Nm ). | Fig. 11.84 |
| --- | --- |

| Fig. 11.85 |  |
| --- | --- |

| - Fasten oil sump F by tightening capscrews E and strictly following the order shown in Fig. 11.73 (tightening torque at 20 Nm ). - Loosen capscrews A and remove housing or plate L ( Fig. 11.72 ). - Fasten oil sump F by tightening capscrews E and strictly following the order shown in Fig. 11.73 (tightening torque at 47 Nm ). Loosen the screw 1 again and tighten it to 47 Nm . | Fig. 11.86 |
| --- | --- |

| - Insert gaskets N into seats P of bypass tube D . - Fasten bypass tube D onto oil sump F using capscrews C (tightening torque at 10 Nm ). | Fig. 11.87 |
| --- | --- |

| 11.12.5 Flange plate / housing assembly - Execute the operations described in point 6 of Par. 11.12.4 . - Fasten housing or plate L by using capscrews A and strictly following the order shown in Fig. 11.75 (tightening torque at 85 Nm ). - Fasten housing or plate L by using capscrews B (tightening torque at 270 Nm ). 11.12.6 Flywheel assembly - Execute the operations described in Par. 9.5.2 . | Fig. 11.88 |
| --- | --- |

## ETB (replacement)

| - Unscrew screws A and remove the ETB valve B with its gasket C . Important - Always replace the gasket C at each assembly. | Fig. 11.89 |
| --- | --- |
| - Fix the ETB valve B and its gasket C by means of screws A (tightening torque 10 Nm ) | Fig. 11.90 |

## ACACT (replacement)

| - Unscrew sensor A and its gasket B . Important - Before assembling the new sensor, see Par. 2.17.5 - Always replace the gasket B at each assembly. | Fig. 11.91 |
| --- | --- |
| - Fix sensor A and gasket B on their support C (tightening torque 20 Nm ). | Fig. 11.92 |

## EGTS (Black | Yellow - replacement)

| - Unscrew sensors A . Important - Before assembling the new sensor, see Par. 2.17.6 | Fig. 11.93 |
| --- | --- |
| - Fix sensors A on the ATS B (tightening torque 30 Nm ). | Fig. 11.94 |

## DPF & DOC filter (replacement)

| Danger - Highly carcinogenic material! - The dust contained inside the DPF is particularly fine and therefore classified as highly dangerous to living beings. - Before proceeding to any operation, wear: dust mask gloves goggles - Do not allow any other operator who is not equipped with the above mentioned protective equipment to approach. |  |
| --- | --- |
| - Obtain a new replacement KIT for the DPF filter or a replacement KIT with regenerated DPF filter from your spare parts service. Important - Before disassembling/assembling any sensors, see Par. 2.17.5 and 2.17.6 - Always replace the seal P upon every assembly. - During disassembly, do not expose the DPF to the surrounding environment for a long time and store it in a sealed bag as soon as possible. - During disassembly operations, avoid using electric screwdrivers as vibrations could release the dust inside the DPF - Do not try to clean the DPF - Do not blow with compressed air - Do not release the DPF or the dust contained in it in the environment - Dispose of the DPF only in authorised centres - Perform the operations indicated in point 1 of Par. 11.15 . - Open the package of the DPF replacement KIT, taking care not to damage it. - Unscrew screw A and loosen clamps B , then remove the Delta-P sensor D . - Disconnect pipes C from their unions J and remove the Delta-P sensor D . - Unscrew and remove unions J . | Fig. 11.95 |
| - Loosen clamps E and remove manifold N . | Fig. 11.96 |
| - Loosen clamp G and remove the DPF filter H . NOTE: do not remove clamp G . - Inspect the DPF filter to detect any visible signs of oil contamination. - Put the removed DPF filter in the plastic container supplied with the replacement KIT and send it to your spare parts service using the replacement KIT package. NOTE : a residual value will be recognised depending on whether the used DPF is intact and recyclable or damaged and requiring a correct disposal. | Fig. 11.97 |
| - Loosen the clamps Q and S and remove the manifold with DOC M from the flexible hose R . | Fig. 11.98 |
| - Fit the new manifold with the DOC filter M on the support T , inserting the flexible hose R inside the inlet of the manifold M . | Fig. 11.99 |
| Important - Do not apply any tension during the assembly of components. - Before fastening the manifold M , it must be oriented in the same position as the previously installed DOC filter. - Tighten the clamp Q (tightening torque of 10 Nm ). - Tighten the clamp S (tightening torque of 12 Nm ). | Fig. 11.100 |
| - Insert the gasket P on the manifold with DOC filter M . - Insert the new DPF filter H inside clamp G . - Insert the DPF filter H on manifold M until reaching the gasket P . | Fig. 11.101 |
| Important - Before fastening the DPF filter H , it must be oriented in the same position as the previous filter. - Do not apply any tension during the assembly of components. - Fasten the DPF filter H with clamp E (tightening torque 12 Nm ). | Fig. 11.102 |
| - Insert gasket P on the DPF filter H . - Insert manifold N on the DPF filter H until reaching the gasket P . Important - Before fastening manifold N , it must be oriented in the original position. - Fasten manifold N with clamp E (tightening torque 12 Nm ). | Fig. 11.103 |
| - Position support K in contact with support K1 and fasten clamp G (tightening torque 12 Nm ). | Fig. 11.104 |
| - Screw unions J on the DPF filter H and position them about 20° away from the centre line (as in Fig. 11.90 ). | Fig. 11.105 |
| - Fix unions J (tightening torque see service letter 700025 ). | Fig. 11.106 |
| - Fit pipes C on unions J and fasten them with clamps B . - Fix the Delta-P sensor D on its support K by means of screw A (tightening torque 10 Nm ). - Provide the machine owner with the warranty certificate of the new DPF filter KIT installed. - Perform the " DPF replacement " procedure to reset the ASH & SOOT through the relevant KOHLER diagnostic tool interfaced with the ECU. | Fig. 11.107 |

## Oil dipstick on timing gears side

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.17.1 Check - Remove dipstick A . - Verificare che il segno lasciato dall'olio sull'asta sia tra le tacche MIN e MAX . NOTE: Check the condition of seal A1 every time dipstick A is inserted into dipstick tube D . | Fig 11.108 |
| 11.17.2 Replacement 11.17.2.1 Disassembly - Unscrew screws B and C . - Remove the oil dipstick tube D from crankcase H along with support E. - Unscrew screw F and remove the dipstick tube D from support E along with cable tie G . | Fig 11.109 |
| 11.1.2.2 Assembly Important - Always replace O-ring D1 during every assembly. - Insert cable tie G on dipstick tube D. - Fit the clamp G onto support E using screw F . NOTE: Do not tighten screw F. - Insert the dipstick tube D into position on the crankcase H . - Position the support E onto cover L , slide clamp G onto the tube D to ensure the correct position. - Secure the clamp G onto support E (tightening torque 10 Nm ). | Fig 11.110 |
| - Secure support E using screws C onto crankcase H (tightening torque at 25 Nm ). - Secure the dipstick tube D to the crankcase H using screw B (tightening torque at 25 Nm ). | Fig 11.111 |

## Crankshaft pulley (2nd PTO)

| Disassembly - Disassemble the starting motor. - Mount the tool ST_34 in the seat of the starter motor and fit it with the two starter motor fixing screws. |  |
| --- | --- |
| - Position the crankshaft with the 1st cylinder in TDC, reference X1, X2 upwards. |  |
| - Undo the nut A (clockwise), remove the washer B and the pulley/targetwheel unit C . |  |
| Montaggio - Position the pulley assembly C on the crankshaft D with the X2 reference upwards and in line with the X1 reference. - Position the washer B on the crankshaft D . - Fix the pulley C using the nut A (tightening torque 240 Nm + Loctite 242 on the thread) and remove the tool ST_34 . |  |