Information on adjustments

KDI 3404 TCR-SCR Workshop Manual (Rev. 10.4)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | manoff |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information on adjustments

## 'Waste Gate' opening valve regulation

Important

- Before proceeding with operation, read Par. 3.3.2 .

- Regulation must not be carried out with the engine running.

- During the procedure in point 5 , pay special attention not to bend rod H .

- Disconnect the hose A from the turbocharger.

- Connect a pressure reducer C to the network of compressed air.

- Position dial gauge D in such a way that feeler F rests onthe Waste Gate rod control valve extremity H (point E ).

- By using gradually the reduction gear C send the air to the Waste Gate actuator control L in order to move rod H forward by 1 mm (value M to check on dial gauge D). Pressure read on gauge B must be: 2500 mbar.

- If pressure is less or more than the indicated value, proceed as follows:
- Remove the retainer cotter pin (point E ) and disconnect rod H from the Waste Gate control lever.
- Tighten (to increase) / or loosen (to decrease) pressure of the ring nut of rod H until reaching the corrected calibration.
- Redo lock nut G .
- Reconnect rod H and assemble the cotter pin point E .

Fig 12.1

## Air filter check

NOTE : Component not supplied by KOHLER . Refer to the technical documentation of the vehicle.

## Oil steam separator check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Loosen clamp B and remove hose C . - Remove rapid fitting D from separator A . - Start the engine at idle speed or without a load and check if air comes out from unions A1 and A2 . NOTE: If what is described in Point 3 does not occur, proceed with cleaning or replacing oil separator A and accurately clean all connecting hoses, and repeat the operation from Point 3. |  |

## Rubber hoses and manifolds check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| The check is carried out by applying slight deflection or bending along the tube/hose and next to the hose clamps. Components must be replaced if they have clear signs of cracks, tears, cuts, leaks, or do not retain a certain degree of elasticity. - Check the condition of all rubber hoses A . - Check whether there are any leakages of air, refrigerant, oil or fuel next to their connections. NOTE : Refer to the technical documentation of the machine for components that are not shown in the figure. | Fig 12.3 |

## Oil leak check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| Check that there are no leakages next to area A . - Start the engine at idle speed or without a load and check whether there are any leakages next to area A. - It is anyhow necessary to also check the seals of all main components and their surface contact, such as: - crankcase and gasket (side 1 a PTO) - oil sump and exhaust caps - cylinder head and its assembled components - rocker arm cover - Timing system carter and gasket (side 2 a PTO) - oil dipstick housing or rod support tube. NOTE: Perform the checks described in Points 1 and 2 periodically and during maintenance procedures. It is also necessary to check for leakages on the components that are not listed. If necessary, disassemble the components that have a leakage and investigate the possible cause. The components must be replaced otherwise they do notguarantee their sealing. | Fig 12.7 Fig 12.8 |

## Oil pressure check

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Replace the oil dipstick A with a thermocouple B (Fig. 12.6). - Unscrew and remove the oil pressure switch C and screw on a 10 bar pressure gauge in its seat (Fig. 12.8) . - Start the engine at idle speed and without a load, check the oil pressure value according to the oil temperature (Fig. 12.7 ). NOTE : The graph in Fig. 12.7 illustrates the pressure line with speed of 1000 Rpm. - If the pressure values are below the values indicated in Fig. 12.7 , check to identify the cause of the problem. Fig. 12.7 | Fig. 12.6 Fig. 12.8 |

## DEF Check

| 1 - The inspection is carried out with refractometer A . Follow the instructions on the device; the correct value must comply with the values of 1% ± 32.5%. Warning Use of the engine with DEF that does not comply with the quality specifications described in point 1 will trigger an error code and result in an inducement strategy (Para. 2.12.3). | Fig 12.4 |
| --- | --- |

## DEF tank filter check and cleaning

| Warning - Do not use pressurised air or water. - Only use hot water for cleaning and lubrication of the gaskets - replace gasket D if damaged. - The tank and its components cannot be repaired - do not damage the components during cleaning operations. |  |
| --- | --- |
| 1 - Turn head A anticlockwise to release tank B. 2 - Remove head A from tank B. | Fig. 12.5 |
| 3 - Visually check filter B and proceed to point 4 if there are traces of crystallisation or impurities. 4 - Wash filter C in a basin with hot water . NOTE: the hot water will dissolve the crystal residues caused by the DEF liquid. It is permitted to use a brush to remove any impurities completely . 5 - Assemble head A by following the instructions in reverse in point 2 and 1. NOTE: only use water to lubricate gasket D. | Fig. 12.6 |