General information

KSD 1403 Workshop Manual (Rev. 00_DRAFT)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | KSD1403-WS |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# General information

## Useful information

- This manual contains the instructions needed to carry out  proper use and maintenance of the engine, therefore it must always be available, for future reference when required.

- Information, descriptions and pictures contained in this manual reflect the basic configuration of the engines ( Par. 1.4 and Par. 1.5 ).

- However, the development of engines is continuous. Therefore, the information in this manual is subject to change without notice.

- KOHLER reserves the right to make, at any time, changes on the engines for technical or commercial reasons.

- These changes do not require KOHLER to intervene on the production marketed up to that time and nor to consider this manual as inappropriate.

- The paragraphs, tables and figures are numbered by chapter and followed by the progressive paragraph, table and/or figure number.

Es: Par. 1.3 - chapter 1 paragraph 3 .
Tab. 2.4 - chapter 2 table 4 .
Fig. 4.5 - chapter 4 figure 5 .

NOTE: The paragraphs may contain sub-paragraphs.

- All technical terms, specific components and symbols ( Tab. 15.1 ) that are in the manual are listed and described inside the glossary, which can be consulted in ( Chap. 15 ).

- The references of the objects described in the text and in the figure are indicated by letters and numbers, which are always and only related to the paragraph you are reading unless there are specific references to other figures or paragraphs.

- Reference to values are indicated by letters or numbers.

- Other important references are highlighted in red.

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- Any additional section that KOHLER will deem necessary to supply at a later stage must be kept with the manual and considered as an integral part of it.

- The information contained in this manual is the sole property of KOHLER , therefore no partial or total reproduction or replication is allowed without the express permission of KOHLER .

1.1.1 Useful Information - accident prevention - environmental impact

- Before proceeding repair - handling the motor , read the entire Chap. 3 , which contains important information about the procedures to be followed for safety and environment .

## Manufacturer and engine identification

Pos.; Descrizione. 1; Production place. 2; Manufacturer identification. 3; Engine model. 4; Engine serial number. 5; Month and year of production. 6; Approval data. 7; Engine version. 8; Data matrix.

## Homologation labels

1.7.1 Label for EPA rules (compilation example) POS.; DESCRIPTION. 1; Model year in compliance with the rules. 2; Power category (kW). 3; Engine displacement (L). 4; Particulate emission limit (g/kWh). 5; Engine family ID. 6; Emission Control System = ECS. 7; Fuel with low sulphur content. 8; Injection timing. 9; Electronic injector opening pressure (bar). 10; Production date (example: 2013.JAN). 1.7.2 Label for China Standards (compilation example) POS.; DESCRIPTION. 1; Manufacturer. 2; Engine model. 3; Manufactoring date. 4; Certificate N°. 5; Power range (kW). 6; Emission level. 7; Rated power. 8; Aftertreat system. 1.7.3 Label for Korea Standards (compilation example) POS.; DESCRIPTION. 1; Tier 4 Final. 2; Engine model. 3; Manufactoring date and manufacturer code. 4; N° Korea emission certificate.

## Identification of the main internal components of the engine

WIEW OF EXHAUST SIDE

Fig 1.5

WIEW OF FLYWHEEL SIDE

Fig 1.6

WIEW OF TIMING SYSTEM SIDE

Fig 1.7

The following chapters contain operating references in order to clearly understand the engine. This paragraph illustrates these references that may be recognised by means of some main internal components.

Should you need to execute complex operations, always consult this paragraph

Tab 1.2

| POS. | DESCRIPTION |
| --- | --- |
| 1 | Cylinder/Piston N. 1 |
| 2 | Cylinder/Piston N. 2 |
| 3 | Cylinder/Piston N. 3 |
| 4 | Fuel injection pre-chamber |
| 5 | Crankshaft pulley (2 nd PTO) |
| 6 | Gear timing system |
| 7 | Thermostatic valve |
| 8 | Oil pump |
| 9 | Oil suction hose |
| 10 | Crankshaft |
| 11 | Exhaust manifold |
| 12 | Intake manifold |
| 13 | Engine valves camshaft |
| 14 | 3rd/4th PTO |
| 15 | Flywheel (1 st PTO) |
| 16 | Fuel injection pump camshaft |

## Engine component identification

NOTE: Some components are for illustrative purposes only and may vary or they are not supplied by Kohler.

POS.; DESCRIPTION. 1; ECU. 2; Oil filler cap. 3; Fuel injection pump. 4; Lub. oil filter. 5; Oil cooler. 6; Fuel filter. 7; Oil sump. 8; Oil drain plug. 9; Oil dipstick. 10; Turbocharger (TC & TCA version only). 11; Alternator. 12; Starter motor. 13; Flywheel. 14; Coolant pump. 15; Thermostatic valve. 16; 3rd PTO.