Service procedures

KSD 1403 Workshop Manual (Rev. 00_DRAFT_01)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
|  | KSD1403-WS |  |  |  |  |  |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Service procedures

## Preliminary information

Important

- The mark (  ) after the title of a paragraph, indicates that the procedure is not required in order to disassemble the engine, however the procedures are featured in order to illustrate the disassembly of components.

- The operator should prepare all equipment and tools in order to enable him to carry out the operations correctly and safely.

- Before proceeding with operation, carefully read Chap. 3 .

- In order to operate safely and easily, we recommend positioning the engine on a rotating stand for engine overhauling.

- Seal all injection component unions as illustrated in Par. xx  during assembly.

- Protect all disassembled components and coupling surfaces subject to oxidation with lubricant.

- Where necessary, reference to special tools to use during disassembly operations is indicated (es. ST_05 ).

- Replace all gaskets or rubber components at each disassembly.

## Cooling circuit

| Uncscrew the cap A1 of the radiator A to drain the remaining coolant. |  |
| --- | --- |
| DISASSEMBLING |  |
| Radiator NA - TC model |  |
| Loosen the clamps B and remove the hose C . Unscrew the nuts D and the screw E to remove the rods F1 , F2 . |  |
| Loosen the clamps B and remove the hose C . Unscrew the nuts B to remove the radiator A . § |  |
| Radiator TCA model | TBD |
| Oil Cooler hoses |  |
| Loosen the clamps B and remove the hoses C . |  |
| NOTE: the oil cooler has various configurations and can be installed also on the intake side. Unscrew the screws E1 and clamps B1 if present. |  |
| Fan and belt |  |
| Unscrew the screws G and remove the fan H . |  |
| Remove the belt L using ST_57 tool. | DISASSEMBLING |
| Coolant pump |  |
| Unscrew the screw M and remove the pulley N . |  |
| Loosen the clamps B and remove the hose C . |  |
| Do not disassemble the flange R . NOTE: the flange R for pulley N can be intalled in various positions based from engine specification, each configuration has a specific belt and pulley. It is not necessary to remove the flange R for pulley N to proceed with following operations below. In case of flange disassembling, take note of its original position to avoid a wrong positioning at the assembling. + Apply Loctite 242 on the thread of screws (tightening torque 10 Nm ). § Unscrew the screws P and remove the coolant pump Q . |  |
| Thermostatic valve |  |
| Unscrew the screws W1 and remove the cover W2 . |  |
| Remove the thermostatic valve W3 . |  |
| + Thermostatic valve specifications Opening start; Opening stop. 0.1 mm; @ 80° C; 7 mm; @ 95° C. |  |
| Radiator support |  |
| Unscrew the screws S and remove the radiator support T . |  |
| ASSEMBLING |  |
| Radiator support |  |
| Fix the radiator support T on the crankcase V using the screws S and washers S1 (tightening torque 45 Nm ). |  |
| Thermostatic valve |  |
| Place the thermostatic valve W3 inside the seat of cylinder head W . Place the gasket W4 inside the seat of cover W2 . + Apply Loctite 270 on the thread of screw W1 . § Fix the cover W2 with gasket W4 on the cylinder head W using the screws W1 (tightening torque 10 Nm ). NOTE: make sure that the gasket W4 stays in its seat during cover W2 assembly process. |  |
| Coolant pump |  |
| Fix the coolant pump Q on the crankcase V with the gasket Q1 using the screws P (tightening torque 25 Nm ). |  |
| Install components N1 , N2 inside pulley N . + Apply Loctite 242 on the thread of screw M . § Fix the pulley N on flange R using the screw M (tightening torque 25 Nm ). |  |
| Fan and belt | INSTALLING |
| Install the new belt A on pulleys N , Q2 , Z1 , Z2 using the ST_57 tool. |  |
| + Apply Loctite 242 on the thread of screw G . § Fix the fan H with the plate H1 and the spacer H2 on pulley N (tightening torque 10 Nm ). NOTE: the spacer H2 can be different based on engine specification. |  |
| Oil Cooler hoses |  |
| Fix the hoses C using the clamps B . |  |
| NOTE: the oil cooler has various configurations and can be installed also on the intake side. Fix the clamps B1 using screws E1 if present. |  |
| Radiator NA - TC model |  |
| Place the radiator A on support T . Fix the radiator A using nuts D (tightening torque 25 Nm ). Fix the hoses C using the clamps B . |  |
| Place the rods F1 , F2 for the radiator fixing A . Fix the rods F1 , F2 using the nuts D with washers D1 and screws E (tightening torque 10 Nm ). |  |
| Radiator TCA model | TBD |

## Exhaust line

| NA Model | TBD |
| --- | --- |
| TC - TCA Model |  |
| Muffler option A § |  |
| DISASSEMBLING Unscrew the nuts A . Unscrew the screws B and remove the muffler C . |  |
| ASSEMBLING Fix the muffler C on the bracket C1 using the screws B (tightening torque xx Nm ). Fix the muffler C on the flange D1 using the nuts A (tightening torque xx Nm ). |  |
| Turbocharger |  |
| DISASSEMBLING Unscrew the screws D3 and remove the tube E2 . Unscrew the screws D2 and remove the tube E1 . Loosen the clamp F1 and disconnect the hose E3 . Loosen the clamp F3 and disconnect the hose E4 . Unscrew the nuts D4 and remove the turbocharger D . |  |
| ASSEMBLING Place the turbocharger D on the interfacing falnge of exhaust manifold H . |  |
| Fix the turbocharger D on the exhaust manifold H using the nuts D4 (tightening torque 25 Nm ). Connect the hose E3 with the turbocharger D and fix it using the clamp F1 . Place the tube E2 on the turbocharger D and the crankcase G with screws D3 and copper washers D5 . Fix the tube E2 using the screws D3 (tightening torque 1 5 Nm ). Place the tube E1 on the crankcase G and under the turbocharger D with screws D2 and gskets E5 , E6 . Tighten the screws D2 (tightening torque 10 Nm ). |  |
| Only for TC model |  |
| DISASSEMBLING Loosen the clamp F2 and remove the hose E3 . § |  |
| ASSEMBLING Connect the hose E3 with the inlet manifold L and fix it using the clamp F2 . |  |

## Fuel System

| Fuel feed pump |  |
| --- | --- |
| Electric fuel feed pump § |  |
| DISASSEMBLING Disconnect the junction A1 and remove the tube E1 with the pump F and the pre-filter G . |  |
| ASSEMBLING Connect the junction A1 on the filter D . Check the condition of tubes F and pre-filter G . |  |
| Mechanical fuel feed pump § |  |
| DISASSEMBLING Unscrew the screw B7 . Unscrew the nuts B8 and remove the pump P . |  |
| ASSEMBLING Place the pump P with gasket P1 on the crankcase Q . Fix the pump P on the crankcase Q using the nuts B8 (tightening torque 25 Nm ). Fix the tube E3 on the pump P using the screw B7 with copper washers B9 (tightening torque 15 Nm ). |  |
| Fuel filter |  |
| DISASSEMBLING Disconnect the connector A1 from the injection pump A . Unscrew the screw B1 . Unscrew the screws B2 and remove the fuel filter D with the tube C . |  |
| ASSEMBLING Fix the fuel filter D using the screws B2 (tightening torque 25 Nm ). Conect the connector A1 on the injection pump A . Fix the tube C using the screw B1 (tightening torque 25 Nm ). |  |
| Rail § |  |
| DISASSEMBLING Unscrew the nuts A2 , A3 and remove the tube E2 from the rail H and injection pump A . |  |
| Remove the lockers H2 . Unscrew the screws B2 and remove the spacers H1 and the rail H . |  |
| ASSEMBLING + Apply oil EMI-385 on injector gaskets. § Place the rail H on the injectors L . |  |
| Install the lockers H2 on the rail H . |  |
| Place the spacers H1 under the rail H and fix the rail H using the screws B2 (tightening torque 25 Nm ). |  |
| Place the tube E2 on the injection pump A and Rail H , screw the nuts A2 , A3 . Tight the nut A2 (tightening torque 28 Nm ). Tight the nut A3 (tightening torque 28 Nm ). |  |
| Electronic injectors |  |
| DISASSEMBLING Place the the sockets L1 of the tool ST_48 on the injectors L . Fix the nuts B3 on the cylinder head N . |  |
| Install the lockers L2 on the sockets L1 to lock the injectors L . |  |
| Unscrew the nuts B4 to extract the injectors L from the cylinder head N . |  |
| ASSEMBLING + In case of injector reinstallation: before the reinstallation, replace all injector gaskets. § + Apply oil EMI-385 on injector gaskets. § Place the injectors L inside the cylinder head N . Place the sockets L1 of the tool ST_48 on the injectors L . Place the washers B6 under the nuts B3 and fix the nuts B3 on the cylinder head N . |  |
| Install the lockers L2 on the sockets L1 to lock the injectors L . Screw the nuts B4 to install the injectors L inside the cylinder head N . |  |
| Injection pump |  |
| DISASSEMBLING Unscrew the screws B5 and remove the injection pump A . |  |
| ASSEMBLING Install the roller tappet A4 inside the crankcase M if not allready present. Place the injection pump A on the crackcase M . + Fix the pump A using the screws B5 respecting the tightening cycle below listed: - cycle 1: 15 Nm - cycle 2: 2 5 Nm |  |

## Sensors and Switches

| T-MAP Sensor (only for model TC - TCA) |  |
| --- | --- |
| DISASSEMBLY Unscrew the screws A1 and remove the sensor A . |  |
| ASSEMBLY Place the sensor A on the intake manifold A3 . Fix the sendor A using the screws A1 (tightening torque 10 Nm ). |  |
| RPM Sensor |  |
| DISASSEMBLY Unscrew the screw B1 and remove the sensor B . |  |
| ASSEMBLY Place the sensor B on the crackcase B3 with the shim B2 . Fix the sensor A using the screw B 1 (tightening torque xx Nm ). |  |
| Phase Sensor |  |
| DISASSEMBLY Unscrew the screw C1 and remove the sensor C . |  |
| ASSEMBLY Place the sensor C on the carter C4 with the shim C2 . + Apply Loctite 242 on the thread of screw C1 . § Fix the sensor C using the screw C1 (tightening torque 10 Nm ). |  |
| Coolant temperature Sensor |  |
| DISASSEMBLY Unscrew and remove the sensor D . |  |
| ASSEMBLY Fix the sensor D with the gasket D1 on the cylinder head D2 (tightening torque 12 Nm ). |  |
| Oil Pressure Switch |  |
| DISASSEMBLY Unscrew and remove the switch E . |  |
| ASSEMBLY Fix the switch E on the crankcase E1 (tightening torque 25 Nm ). |  |

## Electrical components

| Starter Motor |  |
| --- | --- |
| DISASSEMBLY Unscrew the screws A and remove the motor B . |  |
| ASSEMBLY Fix the motor B to the flanging plate/bell C using the screws A (tightening torque 25 Nm ). |  |
| Voltage alternator |  |
| Alternator 45A |  |
| DISASSEMBLY Unscrew the screw A . |  |
| Unscrew the nut B and remove the alternator C . |  |
| ASSEMBLY Place the alternator C on the crankcase D , install the screw B4 inside the fixing hole and install the spacer B3 . Screw the nut B with the washer B1 on the screw B4 . |  |
| Fix the bracket C1 on the cylinder head E using the screw A (tightening torque 25 Nm ). Fix the alternator C using the nut B and the screw B4 (tightening torque 25 Nm ). |  |
| Alternator 80A - 100A § |  |
| DISASSEMBLY Unscrew the screw A . |  |
| Unscrew the nut B and remove the alternator C . |  |
| ASSEMBLY Place the alternator C on the crankcase D , install the screw B4 inside the fixing hole and install the spacer B3 . Screw the nut B with the washer B1 on the screw B4 . |  |
| Fix the alternator C using the screw A , the nut A1 with the washer A2 on the bracket C1 (tightening torque 25 Nm ). Fix the alternator C using the nut B and the screw B4 (tightening torque 25 Nm ). |  |
| Glow plugs |  |
| DISASSEMBLY Unscrew the nuts A and remove the cables B1 , B2 . |  |
| Unscrew and remove the glow plugs C . |  |
| ASSEMBLY Install and fix the glow plugs C on cylinder head D (tightening torque 15 Nm ). |  |
| Fix the cables B1 , B2 on the glow plugs C using the nuts A (tightening torque 2.5 Nm ). |  |

## Intake manifold

| DISASSEMBLY Unscrew the screws A and remove the manifold B . |  |
| --- | --- |
| ASSEMBLY Place the manifold B on the cylinder head C using the tool ST_18 . Fix the manifold B and the gasket B1 on the cylinder head C using the screws A (tightening torque 25 Nm ). |  |

## Exhaust manifold

| DISASSEMBLY Unscrew the nuts A and remove the manifold B . |  |
| --- | --- |
| ASSEMBLY Place the gaskets B1 and the manifold B on the cylinder head C using the studs B2 . Fix the manifold B on the cylinder head C using the nuts A (tightening torque 25 Nm ). |  |

## PTOs

| 1st PTO |  |
| --- | --- |
| Flywheel |  |
| DISASSEMBLY Install the tool ST_16 on pulley C . Install the component T3 on the crankcase D using the screws T2 entering the component T4 inside the groove of the component T3 . Fix the screws T5 to lock the pulley C to the tool ST_16 . NOTE: in this way the crankshaft is locked. |  |
| Unscrew the screw A and remove the flywheel B . |  |
| ASSEMBLY Place the flywheel B on the crankshaft C using the ST_46 . Fix the flywheel B on the crankshaft C using the screws A (tightening torque 140 Nm ). |  |
| Flanging plate |  |
| DISASSEMBLY Unscrew the screws A and remove the plate B . |  |
| ASSEMBLY Place the plate B on the crankcase C . NOTE: use the proper reference pins. Fix the plate B on the crankcase C using the screws A (tightening torque 45 Nm ). |  |
| Flanging bell (TBD) |  |
| DISASSEMBLY Unscrew the screws A and remove the bell B . | TBD |
| ASSEMBLY Place the bell B on the crankcase C . NOTE: use the proper reference pins. Fix the bell B on the crankcase C using the screws A (tightening torque xx Nm ). |  |
| 2nd PTO |  |
| DISASSEMBLY Unscrew the screw A and remove the pulley B with the tool ST_16 . Remove the component T3 from the crankcase D . |  |
| ASSEMBLY Install the pulley B on the crankshaft C using the screw A . NOTE: do not fix the screw A . Install the tool ST_16 on pulley B . Install the component T3 on the crankcase D using the screws T2 entering the component T4 inside the groove of the component T3 . Fix the screws T5 to lock the pulley C to the tool ST_16 . Fix the screws A (tightening torque 360 Nm ). |  |
| 3rd PTO |  |
| DISASSEMBLY Unscrew the nuts A and remove the 3rd PTO B . |  |
| ASSEMBLY Place the 3rd PTO B on the crankcase C with the gasket B1 . Fix the 3rd PTO B using the nuts A (tightening torque xx Nm ). |  |

## Cylinder head

| DISASSEMBLY Unscrew the screws A1 and remove the cover A . |  |
| --- | --- |
| Unscrew the screws B1 and remove the rocker arms B . |  |
| Remove the rods C . |  |
| Unscrew the screws D1 and remove the cylinder head D . |  |
| ASSEMBLY ENTER THE PISTON PROTUSION VALUES TO CHOOSE THE CORRECT HEAD GASKET Check the tappets E1 are correctly installed inside the crackcase E . Place the gasket D2 on the crankcase E using the reference pins E2 . Place the head D on the gasket D2 using the reference pins E2 . |  |
| Apply MOLY GREASE on the screw D1 (20 mm from the head bottom ) Tighten the screws D1 respecting the tightening order as per illustration indication at each cycle below listed: - cycle 1: 10 Nm - cycle 2: 35 Nm - cycle 3: 90° - cycle 4: 90° - cycle 5: 90° - cycle 6: 90° |  |
| Install the rods C inside the proper niches inside the cylinder head D . |  |
| Place the spacers B2 correctly on the valves B3 . Place the rocker arms B on the head D using the proper reference pins. Check all spacers B2 are correctly installed on the valves B3 . + Apply Loctite 242 on the thread of screw B1 . § Fix the rocker arms B using the screws B1 (tightening torque 25 Nm ). |  |
| Check the gasket A2 is correctly installed insde its seat inside the cover A . Place the cover A on the head D using the tool ST_17 . Place the screws A1 inside the fixing holes. |  |
| Tighten the screws A 1 respecting the tightening order as per illustration indication ( tightening torque 10 Nm ). |  |

## Oil circuit

| Oil Cooler |  |
| --- | --- |
| DISASSEMBLY Unscrew and remove the filter A . |  |
| Unscrew the joint C and remove the oil cooler D . |  |
| Unscrew th screw E and remove the support F . |  |
| ASSEMBLY Install the gasket E1 in the proper seat on the screw E . Install the gasket F1 on the support F . Fix the support F on the crankcase G using the screw E with the gasket E2 (tightening torque 25 Nm ). |  |
| Check the oil cooler gasket is correctly installed. Fix the oil cooler D on the support F using the joint C (tightening torque 30 Nm ). |  |
| Check the gasket D1 is correctly installed inside the proper seat of the oil filter D . Tifgten the filter D on the support F (tightening torque 20 Nm ). |  |
| Oil Pressure valve |  |
| DISASSEMBLY Unscrew the plug A and remove the gasket A1 , the spring A2 , the piston A3 from the carter B . |  |
| ASSEMBLY Install the piston A3 , the spring A2 inside the carter B . Tighten the plug A with the cooper washer A1 on the carter B (tightening torque 17.5 Nm ). |  |
| Oil sump |  |
| DISASSEMBLY Unscrew the screws A1 and remove the oil sump A . NOTE: Use a metal sheet/spatula between the crankcase and oil sump to detach them. |  |
| Unscrew the screw B1 . Unscrew the screw B2 and remove the tube B . Unscrew the screws C1 and remove the plate C . |  |
| ASSEMBLY Install the gaskets B3 inside the proper seat of the tube B . Place the tube B installing it inside the oil inlet hole of the crankcase D . + Apply Loctite 242 on the thread of screw B 1, B2 . § Fix the tube B using the screws B2 (tightening torque 25 Nm ). Fix the tube B using the screws B1 (tightening torque 10 Nm ). |  |
| + Apply Loctite 242 on the thread of screw C1 . § Fix the plate C using the screws C1 (tightening torque 25 Nm ). |  |
| Apply about 2.5 mm cord of sealant S ( Loctite 5699 ) on the oil sump A as per illustration. |  |
| Place the oil sump A on the crankcase sulbasamento D using the tool ST_18 . + Apply Loctite 242 on the thread of screw A1 . § Fix the oil sump A on the crankcase D using the screws A1 (tightening torque 25 Nm ). |  |
| Timing gear cover (oil pump) |  |
| DISASSEMBLY Unscrew the screws A1 and remove the carter A . |  |
| ASSEMBLY Apply about 2.5 mm cord of sealant S ( Loctite 5699 ) on the carter A as per illustration. |  |
| Place the gasket A3 in the proper seat of the carter A . Install the gasket A2 on the carter A using the tool ST_58 . Place the carter A on the crankcase B using the tool ST_18 . |  |
| + Apply Loctite 242 on the thread of screw A1 . § Fix the carter A on the crankcase B using the screws A1 respecting the fixing order as per illustration indication (tightening torque 25 Nm ). |  |

## Crankcase

| Timing gear (camshafts) |  |
| --- | --- |
| DISASSEMBLY Remove the tappets D1 from the crankcase D . |  |
| Unscrews the screws A1 and remove the gear A . |  |
| Unscrews the screws B1 and remove the shaft B . Unscrews the screws C1 and remove the shaft C . |  |
| ASSEMBLY Install the shaft C inside the crankcase D . + Apply Loctite 242 on the thread of screw C1 . § Fix the shaft C using the screws C1 (tightening torque 10 Nm ). |  |
| Install the shaft B inside the crankcase D . + Apply Loctite 242 on the thread of screw B1 . § Fix the shaft B using the screws B1 (tightening torque 10 Nm ). |  |
| Install the gear A2 on the support A . Install the washer A3 on the support A . |  |
| Place the gear A2 on the crankcase D aligning the references as per illustration. + Apply Loctite 242 on the thread of screw A1 . § Fix the support A on the crankcase D using the screws A1 (tightening torque 10 Nm ). |  |
| Crankshaft seal support - flywheel side |  |
| DISASSEMBLY Unscrew the screws A1 and remove the flange A . |  |
| ASSEMBLY Install the gasket A2 inside the flange A using the tool ST_42 . Place the flange A on the crankcase B . |  |
| + Apply Loctite 242 on the thread of screw A1 . § Fix the flange A using the screws A1 respecting the fixing order as per illustration (tightening torque 10 Nm ). |  |
| Conrod and piston |  |
| DISASSEMBLY Unscrew the screws B4 and remove the conrod caps B1a , B2a , B3a . |  |
| Extract the conrod and piston P from the crankcase C . |  |
| Replace the bearings D on B1a , B2a , B3a, B1b , B2b , B3b . NOTE: the bearing position D on B1a , B2a , B3a, B1b , B2b , B3b must be centered as per illustration. Install B1a , B2a , B3a su B1b , B2b , B3b using the screws B4 . NOTE: do not tighten the screws B4 . |  |
| Check the conrod and piston groups P , weight difference must not exceed 10 gr . Unscrew the screws B4 and remove B1a , B2a , B3a da B1b , B2b , B3b . § |  |
| ASSEMBLY Rotate the crankshaft D moving the crankpin D1 at BDC . |  |
| The combustion chamber H of the piston must be oriented towards the inlet manifold. |  |
| + Lubricate the piston mantle, segments, conrod bearings and cylinder surface with engine oil 15W-40 . Install the conrod and piston groups P about 10 mm of the piston skirt. |  |
| Align the gudgeon pin axle with engine center axle. Rotate the piston P about 15° anticlockwise. |  |
| This operation avoid the conrod impact against the oil sprayer E during the conrod and piston installation inside the cylinder C . Gently push the piston in the cylinder C paying attention to the sprayer E . |  |
| + Lubricate the crankpins with engine oil 15W-40 . + Apply BRB100 MOLY GREASE under the screw head P1c . Place the conrod P1a on crankpin D1 . Install the cap P1b using the screws P1c . NOTE: the caps P1b have the mating plan fractured, each cap can be mate with its origin conrod P1a , pay attention to the right mating. + Tighten the screws P1c respecting the tightening cycle below listed: - cycle 1: 5 Nm - cycle 2: 20 Nm - cycle 3: 33° - cycle 4: 33° |  |
| Piston |  |
| DISASSEMBLY Remove the lock ring R and extract the gudgeon pin G from the piston P . |  |
| Remove the rings S1 , S2 , S3 from the piston P . |  |
| ASSEMBLY Install the rings S1 , S2 , S3 on the piston P . |  |
| + Apply MOLYSLIP on gudgeon pin G . Mate the conrod P1a with the piston P and install the gudgeon pin G . Install the lock rings R to lock the gudgeon pin G . |  |
| Crankshaft |  |
| DISASSEMBLY Take note of caps A1 , A2 , A3 marking their position on the crankcase. Unscrew the screws A and remove the caps A1 , A2 , A3 , A4 . Remove the crankshaft B . |  |
| Unscrew the screws C1 and remove the sprayers C2 . |  |
| Remove the bearings C3 , C4 . |  |
| Remove the bearings C5 , C4 from the caps A1 , A2 , A3 , A4 . |  |
| ASSEMBLY Fix the sprayers C2 on the crankcase C using the screws C1 (tightening torque 8 Nm ). |  |
| Install the bearings C3 on crankcase C seats respecting the notches C3a and make sure the oil delivery holes C3b is matching with the crankcase ones. |  |
| Install the bearings C4 on the crankcase C seats. |  |
| Install the crankshaft B on the bearings C3 . |  |
| Install the bearings C5 on caps A1 , A2 , A3 seats respecting the notches C5a . |  |
| Install the caps A1 , A2 , A3 on the crankcase C respecting their original position using the screws A . + Apply MOLYSLIP under the screw head A + Tighten the screws A respecting the tightening cycle below listed: - cycle 1: 15 Nm - cycle 2: 45° - cycle 3: 45° |  |
| Install the bearing C5 on cap A4 seat respecting the notch C5a . |  |
| Install the bearings C4 on the cap A4 seat. |  |
| Install the gaskets A4a inside the proper seat on cap A4 and keep in position using the tool ST_49 . Install the cap A4 inside the crankcase C paying attention to not cut the gasket A4a during installation. Install the cap A4 using the screws A . + Apply MOLYSLIP under the screw head A + Tighten the screws A respecting the tightening cycle below listed: - cycle 1: 15 Nm - cycle 2: 45° - cycle 3: 45° |  |