Information about optional components

KDI 2504 M Workshop manual (Rev. 09.6)

Registration of modifications to the document

Any modifications to this document must be registered by the drafting body, by completing the following table.

| Released by | Code | Revision | Release Date | Revision date | Edited by | Endorsed |
| --- | --- | --- | --- | --- | --- | --- |
| Technical Documentation | ED005302-961 | 9.6 | 04/2013 | 03/2023 | M. SILVESTRONE | V. MANINI |

Translated from the original manual in Italian language

Data reported in this issue can be modified at any time by KOHLER.

Sommario

1.	TITOLO 1	2

1.1.	Asdfsdfsdf	2

1.2.	Asdfsdfsdfggg	2

# Information about optional components

## Oil dipstick in cylinder head

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.1.1 Check - Pull out the dipstick B in the direction of the arrow A . - Check that the mark left by the oil on the dipstick is between the MIN . and MAX . notches. | Fig 11.1 |
| 11.1.2 Replacement 11.1.2.1 Disassembly - Undo the screw D . - Pull out the oil dipstick hose E in the direction of the arrow F . | Fig 11.2 |
| 11.1.2.2 Assembly Important - Always replace the gasket G every time it is disassembled. - Insert the gasket G in the seat K of the hose E . - Insert the hose E in the crankcase H . | Fig 11.3 |
| - Tighten the oil dipstick hose E using the screw D on the manifold L (Tightening torque of 10 Nm ). | Fig 11.4 |
| NOTE: Check the integrity of the gaskets J. - Insert the dipstick B inside the hose E . | Fig 11.5 |

## Heater (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.2.1 Disassembly - Undo the screws A and the relevant washers and remove the earth cable B . - Remove the flange C and the manifold D . - Remove the heater E and the relevant gaskets F . | Fig 11.6 |
| 11.2.2 Assembly Important - Always replace gaskets F , with each assembly. - In sequence, fit the manifold G with the gasket F , the new heater E , the second gasket F , the flange C, the washers H , the screws A and the cable B . - Secure the flange H with the screws A (tightening torque at 22 Nm ). - Secure the earth cable B with the nut J and the relevant washer on the heater E . | Fig 11.7 |

## Poly-V alternator belt (replacement and adjustment)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Loosen the nut B and manually tighten the screw C until it just touches the pulley pin D (Fig. 11.9) . | Fig 11.8 |
| - Untighten the screw E by around 32mm (A) . - Untighten the screw C . NOTE: The belt tensioner pulley F should move towards the arrow G . If it does not, please move it manually. | Fig 11.9 |
| - Remove the V-Belt H and install the new one. NOTE: Ensure that the internal profile of belt H is properly inserted into the grooves of the pulley A (as illustrated in D1 e D2 ). | Fig 11.10 |
| - Tighten capscrew C , to shift gudgeon D fully to the bottom of the grooved guide. - Tighten capscrew E (tightening torque at 45Nm ). - Hold the screw C still with a key, and tighten the screw B on the plate L to secure the screw C (tightening torque at 45Nm ). - Check, in point P (Fig. 11.8) ,the tension of the belt. Check by the appropriate tool that at point p the tension value is between 149 and 196 Hz . NOTE: For belt ED0024404960-S the value is between 155 and 201 Hz . NOTE: After the engine has been in operation for around 15 minutes, repeat point 8 . | Fig 11.11 |

## Tightening pulley and alternator for Poly-V belt

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.4.1 Disassembly - Perform the operations from point 1 to 3 of Par. 11.3 . - Remove the belt H ( Fig. 11.10 ) . - Undo and remove the screw A . - Fully undo the screw B and remove the tightening pulley C . | Fig 11.12 |
| - Undo the screws D and remove the plate E and the pin F . | Fig 11.13 |
| - Undo the screws G and H remove the alternator L . | Fig 11.14 |
| - Undo the screws M and remove the bracket N . | Fig 11.15 |
| 11.4.2 Assembly - Secure the bracket N using the screws M on the cylinder head P (tightening torque at 25 Nm ). | Fig 11.16 |
| - Insert the screw H into the fixing hole on the alternator L . - Insert the spacer R on the screw H (between the alternator and crankcase). - Tighten the screw manually H onto the crankcase Q . - Orientate the second fixing hole of the alternator L with the hole of the bracket N , secure the alternator L using the screw G (tightening torque at 25 Nm ) onto the bracket N and then the screw H (tightening torque at 25 Nm ). | Fig 11.17 |
| - Insert the pin F in the plate slot E . - Orientate the pin F with the surface S (support for screw A ) upwards. - Secure the plate E using the screws D on the bracket N (tightening torque at 25 Nm ). | Fig 11.18 |
| - Insert the screw B in the plate C1 and pulley C . - Manually tighten the screw B onto the pin F up to the stop; Undo the screw B again by one turn. NOTE: The screw B must protrude by about 32 mm (A) from the surface of the tightening pulley C (see detail X ). - Install the new belt H (Fig. 11.10) . - Tighten the screw A onto the plate E up to the stop on the pin F . - Perform the operations from point 6 to 8 of Par. 11.3 . | Fig 11.19 |

## Idler gear (for 3rd / 4th PTO)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.3.1 Disassembly - Undo the screw A and remove the gear unit B . | Fig 11.8 |
| 2. Remove the retainer ring C from the seat of the pin D . 3. Remove the shoulder washer E , the gear B , the shoulder ring F and the bushing G from the pin D . | Fig 11.9 |
| 11.3.2 Assembly - Fit into the pin D : - The shoulder ring F (of least thickness) - The gear B - The shoulder ring E - The retainer ring C . - Insert the bushing G on the crankcase L . | Fig 11.10 |
| Important - Always replace the washer H every time it is disassembled. Modified component, see service letter 700019 - 700021 - 700028 . - Check that the perforated screw A is free from impurities inside it. - Position the gear unit B on the hole J using the bushing G to centre. - Secure the gear B using the screw A inserting the washer H (tightening torque at see service letter 700019 - 700021 - 700028 ). | Fig 11.11 |

## 3rd PTO (replacement)

Fig 11.12

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.4.1 Disassembly - Undo the screws A and remove the pump B . | Fig 11.13 |
| - Remove the centring ring C and the relative gaskets - Undo the screws N . | Fig 11.14 |
| - Remove the flange F with the components D, E , G and H in the direction of the arrow P . - Remove the gasket J . | Fig 11.15 |
| - Remove the retainer ring D and the shoulder washer E . - Remove the gear H and the shoulder ring G from the flange F in the direction of the arrow Q . | Fig 11.16 |
| 11.4.2 Assembly Important - Always replace the gasket J after each assembly. - Lubricate the gear H with oil. - It is mandatory to replace the screws N or apply a few beads of Loctite 2701 . - Insert the gear H in the flange F in the direction of the arrow R inserting the shoulder ring G . - Insert the shoulder ring E on the flange F and clamp the gear H using the retainer ring D . - Position the flange F on the crankcase K inserting the gasket J , and inserting the gear H up to the stop on the bearing L ( Fig. 11.31 ). | Fig 11.17 Fig 11.18 |
| - Secure the flange F using the screws N (tightening torque at 25 Nm ). Important - Always replace the gaskets P and Q at each assembly. - Insert the centring ring C in the flange F up to the stop. | Fig 11.19 |
| - Insert the pump B on the flange F engaging the gear H . - Secure the pump B using the screws A on the flange F (tightening torque at 25 Nm ). | Fig 11.20 |

## 4th PTO (replacement)

Fig 11.21

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.5.1 Disassembly - Undo the screws A and remove the pump B . | Fig 11.22 |
| - Undo the screws C and remove the flange D . | Fig 11.23 |
| - Undo the screws E and remove the cover F . | Fig 11.24 |
| - Undo the screws G and remove the flange K with the components H , J , M and N . | Fig 11.25 |
| - Remove the retainer ring H and the shoulder ring J from the flange K . - Remove the gear N and the shoulder ring M from the flange K . | Fig 11.26 |
| 11.5.2 Assembly Important - Always replace the shoulder ring J after each assembly. - Lubricate the gear N with oil. - It is mandatory to replace the screws G or apply a few beads of Loctite 2701 . - Insert the gear N in the flange K in the direction of the arrow W inserting the shoulder ring M . - Insert the shoulder ring J on the flange K and clamp the gear N using the retainer ring H . | Fig 11.27 |
| - Position the flange K on the crankcase Q inserting the gasket P , and inserting the gear N up to the stop on the bearing R ( Fig. 11.41 ). - Secure the flange K using the screws N (tightening torque at 25 Nm ). | Fig 11.28 |
| Important - Always replace the gasket V after each assembly. - Insert the gasket V on the cover F , insert and position the cover F on the flange K . - Secure the cover F using the screws E on the flange K (tightening torque at 25 Nm ) | Fig 11.29 |
| Important - Always replace the gasket T after each assembly. - Position and secure the flange D using the screws C on the crankcase S (tightening torque at 10 Nm ). | Fig 11.30 |
| Important - Always replace the gasket U after each assembly. - Position the gasket U on the flange D . - Secure the pump B using the screws A on the flange D (tightening torque at 25 Nm ). | Fig 11.31 |

## 3rd + 4th PTO (configurations)

Fig. 11.32

| 11.6.1 Information Hydraulic pumps on the 3rd and 4th PTO can be installed at the same time. In some configurations, there is also the centering ring C on the 4th PTO. Important - For disassembly or installation, refer to Par. 11.5 , Par. 11.6 and Par. 11.7 . - Always replace the gasket of the rings B and C and flanges D and K at each assembly. - Lubricate the gear H with oil. | Fig. 11.33 |
| --- | --- |

## Balancer device (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.7.1 Disassembly - Perform the operations described in Par. 5.2 . - Undo the screws A and remove the oil sump B . | Fig 11.34 |
| - Undo the screws C and remove the hose D . | Fig 11.35 |
| - Undo the screws E and remove the shaft support box F . | Fig 11.36 |
| - Undo the screws G and remove the plate H . | Fig 11.37 |
| - Remove the shafts J and K in the direction of the arrow L from box F . | Fig 11.38 |
| 11.7.2 Assembly - Lubricate the bushings V with Molikote grease. - Insert the shafts J and K inside the box F in the direction of the arrow M . | Fig 11.39 |
| - Make sure that the shafts J and K inside the box F observe the marks N and that the shaft J with the gear indicated by letter " S " stamped on it is on the left with respect to the box F . | Fig 11.40 |
| - Secure the plate H using the screws G on the box F (tightening torque at 8 Nm ). | Fig 11.41 |
| - Manually tighten the retainer screw ST_15 on the box F by slightly rotating the shaft K , centring the hole on it using the ST_15 , to lock the device. | Fig 11.42 |
| - Rotate the crankshaft and clamp it on the TDC ( Ref. P upwards) using the tool ST_34 secured in place of the starter motor (detail Q ). | Fig 11.43 |
| - Position the box F on the surface R of the crankcase observing the reference bushings. - Secure the box F using the screws E and Loctite 242 inserting the washers U (tightening torque at 50 Nm ). - Remove the retainer screw ST_15 from the box F . | Fig 11.44 |
| Important - Check that the retainer screw ST_15 is not present on the box F ( Fig. 11.42 ). - Always replace the gasket W after each assembly. - Lubricate the gasket W with oil before assembling it. - Insert the gasket W in the seat on the flange of the oil hose D . - Secure the oil intake hose D using the screws C . - Perform all operations described in Par. 9.4.3 to assemble the oil sump. | Fig 11.45 |

## Air filter (cartridge replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| - Release the two hooks A and remove the cover B from the body C . - Remove the cartridges D . | Fig 11.46 |
| - Insert the new cartridge D and both of them inside the filter body C . - Secure the cover B via the hooks A . | Fig. 11.47 |

## Remote oil filter (disassembly and assembly)

| Important - Modified component, see service letter 700018 . 11.9.1 Disassembly - Perform the operations described in Par. 5.2 . Important - Before proceeding with operation, read Par. 3.3.2 . - For the replace the cartridge, please refer to operation number 6 (Par. 11.9.1) and 2 (Par. 11.9.2) . - For the disassembly of the pipes B and C, lock with a tool the fittings K, H (Fig. 11.49) and L (Fig. 11.50) in order to prevent their lose together with the nuts A , with the consequent of oil leakage. - Undo the nuts A and remove the hoses B and C . | Fig 11.48 |
| --- | --- |
| - Unscrew and remove the fitting H with its copper gasket from the head J . - Unscrew the fittings K and remove the copper gaskets from the head J . | Fig 11.49 |
| - Unscrew the fittings L and remove the copper gaskets from the support M . - Unscrew the cartridge N with gasket from the support M . | Fig 11.50 |
| 11.9.2 Assembly Important - Always replace the gaskets V after each assembly. - Clamp the fittings H on the support M inserting the gasket V (tightening torque at 65 Nm ). - Lubricate the gasket W and clamp the cartridge N on the support M (tightening torque at 20 Nm ). Important - Always replace the gaskets P, Q, and U at each assembly. - Lubricate the gaskets P, Q with oil before assembling them. - Insert the gasket P on the seat of the fitting K . - Insert the head J on the fitting K and the gasket Q in the seat of the head J . - Clamp the fitting K (tightening torque at 45 Nm + Loctite 2701 ). - Clamp the fitting H on the flange J inserting the gasket U (tightening torque at 65 Nm ). | Fig 11.51 |
| - Connect the hose B to the central fitting of support M and of flange J . - Connect the hose C to the side fitting of the support M and of head J . - Clamp the nuts A on the head J (tightening torque at 48 Nm ). - Clamp the nuts A on the support M (tightening torque at 48 Nm ). Important - Check the tightening of the fittings H (Fig. 11.51) and L (Fig. 11.52) (tightening torque at 65 Nm ). | Fig 11.52 |

## Intake circuit (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.10.1 Air filter disassembly - Release the clamp B . - Untighten the screws A and remove the filter C . | Fig. 11.53 |
| 11.10.2 Manifold air filter disassembly - Release the clamp D . - Remove the manifold E . - Loosen capscrews G and remove plate H . | Fig. 11.54 |
| 11.10.3 Air filter manifold assembly - Insert the fitting G on the flange H . - Tighten the clamps D . - Fasten plate G by means of capscrews F (tightening torque at 25 Nm ). | Fig. 11.55 |
| 11.10.4 Air filter assembly - Insert the filter C into the manifold E . - Tighten the clamps B . - Tighten the screws A (tightening torque at 25 Nm ) | Fig. 11.56 |

## Muffler (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.11.1 Disassembly - Undo the screw A . - Undo the nuts B . - Remove the muffler C . | Fig. 11.57 |
| 11.11.2 Assembly - Position the gasket D and the muffler C on the intake manifold E . - Position the muffler C on the bracket F and tighten the screws A (tightening torque at 25 Nm ). - Tighten the nuts B (tightening torque at 25 Nm ). | Fig. 11.58 |

## Cooling circuit (replacement)

| Important - Before proceeding with operation, read Par. 3.3.2 . |  |
| --- | --- |
| 11.12.1 Radiator disassembly - Release the clamp A . - Remove the sleeve B . | Fig. 11.59 |
| - Loosen the 4 capscrews C and capscrew E . | Fig. 11.60 |
| - Remove guard D . | Fig. 11.61 |
| - Release the clamp H . - Release the pipe L from the radiator M . | Fig. 11.62 |
| - Undo nuts N and remove the radiator M . | Fig. 11.63 |
| 11.12.2 Fan disassembly - Undo the screws P and remove the plate Q from the fan R . | Fig. 11.64 |
| 11.12.3 Fan assembly - Assemble the fan R on the pulley U . - Position the plate Q on the fan R . - Fasten the fan R by using the screws P and interposin washers S (tightening torque at 10 Nm ). | Fig. 11.65 |
| 11.12.4 Radiator assembly - Position the radiator on the support V. - Fasten radiator M on support V by means of nut N inserting washer W (tightening torque at 20 Nm ). | Fig. 11.66 |
| - Fit the sleeve L on the fitting of the radiator M . - Tighten the clamp H . | Fig. 11.67 |
| - Position bulkhead D onto shroud F . | Fig. 11.68 |
| - Fasten brace G by means of capscrews E (tightening torque at 25 Nm ). - Fasten guard D by means of capscrews C (tightening torque 10 Nm ). | Fig. 11.69 |
| - Fit the sleeve B on the fitting of the radiator M and of the thermostat cover Y . - Tighten the clamps A . | Fig. 11.70 |

## Engine feet (information)

| NOTE : Component not necessarily supplied by KOHLER . The representation of the engine is purely indicative. Important - The motor supports A can be installed on the bell B or on the crankcase C by using the screws D (tightening torque at 50 Nm ). | Fig. 11.71 |
| --- | --- |

## Oil sump with supporting structure

| 11.14.1 Flywheel (J) disassembly - Execute the operations described in Par. 7.10.1 . 11.14.2 Plate/flange housing (L) disassembly - Loosen supplementary capscrews A and B . - Execute the operations described in Par. 7.10.2 . - Remove housing or plate L . | Fig. 11.72 |
| --- | --- |

| 11.14.3 Oil sump disassembly - Execute the operations described in Par. 5.2 . - Loosen capscrews C and remove bypass tube D . - Loosen capscrews E and remove oil sump F . | Fig. 11.73 |
| --- | --- |

| 11.14.4 Oil sump assembly - Make sure contact surfaces G of oil sump F and crankcase H have no impurities. - Apply a sealing bead of approximately 2.5 mm ( Loctite 5660 ) onto surface G of crankcase H . - Place oil sump F onto crankcase H in correspondence with the fastening holes (use tool ST_18 ). | Fig. 11.74 |
| --- | --- |

| Fig. 11.75 |  |
| --- | --- |

| - Apply capscrews E into the fastening holes and use torque at 10 Nm . - Loosen capscrews E , leaving approximately 1 mm leeway ( position A ) between the neck surface of capscrews E and oil sump F . - Place flange housing or plate L onto crankcase H , complying with centring tap pins M . - Using 2 capscrews A , fasten housing or plate L onto crankcase H (tightening torque at 20 Nm ). - Using 2 capscrews A , fasten housing or plate L onto oil sump F (tightening torque at 20 Nm ). | Fig. 11.76 |
| --- | --- |

| Fig. 11.77 |  |
| --- | --- |

| - Fasten oil sump F by tightening capscrews E and strictly following the order shown in Fig. 11.77 (tightening torque at 20 Nm ). - Loosen capscrews A and remove housing or plate L ( Fig. 11.76 ). - Fasten oil sump F by tightening capscrews E and strictly following the order shown in Fig. 11.77 (tightening torque at 47 Nm ). Loosen the screw 1 again and tighten it to 47 Nm . | Fig. 11.78 |
| --- | --- |

| - Insert gaskets N into seats P of bypass tube D . - Fasten bypass tube D onto oil sump F using capscrews C (tightening torque at 10 Nm ). | Fig. 11.79 |
| --- | --- |

| 11.14.5 Flange plate / housing assembly - Execute the operations described in point 6 of Par. 11.14.4 . - Fasten housing or plate L by using capscrews A and strictly following the order shown in Fig. 11.79 (tightening torque at 85 Nm ). - Fasten housing or plate L by using capscrews B (tightening torque at 270 Nm ). 11.14.6 Flywheel assembly - Execute the operations described in Par. 9.5.2 . | Fig. 11.80 |
| --- | --- |